| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0656 |
| --- | --- |

| Title: | Non-CE5: Boundary strength derivation of deblocking filter for transform-skipped block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes modification of the boundary strength derivation of deblocking filter for transform-skipped block. In the current VTM, boundary strength is set equal to 1 when one of the adjacent blocks has non-zero transform coefficients. This process is based on the assumption that the energy distribution of the residual signal spreads in a whole block if a given block has non-zero transform coefficients. However, considering the transform-skipped case, the energy distribution may not spread in a whole block since inverse transform is not applied to the transform-skipped block. The proposed modification adds one condition to the boundary strength derivation to check whether the block is transform-skipped or not. The experimental results show that the proposed modification reduce the over-smoothing on the reconstructed picture.

Introduction

In VVC working draft 5, deblocking filtering process is mostly same as those implemented in HEVC. For each block boundary, the parameter of the deblocking filtering which is called boundary strength (bS) was derived by the following conditions in order.

| Conditions | bS |
| --- | --- |
| At least one of the adjacent blocks is intra | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater that or equal to one integer luma sample | 1 |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 |
| Otherwise | 0 |

The condition related to the non-zero transform coefficients is based on the assumption that there might be a discontinuity on the reconstructed picture when one of the adjacent blocks has non-zeros coefficient. That is, if one of the blocks has non-zero coefficients, the residual signal generated by inverse transform might spread in a whole block and discontinuity near the block boundary appears even if the difference of the motion vectors that belong to the adjacent block is smaller than integer luma sample.

However, considering the transform-skipped case, the residual signal was not generated by inverse transform, so that the energy distribution of the residual signal might not spread in a whole block. Nevertheless, for all the block boundaries which satisfy the abovementioned condition related to non-zero transform coefficients, the deblocking filter is applied whether or not the block which has non-zero transform coefficients applies transform skip mode. This may cause over-smoothing in the reconstructed picture.

Proposed modification

This contribution proposes the modification of the abovementioned condition as follows:

- At least one of the adjacent blocks satisfies all the following conditions

  - The block has non-zero transform coefficients

  - The block is not transform-skipped block

The proposed modifications are illustrated in Figure 1. As shown the figure, only if one of the adjacent block is the block which has non-zero transform coefficients and is not transform-skipped block, bS is set equal to 1. Otherwise, set equal to 0.

Figure 1. Proposed modification of bS derivation

Experimental results

The proposed modification of the boundary strength derivation was implemented on top of VTM-5.0. The BD-rate results for random access and low delay configuration were shown in Table 1 and 2, respectively. No performance impact was observed for both configuration and encoding/decoding run time were unchanged.

Table 1. BD-rate performance of the proposed modification in randomaccess configuration over VTM-5.0

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | 0.01% | -0.02% | 0.03% | 100% | 99% |
| Class B | 0.01% | 0.07% | 0.02% | 100% | 101% |
| Class C | -0.03% | -0.04% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.13% | 0.06% | 100% | 100% |
| Class F | -0.02% | 0.08% | 0.03% | 100% | 101% |

Table 2. BD-rate performance of the proposed modification in low delay configuration over VTM-5.0

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.14% | 0.11% | 100% | 100% |
| Class C | 0.04% | 0.05% | -0.01% | 100% | 103% |
| Class E | 0.02% | -0.28% | -0.32% | 100% | 102% |
| Overall | 0.01% | 0.01% | -0.04% | 100% | 101% |
| Class D | -0.04% | 0.21% | 0.57% | 100% | 100% |
| Class F | 0.00% | -0.75% | -0.81% | 100% | 101% |

Figure 2 shows the visual quality comparison between VTM-5.0 and the proposal. It is observed that the reconstructed image of VTM-5.0 is over-smoothed near the boundary of the transform skipped block whereas the edge information was preserved in the reconstructed image of the proposed method.

- VTM-5.0                   (b) Proposal

Figure 2. Visual quality comparison (zoomed portion of reconstructed picture of SlideShow lowdelay QP27)

Proposed specification change

- 8.8.3.5 Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- a variable tu_mts_idx specifying which transform kernels are applied to the residual samples of the current transform block.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

The variables xDi, yDj, xN and yN are derived as follows:

- If edgeType is equal to EDGE_VER,

xDi = ( i  <<  3 )		(8-1026)

yDj = cIdx = = 0  ?  ( j  <<  2 )  :  ( j  <<  1 )		(8-1027)

xN is set equal to Max( 0, ( nCbW / 8 ) − 1 )		(8-1028)

yN = cIdx = = 0  ?  ( nCbH / 4 ) − 1  :  ( nCbH / 2 ) − 1	(8-1029)

- Otherwise (edgeType is equal to EDGE_HOR),

xDi = cIdx = = 0  ?  ( i  <<  2 )  :  ( i  <<  1 )		(8-1030)

yDj = ( j  <<  3 )		(8-1031)

xN = cIdx = = 0  ?  ( nCbW / 4 ) − 1  :  ( nCbW / 2 ) − 1	(8-1032)

yN = Max( 0, ( nCbH / 8 ) − 1 )		(8-1033)

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

- The sample values p0 and q0 are derived as follows:

    - If edgeType is equal to EDGE_VER, p0 is set equal to recPicture[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

    - Otherwise (edgeType is equal to EDGE_HOR), p0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block tu_mts_idx of which is not equal to 1 and which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the motion vectors used in the prediction of the two coding subblocks is greater than or equal to 4 in units of quarter luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 4 in units of quarter luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 4 in quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 4 in units of quarter luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 4 in units of quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 4 in units of quarter luma samples.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Conclusion

This contribution proposes modification of the boundary strength derivation of deblocking filter for transform-skipped block. The proposed modification adds one condition to the boundary strength derivation to check whether the block is transform-skipped or not. The experimental results show that the proposed modification reduce the over-smoothing on the reconstructed picture. It is recommended that the proposed modification is adopted to the next version of VVC draft specifications.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).