| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0657 |
| --- | --- |

| Title: | AHG7: Tool-on/off experiments for HLG CTC sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports tool on/off experimental results vs. VTM anchor for HLG common test sequences. For a total of 11 tools for all intra and 23 tools for random access configuration, tool-off experiments were conducted.

Results

Table 1 and 2 show the experimental results of tool-off test for all intra and random access configuration, respectively. The abbreviation of the tool tested is aligned with those of described in JVET-N1005 “Methodology and reporting template for tool testing”.

For each test, configuration files used for the experiments were obtained from the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-5.0/VTM_tools

More details of the results for each test can be found in the attached spreadsheet.

Table 1. Simulation results in all intra configuration of VTM tool tests

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool off | Y | U | V | Enc | Dec |
| ALF | 2.91% | 1.39% | 1.63% | 97% | TBP |
| CCLM | 1.95% | 46.16% | 22.71% | 101% | TBP |
| CST | 0.70% | 17.43% | 24.96% | 178% | TBP |
| DQ | 1.89% | -1.02% | -1.67% | 95% | TBP |
| ISP | 0.24% | 0.21% | 0.16% | 91% | TBP |
| JCC | 0.11% | 0.03% | 4.03% | 97% | 98% |
| MIP | 0.36% | 0.18% | 0.13% | 85% | TBP |
| MRL | 0.07% | -0.02% | -0.05% | 95% | 97% |
| MTS | 1.24% | 1.01% | 0.93% | 87% | TBP |
| NSST | 0.71% | -0.49% | -0.57% | 78% | TBP |
| RDPCM-on | -0.02% | -0.01% | -0.03% | 105% | TBP |

Table 2. Simulation results in random access configuration of VTM tool tests.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool off | Y | U | V | Enc | Dec |
| AFF | 0.55% | 0.47% | 0.45% | 87% | 100% |
| ALF | 3.69% | 2.48% | 1.33% | 97% | 83% |
| AMVR | 0.55% | 0.95% | 1.36% | 94% | 96% |
| BCW | 0.23% | 0.20% | 0.23% | 95% | 103% |
| BDOF | 0.66% | 0.32% | 0.32% | 94% | 96% |
| CCLM | 0.87% | 51.82% | 27.51% | 101% | 102% |
| CIIP | 0.26% | -0.33% | -0.37% | 97% | 101% |
| CST | 0.26% | 10.88% | 19.65% | 105% | 102% |
| DMVR | 0.89% | 1.19% | 1.02% | 101% | 97% |
| DQ | 1.83% | -0.42% | -1.34% | 101% | 104% |
| ISP | 0.11% | 0.20% | 0.07% | 100% | 101% |
| JCC | 0.09% | -0.45% | 5.76% | 99% | 100% |
| MIP | 0.37% | 0.39% | -0.02% | 97% | 102% |
| MMVD | 0.31% | 0.29% | 0.41% | 88% | 102% |
| MRL | 0.05% | 0.09% | -0.15% | 99% | 100% |
| MTS | 0.64% | 1.04% | 0.28% | 95% | 101% |
| NSST | 0.42% | 0.56% | 0.07% | 91% | 101% |
| RDPCM-on | -0.01% | -0.06% | 0.04% | 102% | 101% |
| SbTMVP | 0.39% | 0.23% | 0.14% | 102% | 102% |
| SBT | 0.31% | -0.16% | -0.07% | 97% | 100% |
| SMVD | 0.21% | 0.01% | 0.18% | 96% | 102% |
| TMVP | 1.01% | 0.77% | 0.80% | 101% | 101% |
| TPM | 0.54% | 0.89% | 0.69% | 93% | 100% |

Conclusion

This document provides tool-on/off test for a total of 11 tools for AI and 23 tools for RA. It is recommended to further investigate individual tool efficiency for HDR test sequences.

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.