| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0658 |
| --- | --- |

| Title: | Crosscheck of JVET-O0432 on LMCS encoder improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | S. Iwamura | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of JVET-O0432 on LMCS encoder improvement. No significant inconsistencies were found between the software implementation and the proposed modification described in JVET-O0432. The BD-rate results of ClassH2 (HLG) sequences matched with those provided by the proponent. Although encoding and decoding time is slightly different from that of proponent, the difference can be considered within the margin of error.

Simulation results

| All intra | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| H2_DayStreet | -0.18% | 2.55% | 2.73% |  |  |
| H2_FlyingBirds2 | -0.21% | 1.70% | 1.24% |  |  |
| H2_PeopleInShoppingCenter | -0.30% | 3.92% | 3.98% |  |  |
| H2_SunsetBeach2 | 0.00% | 0.00% | 0.00% |  |  |
| All | -0.17% | 2.04% | 1.99% | 102% | 98% |

| Random access | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| H2_DayStreet | -0.79% | 3.07% | 2.90% |  |  |
| H2_FlyingBirds2 | -1.52% | 3.48% | 2.39% |  |  |
| H2_PeopleInShoppingCenter | -1.21% | 3.62% | 3.76% |  |  |
| H2_SunsetBeach2 | -0.59% | -1.03% | -1.29% |  |  |
| All | -1.03% | 2.28% | 1.94% | 101% | 95% |

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.