| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0660-r1 |
| --- | --- |

| Title: | Simplification of MV storage for blending blocks in triangle mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone and Limin Wang | Email: | Krit.panusopone@nokia.com limin.2.wang@nokia.com |
| Source: | Nokia, USA |  |  |

_____________________________

# Abstract

This contribution proposes two approaches to simplify MV storage for blending blocks in triangle mode. The first approach eliminates blending blocks and only one MV is stored for all blocks in triangle mode. The second approach always stores one MV when MVs of first and second triangle partition are from the same reference list. The results show that methods 1 and 2 from the first approach have luma BD-rate of approximately 0.04% and 0.03% for RA configuration, respectively. The results also show that methods 3 and 4 from the second approach have luma BD-rate of approximately 0.00% and -0.01% for RA configuration, respectively.

# Introduction

Triangle partition mode [1] is one of merge modes. When using triangle partition mode, a CU is split into 2 triangle partitions. Each triangle partition has its own uni-prediction MV, derived from merge candidate index. Triangle partition mode can be split along 45 degree (from top right corner to bottom left corner), or 135 degree (from top left corner to bottom right corner). Blending applies to pixels in the CU around triangle split by weighted averaging MC predictors from both triangle partitions.

MV derived for triangle partition mode is stored at 4x4 block granularity for later use as neighbor PMV. For triangle partition mode, each 4x4 block stores MV of triangle partition that it belongs to 4x4 blocks along triangle split (blending block) belong to two triangle partitions and hence MV from two triangle partitions are stored for these blending blocks. Table 1 shows an example of MV partition assignment for triangle partition mode. In this table, ‘0’ represents block storing partition index 0 MV, ‘1’ represents block storing partition index ‘1’ MV, and ‘2’ represents blending block.

Table 1. Example of partition index assignment in the current VVC design for 32x16 CU; 45 degree split (left) and 135 degree (right).

| 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |  | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 2 | 2 | 1 | 1 |  | 1 | 1 | 2 | 2 | 0 | 0 | 0 | 0 |
| 0 | 0 | 2 | 2 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 2 | 2 | 0 | 0 |
| 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |

# MV storage for blending block

MV storage for blending block is straightforward when MVs of first and second triangle partition are from different reference lists. In this case, MVs from both first and second triangle partition are stored.

MV storage for blending block is complicated when MVs of first and second triangle partition are from the same reference list. In this case, reference mapping process takes place to find the same reference in unused list. If that reference is not in the unused list, then only MV from the first triangle partition is stored.

VTM-5.0 [2] requires a lot of steps to store MV for blending block. At the beginning of sequence, buffer is created to store partition index assignment for every block in CU for all possible sizes (Table 1 is one example of this buffer). For each triangle mode CU, one or two MV is/are stored for blending block depending on reference list used by first partition MV and second partition MV. Finally, partition index assignment buffer is used to determined which MV to store according to location of the block within a CU.

# Proposal

Two approaches are proposed in this contribution to simplify MV storage of blending block.

Approach 1: modify partition index assignment buffer such that blending block (index 2) is not used. Hence, only one MV is stored for all blocks in triangle mode. This contribution describes two methods (methods 1 and 2) for approach 1.

For method 1, MV from partition bordering right neighbor is stored for all blending blocks. Table 2 shows an example of partition index assignment for method 1.

Table 2. Example of partition index assignment for method 1 for 32x16 CU; 45 degree split (left) and 135 degree (right).

| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

Method 2 includes one change compared to method 1 to improve coding efficiency for non-square CU. Specifically, method 2 stores MV from partition bordering left neighbor for all blending blocks when CU with width greater than height is split in 135 degree. Table 3 shows an example of partition index assignment for method 2.

Table 3. Example of partition index assignment for method 2 for 32x16 CU; 45 degree split (left) and 135 degree (right).

| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Approach 2: always store one MV from a predetermined partition for all blending blocks in a CU when MVs from first and second triangle partition of that CU are from the same reference list (worst case). There is no change in partition index assignment buffer. This contribution describes two methods (methods 3 and 4) for approach 2. For triangle mode CU in the worst case, method 3 stores MV from partition bordering right neighbor for all blending blocks in that CU.

Method 4 includes one change compared to method 3 to improve coding efficiency for non-square CU. Specifically, method 4 stores MV from partition bordering left neighbor for all blending blocks when CU with width greater than height is split in 135 degree and MVs from first and second triangle partition of that CU are from the same reference list.

# Simulation results

The proposed method 1 is implemented in VTM-5.0 [2] and the results are shown in Tables 4 and 5 against the VTM-5.0 under RA and LB CTC [3], respectively.

Table 4. Simulation result of method 1 for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.05% | 172% | 168% |
| Class A2 | 0.05% | 0.03% | 0.09% | 172% | 163% |
| Class B | 0.07% | 0.04% | -0.04% | 171% | 165% |
| Class C | -0.01% | -0.13% | -0.15% | 173% | 163% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.05% | -0.02% | 172% | 165% |
| Class D | 0.05% | -0.13% | 0.35% | 172% | 161% |
| Class F | -0.01% | 0.05% | 0.01% | 164% | 165% |

Table 5. Simulation result of method 1 for LB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.26% | 0.05% | 100% | 100% |
| Class C | 0.16% | 0.12% | 0.21% | 100% | 99% |
| Class E | 0.23% | 0.30% | -0.29% | 100% | 101% |
| Overall | 0.17% | 0.22% | 0.02% | 100% | 100% |
| Class D | 0.16% | -0.21% | 1.03% | 102% | 103% |
| Class F | 0.01% | 0.04% | -0.22% | 100% | 94% |

The proposed method 2 is implemented in VTM-5.0 [2] and the results are shown in Tables 6 and 7 against the VTM-5.0 under RA and LB CTC [3], respectively.

Table 6. Simulation result of method 2 for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | -0.01% | 172% | 168% |
| Class A2 | 0.03% | 0.15% | 0.13% | 170% | 162% |
| Class B | 0.05% | 0.04% | -0.05% | 171% | 166% |
| Class C | -0.01% | -0.09% | -0.17% | 173% | 166% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.01% | -0.04% | 171% | 166% |
| Class D | 0.09% | 0.03% | 0.16% | 172% | 165% |
| Class F | 0.00% | 0.04% | -0.02% | 163% | 166% |

Table 7. Simulation result of method 2 for LB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | 0.39% | 0.19% | 56% | 62% |
| Class C | 0.14% | -0.29% | 0.05% | 55% | 60% |
| Class E | 0.09% | -0.08% | -0.08% | 57% | 62% |
| Overall | 0.12% | 0.04% | 0.08% | 56% | 62% |
| Class D | 0.11% | 0.14% | 0.64% | 57% | 59% |
| Class F | -0.07% | -0.33% | -0.24% | 58% | 62% |

The proposed method 3 is implemented in VTM-5.0 [2] and the results are shown in Tables 8 and 9 against the VTM-5.0 under RA and LB CTC [3], respectively.

Table 8. Simulation result of method 3 for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.08% | 96% | 110% |
| Class A2 | -0.02% | 0.04% | -0.02% | 96% | 109% |
| Class B | 0.01% | 0.13% | -0.10% | 95% | 112% |
| Class C | -0.02% | -0.14% | -0.10% | 96% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.05% | 96% | 110% |
| Class D | 0.01% | 0.18% | 0.15% | 95% | 103% |
| Class F | -0.01% | 0.03% | 0.00% | 93% | 110% |

Table 9. Simulation result of method 3 for LB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.21% | -0.03% | 56% | 64% |
| Class C | 0.06% | 0.10% | 0.07% | 55% | 63% |
| Class E | 0.02% | 0.14% | -0.43% | 58% | 63% |
| Overall | 0.06% | 0.16% | -0.10% | 56% | 64% |
| Class D | 0.01% | 0.00% | 0.43% | 57% | 62% |
| Class F | 0.05% | 0.31% | -0.05% | 58% | 65% |

The proposed method 4 is implemented in VTM-5.0 [2] and the results are shown in Tables 10 and 11 against the VTM-5.0 under RA and LB CTC [3], respectively.

Table 10. Simulation result of method 4 for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | 0.00% | 177% | 164% |
| Class A2 | 0.00% | 0.13% | 0.03% | 176% | 164% |
| Class B | 0.01% | 0.02% | -0.06% | 177% | 168% |
| Class C | -0.05% | -0.22% | -0.14% | 178% | 168% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.05% | 177% | 166% |
| Class D | 0.00% | 0.18% | 0.19% | 177% | 167% |
| Class F | -0.01% | 0.03% | -0.03% | 170% | 166% |

Table 11. Simulation result of method 4 for LB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.08% | 56% | 64% |
| Class C | 0.08% | 0.12% | 0.38% | 56% | 63% |
| Class E | 0.08% | 0.32% | -0.28% | 58% | 64% |
| Overall | 0.07% | 0.14% | 0.09% | 56% | 64% |
| Class D | 0.11% | 0.11% | 0.30% | 57% | 62% |
| Class F | -0.04% | 0.29% | 0.45% | 58% | 64% |

# Conclusions

Two approaches to simplify MV storage for blending block are proposed in this contribution. The first approach reduces more complexity by not allowing blending block. The second approach mainly targets the worst-case complexity by using MV from a predetermined partition for all blending blocks in a CU that has MV from first triangle partition using the same reference list as MV from second triangle partition. The results show a BD-rate performance of 0.00% and -0.01% for methods 3 and 4, respectively, under RA coding condition.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding”, JVET- JVET-N1001-v6, March 2019.

- VTM-5.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).