| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0662-v1 |
| --- | --- |

| Title: | Non-CE5: Modified ALF filtering for Slice, Brick and Virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

In VTM-5.0, Slice, Brick and Virtual boundaries are treated as picture boundaries whenever ALF filtering is chosen to be turned off. Therefore the ALF filtering would simply use “sample padding” as done at picture boundaries. Since Slice, Brick and Virtual boundaries can occur anywhere inside the picture, it could be important that the subjective quality is preserved and therefore the current contribution proposes to apply the same design of ALF filtering performed at horizontal CTU boundaries, to the ALF filtering performed at Slice, Brick and Virtual boundaries whenever ALF filtering is chosen to be turned off.

Under CTC conditions, no BD-Rate difference is expected.

# Introduction

VTM-5.0 [1] uses the concept of Slice, Bricks and Virtual boundaries and the syntax elements  “loop_filter_across_slices_enabled_flag”,  “loop_filter_across_bricks_enabled_flag” and “pps_loop_filter_across_virtual_boundaries_disabled_flag”  are used to determine if the loop filtering (which includes deblocking, SAO and ALF) are turned on (or) off across the boundaries. For ALF, whenever the filtering is turned off, the process switches to the way ALF filtering is done at picture boundaries, where the samples are just padded to replace the unavailable samples.

Since the Slice, Brick or Virtual boundaries can occur anywhere inside the given picture, it could be important that the subjective quality of ALF is preserved. Therefore the current proposal targets to improve the ALF filtering design whenever the filtering is chosen to be turned off at the Slice, Brick or Virtual boundaries.

# Proposed Solution

The proposed solution uses the same ALF filter design which is used at the horizontal CTU boundaries for reducing line buffer as described in [2].  For the ALF block classification, only the samples which are available are used and the activity value is scaled accordingly.  For the ALF filtering, symmetric padding is applied both in horizontal and vertical directions for horizontal and vertical boundaries respectively.

# Experimental results (non-CTC)

To be added

# Conclusion

Since Slice, Brick and Virtual boundaries can occur anywhere inside the picture, it could be important that the subjective quality is preserved and therefore the current contribution proposes to apply the same design of ALF filtering performed at horizontal CTU boundaries to the ALF filtering performed at Slice, Brick and Virtual boundaries whenever ALF filtering is chosen to be turned off.  It is recommended to further study this proposal in a core experiment with well-defined test conditions.

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- CE5-2: Loop filter line buffer reduction JVET-N0180, A. M. Kotra et al.

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).