| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0663 |
| --- | --- |

| Title: | CE11-3.2: Gradual Random Access (GRA) with Sub-Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kashyap Kammachi-Sreedhar, Miska M. Hannuksela, Alireza Aminlou Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Tel: Email: | firstname.lastname@nokia.com |
| Source: | Nokia. |  |  |

_____________________________

# Abstract

This contribution presents the result of CE11 test 3.2 for Gradual Random Access (GRA) using sub-picture-based picture partitioning. In the proposed method, the GRA is achieved by separating the refreshed region (clean area) and the un-refreshed region (dirty area) into two different sub-pictures as detailed in [1].

Simulations were carried out according to CE11 specific test conditions. The reported results show that the average Y component BD-rate overhead is xxx% in low-delay B configuration of [2].

# Introduction

An AHG was established, during the 14th JVET meeting in Geneva, March 2019. The aim of the AHG was to study alternate solutions including both normative and non-normative categories for achieving gradual random access (GRA).   Multiple solutions for achieving gradual random access (GRA) techniques have been recommended for further study in CE11.

The proposal in test CE11 3.2 is summarized as follows:

- Pictures from the random access point (inclusive) to the recovery point (exclusive) have two sub-pictures, a refresh sub-picture and a "dirty" sub-picture. The size of the refresh sub-picture increases from the random access point towards the recovery point.

- full_pic_ref_flag is included in the PPS. When equal to 0, the sub-picture with the index (in decoding order, within the referenced coded picture) that has the same index as the current sub-picture in the current picture is extracted and used as reference for inter prediction. When equal to 1, the entire reference picture is used as reference for inter prediction of the current sub-picture.

- full_pic_ref_flag is equal to 0 in the active PPS for the refresh sub-picture. Thus, only the previous refresh sub-pictures are used as reference for prediction of the current refresh sub-picture.

- full_pic_ref_flag is equal to 1 in the active PPS for the "dirty" sub-picture. Thus, a "dirty" sub-picture can use samples from both "dirty" sub-picture and refresh sub-pictures for deriving prediction blocks of inter prediction.

# Refreshed and unrefreshed regions

Figure 1 provides an example illustrating the proposal. Refresh sub-pictures are indicated with the diagonally striped rectangles, while the remaining picture area forms another sub-picture, which is casually called "dirty" sub-picture.

Figure 1. Example of the proposed sub-picture based gradual decoding refresh

As shown above, blocks within a refresh sub-picture refer only to blocks that belong to the refresh sub-picture region in a reference picture within the same GRA period. Blocks within an unrefreshed sub-picture can refer to blocks anywhere in the reference picture including both refresh sub-picture region and the unrefreshed sub-picture region.

# Simulation results

The implementation of CE11 test 3.2 was done on top of VTM 5.0 [3]. The implementation is available in CE11 software repository https://vcgit.hhi.fraunhofer.de/jvet-n-ce11/VVCSoftware_VTM/branches.

Simulation were performed using CE11 specific test conditions as detailed in [2]. The BD-rate results are shown in table below.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE11 Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 6,55 % | 1,86 % | 2,49 % | #VALUE! | #VALUE! |
| Class E | 9,91 % | 2,41 % | 6,38 % | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 13,89 % | 15,06 % | 15,30 % | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 1: GRA with Sub-pictures

# References

- M. M. Hannuksela, K.K. Sreedhar and A. Aminlou, “AHG14: On gradual decoding refresh” Document of Joint Video Experts Team, JVET-N0310, March 2019.

- K. Kazui, J-M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 11 (CE11): Gradual random access”, Document of Joint Video Experts Team, JVET-N1031, March 2019

- Reference software VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).