      - 7.3.5.3 Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 && alf_luma_num_filters_signalled_minus1 < NumAlfFilters - 1 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| if( alf_luma_fixed_filter_pred_present_flag ) { |  |
| for( i = 0; i < NumAlfFilters; i++ ) |  |
| alf_luma_fixed_filter_pred_flag[ i ] | u(1) |
| } |  |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| if(!(alf_luma_coeff_delta_flag == 1 && alf_luma_num_filters_signalled_minus1 == 0)) |  |
| { |  |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |

      - 7.4.6.3 Adaptive loop filter data semantics

alf_luma_filter_signal_flag equal to 1 specifies that a luma filter set is signalled. alf_luma_filter_signal_flag equal to 0 specifies that a luma filter set is not signalled.

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled.

The variable NumAlfFilters specifying the number of different adaptive loop filters is set equal to 25.

alf_luma_clip_flag equal to 0 specifies that linear adaptive loop filtering is applied on luma component. alf_luma_clip_flag equal to 1 specifies that non-linear adaptive loop filtering may be applied on luma component.

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to NumAlfFilters − 1.

alf_luma_coeff_delta_idx[ filtIdx ] specifies the indices of the signalled adaptive loop filter luma coefficient deltas for the filter class indicated by filtIdx ranging from 0 to NumAlfFilters − 1. When alf_luma_coeff_delta_idx[ filtIdx ] is not present and alf_luma_num_filters_signalled_minus1 equals to 0, it is inferred to be equal to 0. When alf_luma_coeff_delta_idx[ filtIdx ] is not present and alf_luma_num_filters_signalled_minus1 equals to NumAlfFilters − 1, it is inferred to be equal to filtIdx.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to alf_luma_num_filters_signalled_minus1.

alf_luma_use_fixed_filter_flag equal to 1 specifies that a fixed filter set is used to signal adaptive loop filter coefficients. alf_luma_use_fixed_filter_flag equal to 0 specifies that a fixed filter set is not used to signal adaptive loop filter coefficients.

alf_luma_fixed_filter_set_idx specifies a fixed filter set index. The value of alf_luma_fixed_filter_set_idx shall be in a range of 0 to 15, inclusive. The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 15.

alf_luma_fixed_filter_pred_present_flag equal to 1 specifies that alf_luma_fixed_filter_pred_flag[ i ] is present. alf_luma_fixed_filter_pred_present_flag equal to 0 specifies that alf_luma_fixed_filter_pred_flag[ i ] is not present.

alf_luma_fixed_filter_pred_flag[ i ] equal to 1 specifies that a fixed filter is used to predict the i-th filter coefficients. alf_luma_fixed_filter_pred_flag[ i ] equal to 0 specifies that a fixed filter is not used to predict the i-th filter coefficients. When alf_luma_fixed_filter_pred_flag[ i ] is not present, it is inferred to be equal to 1.

alf_luma_coeff_delta_flag equal to 1 indicates that alf_luma_coeff_delta_prediction_flag is not signalled. alf_luma_coeff_delta_flag equal to 0 indicates that alf_luma_coeff_delta_prediction_flag may be signalled.

alf_luma_coeff_delta_prediction_flag equal to 1 specifies that the signalled luma filter coefficient deltas are predicted from the deltas of the previous luma coefficients. alf_luma_coeff_delta_prediction_flag equal to 0 specifies that the signalled luma filter coefficient deltas are not predicted from the deltas of the previous luma coefficients. When not present, alf_luma_coeff_delta_prediction_flag is inferred to be equal to 0.

alf_luma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma filter coefficient signalling. The value of alf_luma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is incremented by 1. alf_luma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is not incremented by 1.

The order expGoOrderY[ i ] of the exp-Golomb code used to decode the values of alf_luma_coeff_delta_abs[ sfIdx ][ j ] is derived as follows:

expGoOrderY[ i ] = ( i = = 0  ?  alf_luma_min_eg_order_minus1 + 1  :  expGoOrderY[ i − 1 ] ) +	(7-66)
	alf_luma_eg_order_increase_flag[ i ]

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

alf_luma_coeff_delta_abs[ sfIdx ][ j ] specifies the absolute value of the j-th coefficient delta of the signalled luma filter indicated by sfIdx. When alf_luma_coeff_delta_abs[ sfIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxY[ ] = { 0, 0, 1, 0, 0, 1, 2, 1, 0, 0, 1, 2 }		(7-67)

k = expGoOrderY[ golombOrderIdxY[ j ] ]		(7-68)

alf_luma_coeff_delta_sign[ sfIdx ][ j ] specifies the sign of the j-th luma coefficient of the filter indicated by sfIdx as follows:

- If alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 0, the corresponding luma filter coefficient has a positive value.

- Otherwise (alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 1), the corresponding luma filter coefficient has a negative value.

When alf_luma_coeff_delta_sign[ sfIdx ][ j ] is not present, it is inferred to be equal to 0.

The variable filtCoeff[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filtCoeff[ sfIdx ][ j ] = alf_luma_coeff_delta_abs[ sfIdx ][ j ] * 	(7-69)
	( 1 − 2 * alf_luma_coeff_delta_sign[ sfIdx ][ j ] )

When alf_luma_coeff_delta_prediction_flag is equal 1, filtCoeff[ sfIdx ][ j ] with sfIdx = 1..alf_luma_num_filters_signalled_minus1 and j = 0..11 are modified as follows:

filtCoeff[ sfIdx ][ j ] += filtCoeff[ sfIdx − 1 ][ j ]		(7-70)

The luma filter coefficients AlfCoeffL[ adaptation_parameter_set_id ] with elements AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filtCoeff[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]	(7-71)

When alf_luma_use_fixed_filter_flag is equal to 1 and alf_luma_fixed_filter_pred_flag[ filtIdx ] is equal to 1, the following applies for filtIdx = 0..NumAlfFilters − 1 and j = 0..11:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] += 	(7-72)
	AlfFixFiltCoeff[ AlfClassToFiltMap[ alf_luma_fixed_filter_set_idx ][ filtIdx ] ][ j ]

The fixed filter coefficients AlfFixFiltCoeff[ i ][ j ] with i = 0..64, j = 0..11 and the class to filter mapping AlfClassToFiltMap[ m ][ n ] with m = 0..15 and n = 0..24 are derived as follows:

AlfFixFiltCoeff =	(7-73)

{

{ 0,   0,   2,  −3,   1,  −4,   1,   7,  −1,   1,  −1,   5}

{ 0,   0,   0,   0,   0,  −1,   0,   1,   0,   0,  −1,   2}

{ 0,   0,   0,   0,   0,   0,   0,   1,   0,   0,   0,   0}

{ 0,   0,   0,   0,   0,   0,   0,   0,   0,   0,  −1,   1}

{ 2,   2,  −7,  −3,   0,  −5,  13,  22,  12,  −3,  −3,  17}

{−1,   0,   6,  −8,   1,  −5,   1,  23,   0,   2,  −5,  10}

{ 0,   0,  −1,  −1,   0,  −1,   2,   1,   0,   0,  −1,   4}

{ 0,   0,   3, −11,   1,   0,  −1,  35,   5,   2,  −9,   9}

{ 0,   0,   8,  −8,  −2,  −7,   4,   4,   2,   1,  −1,  25}

{ 0,   0,   1,  −1,   0,  −3,   1,   3,  −1,   1,  −1,   3}

{ 0,   0,   3,  −3,   0,  −6,   5,  −1,   2,   1,  −4,  21}

{−7,   1,   5,   4,  −3,   5,  11,  13,  12,  −8,  11,  12}

{−5,  −3,   6,  −2,  −3,   8,  14,  15,   2,  −7,  11,  16}

{ 2,  −1,  −6,  −5,  −2,  −2,  20,  14,  −4,   0,  −3,  25}

{ 3,   1,  −8,  −4,   0,  −8,  22,   5,  −3,   2, −10,  29}

{ 2,   1,  −7,  −1,   2, −11,  23,  −5,   0,   2, −10,  29}

{−6,  −3,   8,   9,  −4,   8,   9,   7,  14,  −2,   8,   9}

{ 2,   1,  −4,  −7,   0,  −8,  17,  22,   1,  −1,  −4,  23}

{ 3,   0,  −5,  −7,   0,  −7,  15,  18,  −5,   0,  −5,  27}

{ 2,   0,   0,  −7,   1, −10,  13,  13,  −4,   2,  −7,  24}

{ 3,   3, −13,   4,  −2,  −5,   9,  21,  25,  −2,  −3,  12}

{−5,  −2,   7,  −3,  −7,   9,   8,   9,  16,  −2,  15,  12}

{ 0,  −1,   0,  −7,  −5,   4,  11,  11,   8,  −6,  12,  21}

{ 3,  −2,  −3,  −8,  −4,  −1,  16,  15,  −2,  −3,   3,  26}

{ 2,   1,  −5,  −4,  −1,  −8,  16,   4,  −2,   1,  −7,  33}

{ 2,   1,  −4,  −2,   1, −10,  17,  −2,   0,   2, −11,  33}

{ 1,  −2,   7, −15, −16,  10,   8,   8,  20,  11,  14,  11}

{ 2,   2,   3, −13, −13,   4,   8,  12,   2,  −3,  16,  24}

{ 1,   4,   0,  −7,  −8,  −4,   9,   9,  −2,  −2,   8,  29}

{ 1,   1,   2,  −4,  −1,  −6,   6,   3,  −1,  −1,  −3,  30}

{−7,   3,   2,  10,  −2,   3,   7,  11,  19,  −7,   8,  10}

{ 0,  −2,  −5,  −3,  −2,   4,  20,  15,  −1,  −3,  −1,  22}

{ 3,  −1,  −8,  −4,  −1,  −4,  22,   8,  −4,   2,  −8,  28}

{ 0,   3, −14,   3,   0,   1,  19,  17,   8,  −3,  −7,  20}

{ 0,   2,  −1,  −8,   3,  −6,   5,  21,   1,   1,  −9,  13}

{−4,  −2,   8,  20,  −2,   2,   3,   5,  21,   4,   6,   1}

{ 2,  −2,  −3,  −9,  −4,   2,  14,  16,   3,  −6,   8,  24}

{ 2,   1,   5, −16,  −7,   2,   3,  11,  15,  −3,  11,  22}

{ 1,   2,   3, −11,  −2,  −5,   4,   8,   9,  −3,  −2,  26}

{ 0,  −1,  10,  −9,  −1,  −8,   2,   3,   4,   0,   0,  29}

{ 1,   2,   0,  −5,   1,  −9,   9,   3,   0,   1,  −7,  20}

{−2,   8,  −6,  −4,   3,  −9,  −8,  45,  14,   2, −13,   7}

{ 1,  −1,  16, −19,  −8,  −4,  −3,   2,  19,   0,   4,  30}

{ 1,   1,  −3,   0,   2, −11,  15,  −5,   1,   2,  −9,  24}

{ 0,   1,  −2,   0,   1,  −4,   4,   0,   0,   1,  −4,   7}

{ 0,   1,   2,  −5,   1,  −6,   4,  10,  −2,   1,  −4,  10}

{ 3,   0,  −3,  −6,  −2,  −6,  14,   8,  −1,  −1,  −3,  31}

{ 0,   1,   0,  −2,   1,  −6,   5,   1,   0,   1,  −5,  13}

{ 3,   1,   9, −19, −21,   9,   7,   6,  13,   5,  15,  21}

{ 2,   4,   3, −12, −13,   1,   7,   8,   3,   0,  12,  26}

{ 3,   1,  −8,  −2,   0,  −6,  18,   2,  −2,   3, −10,  23}

{ 1,   1,  −4,  −1,   1,  −5,   8,   1,  −1,   2,  −5,  10}

{ 0,   1,  −1,   0,   0,  −2,   2,   0,   0,   1,  −2,   3}

{ 1,   1,  −2,  −7,   1,  −7,  14,  18,   0,   0,  −7,  21}

{ 0,   1,   0,  −2,   0,  −7,   8,   1,  −2,   0,  −3,  24}

{ 0,   1,   1,  −2,   2, −10,  10,   0,  −2,   1,  −7,  23}

{ 0,   2,   2, −11,   2,  −4,  −3,  39,   7,   1, −10,   9}

{ 1,   0,  13, −16,  −5,  −6,  −1,   8,   6,   0,   6,  29}

{ 1,   3,   1,  −6,  −4,  −7,   9,   6,  −3,  −2,   3,  33}

{ 4,   0, −17,  −1,  −1,   5,  26,   8,  −2,   3, −15,  30}

{ 0,   1,  −2,   0,   2,  −8,  12,  −6,   1,   1,  −6,  16}

{ 0,   0,   0,  −1,   1,  −4,   4,   0,   0,   0,  −3,  11}

{ 0,   1,   2,  −8,   2,  −6,   5,  15,   0,   2,  −7,   9}

{ 1,  −1,  12, −15,  −7,  −2,   3,   6,   6,  −1,   7,  30}

},

AlfClassToFiltMap =	(7-74)

{

{  8,  2,  2,  2,  3,  4, 53,  9,  9, 52,  4,  4,  5,  9,  2,  8, 10,  9,  1,  3, 39, 39, 10,  9, 52 }

{ 11, 12, 13, 14, 15, 30, 11, 17, 18, 19, 16, 20, 20,  4, 53, 21, 22, 23, 14, 25, 26, 26, 27, 28, 10 }

{ 16, 12, 31, 32, 14, 16, 30, 33, 53, 34, 35, 16, 20,  4,  7, 16, 21, 36, 18, 19, 21, 26, 37, 38, 39 }

{ 35, 11, 13, 14, 43, 35, 16,  4, 34, 62, 35, 35, 30, 56,  7, 35, 21, 38, 24, 40, 16, 21, 48, 57, 39 }

{ 11, 31, 32, 43, 44, 16,  4, 17, 34, 45, 30, 20, 20,  7,  5, 21, 22, 46, 40, 47, 26, 48, 63, 58, 10 }

{ 12, 13, 50, 51, 52, 11, 17, 53, 45,  9, 30,  4, 53, 19,  0, 22, 23, 25, 43, 44, 37, 27, 28, 10, 55 }

{ 30, 33, 62, 51, 44, 20, 41, 56, 34, 45, 20, 41, 41, 56,  5, 30, 56, 38, 40, 47, 11, 37, 42, 57,  8 }

{ 35, 11, 23, 32, 14, 35, 20,  4, 17, 18, 21, 20, 20, 20,  4, 16, 21, 36, 46, 25, 41, 26, 48, 49, 58 }

{ 12, 31, 59, 59,  3, 33, 33, 59, 59, 52,  4, 33, 17, 59, 55, 22, 36, 59, 59, 60, 22, 36, 59, 25, 55 }

{ 31, 25, 15, 60, 60, 22, 17, 19, 55, 55, 20, 20, 53, 19, 55, 22, 46, 25, 43, 60, 37, 28, 10, 55, 52 }

{ 12, 31, 32, 50, 51, 11, 33, 53, 19, 45, 16,  4,  4, 53,  5, 22, 36, 18, 25, 43, 26, 27, 27, 28, 10 }

{  5,  2, 44, 52,  3,  4, 53, 45,  9,  3,  4, 56,  5,  0,  2,  5, 10, 47, 52,  3, 63, 39, 10,  9, 52 }

{ 12, 34, 44, 44,  3, 56, 56, 62, 45,  9, 56, 56,  7,  5,  0, 22, 38, 40, 47, 52, 48, 57, 39, 10,  9 }

{ 35, 11, 23, 14, 51, 35, 20, 41, 56, 62, 16, 20, 41, 56,  7, 16, 21, 38, 24, 40, 26, 26, 42, 57, 39 }

{ 33, 34, 51, 51, 52, 41, 41, 34, 62,  0, 41, 41, 56,  7,  5, 56, 38, 38, 40, 44, 37, 42, 57, 39, 10 }

{ 16, 31, 32, 15, 60, 30,  4, 17, 19, 25, 22, 20,  4, 53, 19, 21, 22, 46, 25, 55, 26, 48, 63, 58, 55 }

},

It is a requirement of bitstream conformance that the values of AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] with filtIdx = 0..NumAlfFilters − 1, j = 0..11 shall be in the range of −27 to 27 − 1, inclusive.

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

The variable filterClips[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filterClips[ sfIdx ][ j ] = Round( 2( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) / 4 ) )	(7-77)

The luma filter clipping values AlfClipL[ adaptation_parameter_set_id ] with elements AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows:

AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filterClips[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]	(7-78)

alf_chroma_clip_flag equal to 0 specifies that linear adaptive loop filtering is applied on chroma components; alf_chroma_clip_flag equal to 1 specifies that non-linear adaptive loop filtering is applied on chroma component. When not present, alf_chroma_clip_flag is inferred to be equal to 0.

alf_chroma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma filter coefficient signalling. The value of alf_chroma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is incremented by 1. alf_chroma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is not incremented by 1

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_coeff_abs[ j ] is derived as follows:

expGoOrderC[ i ] = ( i = = 0  ?  alf_chroma_min_eg_order_minus1 + 1  :  expGoOrderC[ i − 1 ] ) +	(7-79)
	alf_chroma_eg_order_increase_flag[ i ]	(7-80)

alf_chroma_coeff_abs[ j ] specifies the absolute value of the j-th chroma filter coefficient. When alf_chroma_coeff_abs[ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_chroma_coeff_abs[ j ] shall be in the range of 0 to 27 − 1, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxC[ ] = { 0, 0, 1, 0, 0, 1 }		(7-81)

k = expGoOrderC[ golombOrderIdxC[ j ] ]		(7-82)

alf_chroma_coeff_sign[ j ] specifies the sign of the j-th chroma filter coefficient as follows:

- If alf_chroma_coeff_sign[ j ] is equal to 0, the corresponding chroma filter coefficient has a positive value.

- Otherwise (alf_chroma_coeff_sign[ j ] is equal to 1), the corresponding chroma filter coefficient has a negative value.

When alf_chroma_coeff_sign[ j ] is not present, it is inferred to be equal to 0.

The chroma filter coefficients AlfCoeffC[ adaptation_parameter_set_id ] with elements AlfCoeffC[ adaptation_parameter_set_id ][ j ], with j = 0..5 are derived as follows:

AlfCoeffC[ adaptation_parameter_set_id ][ j ] = alf_chroma_coeff_abs[ j ] * 	(7-83)
	( 1 − 2 * alf_chroma_coeff_sign[ j ] )

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ j ] with j = 0..5 shall be in the range of −27 − 1 to 27 − 1, inclusive.

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ] with elements AlfClipC[ adaptation_parameter_set_id ][ j ], with j = 0..5 are derived as follows:

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − alf_chroma_clip_idx[ j ] ) / 3 ) )	(7-87)