| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0666-v4 |
| --- | --- |

| Title: | Non-CE4: Fix for missing MV in 4xN and Nx4 triangle partition mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kevin Reuzé, Chun-Chi Chen, Wei-Jung Chien, Han Huang, Marta Karczewicz 10160 Pacific Mesa Blvd San Diego, CA 92121 (USA) | Email: | kreuz@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In triangle partition mode, the current triangle MC doesn’t use the motion information from the motion buffer, which is different from all other modes. It is asserted that this is not good for hardware implementation. It needs to be modified it so that triangle MC can be performed by only using the motion information in motion buffer. However, using the current motion storage design for triangle, there is a corner case (in 4xN/Nx4 block) that doesn’t have both motion information for partition 0 and partition 1.

Results show no coding difference in RA configuration and 0.01% BD-rate loss in LDB configuration.

It was requested in BoG to combine this test with the adopted method for TPM storage from JVET-O0411, Test B. This is reported in Test 3, where results over VTM5.0+JVET-O0411 Test B show that there is BD-rate loss of 0.01% in RA and 0.04% in LDB.

# Problem Statement

## Missing Mv fix for 4xN and Nx4

In 4xN and Nx4 CUs using TPM, only the bi-prediction motion vector is stored for the whole CU. In order to be able to perform triangle MC by only using the information in motion buffer this needs to be fixed to that each partition has the proper motion information.

# Proposed solution

## Test 1

It is proposed to always store Mv1 and Mv2 in the first and last CUs of 4xN and Nx4 triangle partitions.

This results in the following storage map for Nx4 partitions:

| Mv2 | biMv | biMv | biMv | biMv | biMv | biMv | Mv1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Mv1 | biMv | biMv | biMv | biMv | biMv | biMv | Mv2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Where is block is a 4x4 unit. For 4xN partitions the top 4x4 unit will always use Mv1 and bottom 4x4 unit will always use Mv2.

## Test 2

It is proposed to combine this solution with Test C from JVET-O0411.

# Results

## Test 1:

Random access results for test 1. No SIMD code has been used.

|  |  |  | Over VTM-5.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | -0.03% | 105% | 104% |
| Class A2 | 0.00% | 0.12% | 0.06% | 102% | 101% |
| Class B | 0.02% | 0.09% | -0.03% | 107% | 110% |
| Class C | -0.04% | -0.06% | -0.14% | 102% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | -0.04% | 104% | 107% |
| Class D | 0.00% | 0.00% | 0.07% | 99% | 100% |
| Class F | 0.00% | 0.03% | -0.01% | 104% | 109% |

.

Lowdelay B results for test 1. No SIMD code has been used.

|  |  |  | Over VTM-5.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.01% | 0.26% | 100% | 101% |
| Class C | 0.03% | 0.15% | 0.08% | 102% | 101% |
| Class E | -0.03% | 0.17% | -0.33% | 102% | 106% |
| Overall | 0.01% | 0.10% | 0.05% | 101% | 102% |
| Class D | 0.08% | -0.46% | 0.09% | 100% | 103% |
| Class F | -0.03% | -0.25% | -0.29% | 103% | 103% |

## Test 2: combination with test C from JVET-O0411

Random access results for test 2. No SIMD code has been used.

|  |  |  | Over VTM-5.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | 0.13% | 102% | 96% |
| Class A2 | 0.02% | 0.05% | 0.00% | 100% | 74% |
| Class B | 0.02% | 0.00% | -0.01% | 106% | 108% |
| Class C | -0.03% | -0.03% | -0.12% | 104% | 115% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.01% | 103% | 100% |
| Class D | 0.00% | -0.07% | 0.06% | 107% | 114% |
| Class F | 0.01% | 0.04% | -0.03% | 105% | 111% |

Lowdelay B results for test 2. No SIMD code has been used.

|  |  |  | Over VTM-5.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.08% | 0.25% | 0.07% | 101% | 93% |
| Class E | 0.09% | 0.63% | 0.12% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | -0.25% | 0.54% | 105% | 111% |
| Class F | 0.02% | -0.28% | -0.45% | 106% | #NUM! |

## Test 3: combination with test B from JVET-O0411

Random access results for test 2. No SIMD code has been used.

|  |  | Over VTM-5.0+JVET-O0411 Test B |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.03% | #DIV/0! | #DIV/0! |
| Class A2 | 0.01% | -0.01% | 0.01% | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.10% | -0.05% | 100% | 99% |
| Class C | 0.03% | -0.07% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.02% | #DIV/0! | #DIV/0! |
| Class D | 0.04% | 0.15% | 0.11% | 96% | 96% |
| Class F | 0.00% | 0.01% | 0.05% | 99% | 98% |

Lowdelay B results for test 2. No SIMD code has been used.

|  |  | Over VTM-5.0+JVET-O0411 Test B |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.11% | 0.00% | #DIV/0! | #DIV/0! |
| Class C | 0.03% | 0.05% | -0.26% | 99% | 96% |
| Class E | 0.11% | -0.07% | 0.16% | 99% | 101% |
| Overall | 0.04% | 0.05% | -0.04% | #DIV/0! | #DIV/0! |
| Class D | 0.01% | -0.42% | -0.69% | 98% | 97% |
| Class F | -0.11% | -0.14% | 0.14% | 96% | 95% |

# Conclusion

In this contribution it is proposed to solve the problem of missing motion vector in triangle partitions of Nx4 and 4xN by always storing Mv1 and Mv2 in those cases.

# Acknowledgement

We would like to thank Orange who accepted to cross-check this contribution.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).