| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0668-v3 |
| --- | --- |

| Title: | Non-CE2: CCLM latency reduction for single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thibaud Biatek, Geert Van Der Auwera, Marta Karczewicz | Email: | tbiatek@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes block-size dependant CCLM activation for single tree case. It enables to comply with 32x32 luma reconstruction latency during CLLM process. Results show limited losses over single-tree anchor with 0.05%(Y)/0.55%(U)/0.53%(V) BD-Rate in all-intra configuration

# Introduction

The reduction of luma-chroma latency for dual-tree case is addressed in CE2. For single-tree case CCLM luma reconstruction latency limit can be overtaken when chroma CU size is greater than 16x16. This proposal addresses this problematic by introducing conditions on CCLM activation depending on chroma CU dimensions.

# Proposed method

The proposed method enables CCLM for a chroma CU only when the its dimensions (W,H) respect the following condition:

W*H≤2561024

It guarantees that chroma block reconstruction using CCLM can be started after reconstruction of 32*32 luma samples.

# Specification changes

“

### 8.4.4 Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- a variable treeType specifying whether a single or a dual tree is used

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

A variable cclmEnabled is derived as follows:

- If treeType is single tree, the following applies:

  - If sps_cclm_enabled_flag is equal to 0, cclmEnabled is set equal to 0.

  - Otherwise, the following applies:

    - If (cbWidth/SubWidthC)*(cbHeight/SubHeightC) > 256, cclmEnabled is set equal to 0.

    - Otherwise, cclmEnabled is set equal to 1.

- Otherwise cclmEnabled is set equal to sps_cclm_enabled_flag

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is derived using IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] and sizeId set equal to MipSizeId[ xCb ][ yCb ] as specified in Table 8-4.

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6.

Table 8-5 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when cclmEnabledsps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when cclmEnabledsps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

“

# Experimental Results

The proposed method was integrated into VTM5.0 and tested under the CTC [1] against VTM-5 single-tree anchors. Results against VTM5.0 dual-tree are also attached to the contribution. Two tests are carried out:

- Test 1: Proposal enabled on I-Slices

- Test 2: Proposal enabled on all slices

Results are reported below.

## Test 1: I slices only

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 2.04% | 2.02% | 100% | #DIV/0! |
| Class A2 | 0.05% | 0.35% | 0.26% | 100% | #DIV/0! |
| Class B | 0.01% | 0.46% | 0.52% | 100% | #DIV/0! |
| Class C | -0.01% | 0.04% | -0.01% | 99% | 97% |
| Class E | 0.01% | 0.08% | 0.01% | 98% | 95% |
| Overall | 0.05% | 0.55% | 0.53% | 99% | #DIV/0! |
| Class D | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class F (optional) | 0.01% | 0.10% | -0.24% | 100% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.52% | 0.37% | 101% | #DIV/0! |
| Class A2 | 0.02% | 0.16% | 0.15% | 99% | #DIV/0! |
| Class B | 0.01% | 0.29% | 0.15% | 100% | 100% |
| Class C | -0.01% | 0.00% | 0.04% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.23% | 0.16% | 100% | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F (optional) | 0.08% | -0.09% | -0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.39% | 0.31% | 100% | #DIV/0! |
| Class C | 0.00% | -0.11% | -0.06% | 101% | #DIV/0! |
| Class E | -0.15% | -1.18% | -0.49% | 99% | 94% |
| Overall | -0.04% | -0.17% | -0.01% | 100% | #DIV/0! |
| Class D | 0.01% | 1.43% | 3.16% | 104% | 116% |
| Class F (optional) | 0.07% | -0.05% | -0.36% | 101% | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.28% | 0.21% | 100% | #DIV/0! |
| Class C | 0.00% | -0.03% | 0.10% | 101% | 103% |
| Class E | -0.10% | -0.19% | -0.90% | 99% | 105% |
| Overall | -0.03% | 0.06% | -0.10% | 100% | #DIV/0! |
| Class D | 0.07% | 1.91% | 2.17% | 103% | 113% |
| Class F (optional) | 0.09% | 0.05% | -0.47% | 99% | #DIV/0! |

## Test 2: all slices

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class B | 0.01% | 0.37% | 0.38% | 100% | 104% |
| Class C | -0.01% | 0.05% | 0.04% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F (optional) | 0.08% | -0.09% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | -0.01% | -0.08% | -0.02% | 101% | #DIV/0! |
| Class E | -0.12% | -1.35% | -0.58% | 97% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | 0.02% | 1.33% | 2.96% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM5.0 (singletree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | 0.01% | 0.10% | 0.04% | 100% | 100% |
| Class E | -0.07% | 0.00% | -0.68% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | 0.05% | 2.02% | 2.23% | 96% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |

# Conclusion

This contribution proposes to restrict CCLM usage in chroma CU when its dimensions respect 32*32 luma reconstruction latency constraints. Experimental results show limited impact on performance compared to VTM-5.0 single tree anchor with 0.05%(Y)/0.55%(U)/0.53%(V) BD-Rate in all-intra configuration.

# Acknowledgement

We would like to thank Interdigital for crosschecking our contribution.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sürhing, “JVET Common Test Condition and Software Reference Configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).