| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0670-v1 |
| --- | --- |

| Title: | CE7-related: Alternative joint coding of chroma residuals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Dr San Diego, CA, 92121, USA | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports an alternative version of joint coding of chroma residuals, where the method described in JVET-N0347, which is studied in test CE7-2.3, is only applied for intra blocks, and for non-intra blocks, the joint chroma coding is kept unchanged, i.e., the method in VTM-5.0 that was adopted from JVET-N0054. The results for CTC and lowQP test conditions compared to VTM-5.0 are reported below:

Table-1: Average BD rate results for CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.12% | -0.05% | 0.46% | 102% | 99% |
| RA | -0.02% | -0.66% | 0.37% | 102% | 102% |
| LD | 0.03% | 0.10% | 0.34% | 99% | 101% |

Table-2: Average BD rate results for lowQP (Pending)

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI |  |  |  |  |  |
| RA |  |  |  |  |  |
| LD |  |  |  |  |  |

# Introduction

From the results reported in JVET-O0115 (test CE7-2.3), it is observed that the proposed method works better for AI configuration, and provides a significant chroma loss in LD configuration. One of the underlying reasons is that the proposed method is only beneficial for intra blocks, but not so much for the non-intra blocks. Accordingly, in this document, we report the results when the proposed method of JVET-O0115 is only applied to intra blocks, while keeping the coding process for non-intra blocks unchanged.

In this case, a different joint chroma coding process is applied depending on whether the block is intra or not. So, we propose to have separate contexts for the joint_Cb_Cr residual flag, depending on whether the block is intra or not. Moreover, for intra blocks, we use two different contexts depending on whether cbf_cr is equal to 0 or 1, in order to exploit the correlation between joint_Cb_Cr and cbf_cr. At the encoder side, the lambda value is modified by 0.8x for intra blocks (compared to the block which uses regular chroma residual coding), as described in JVET-O0115.

# Experimental results

The proposed technique has been implemented on top of VTM5.0. The experimental results under the AI, RA and LDB configurations are reported in Table-3 for QP values {22, 27, 32, 37} and in Table-4 for low-Qp values {2, 7, 12, 17} (with first 100 frames), as required for CE7-2 experiments [3][4].

Table-3: BD rate results for CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.59% | 0.35% | 103% | 101% |
| Class A2 | -0.47% | -0.33% | 0.77% | 103% | 101% |
| Class B | -0.04% | -0.01% | 0.61% | 101% | 101% |
| Class C | -0.06% | 0.14% | 0.37% | 103% | 98% |
| Class E | -0.09% | 0.45% | 0.11% | 102% | 95% |
| Overall | -0.12% | -0.05% | 0.46% | 102% | 99% |
| Class D | -0.05% | 0.45% | -0.09% | 106% | 115% |
| Class F | -0.15% | 0.32% | 0.20% | 102% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -1.46% | 0.11% | 101% | 102% |
| Class A2 | -0.13% | -0.69% | 0.60% | 102% | 103% |
| Class B | 0.01% | -0.39% | 0.49% | 100% | 101% |
| Class C | -0.02% | -0.36% | 0.26% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.66% | 0.37% | 102% | 102% |
| Class D | 0.04% | 0.08% | 0.11% | 107% | 112% |
| Class F | -0.10% | 0.22% | 0.32% | 101% | 102% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.36% | 101% | 105% |
| Class C | 0.02% | -0.05% | 0.27% | 100% | 106% |
| Class E | 0.07% | 0.05% | 0.38% | 100% | 101% |
| Overall | 0.03% | 0.10% | 0.34% | 100% | 104% |
| Class D | 0.06% | 0.45% | 0.71% | 105% | 103% |
| Class F | 0.01% | 0.28% | 0.42% | 100% | 103% |

Table-4: BD rate results for lowQP (Pending)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.11% | 0.15% | 0.08% | 105% | 100% |
| Class C | -0.44% | 0.55% | 0.26% | 107% | 113% |
| Class E | -0.17% | 0.49% | 0.13% | 106% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.53% | 0.60% | -0.02% | 104% | 99% |
| Class F | -0.86% | 0.16% | -0.26% | 102% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.07% | 0.00% | 0.20% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.10% | 0.09% | -0.06% | 101% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04% | 0.02% | 0.14% | 102% | 105% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.03% | 0.06% | 98% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

HDR results are pending

In this proposed method, the loss in LDB configuration is reduced drastically compared to JVET-O0115. Although, compared to VTM-5.0, minor loss is observed for LDB. For AI and RA configuration, ClassA2 provides the highest performance improvement.

# References

- G. Van der Auwera et al., “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-N0347, Geneva, CH, Mar. 2019.

- J. Lainema, “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-N0054, Geneva, CH, Mar. 2019.

[3]   F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

[4]   H Schwarz, M. Coban, "Description of Core Experiment 7 (CE7): Quantization and coefficient coding", Document of Joint Video Experts Team, JVET-N1027.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).