| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0671-v3 |
| --- | --- |

| Title: | Non-CE3: Intra chroma mode coding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Dr San Diego, CA, 92121, USA | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the specification text related to intra chroma mode coding. The first proposal is to use the same intra_chroma_pred_mode for DM_CHROMA mode to improve the readability and consistency of the specification text. This proposal does not impact the compression performance. The second proposal uses a single binarization table for both sps_cclm_enabled_flag = 0 and 1. The results for the second proposal for CTC, compared to VTM-5.0, are reported below:

Table-1.a: Average BD rate results for CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.01% | -0.08% | -0.13% | 100% | 100% |
| RA | 0.05% | -0.35% | -0.47% | 101% | 101% |
| LD | 0.08% | -0.38% | -0.55% | 101% | 104% |

We have an update on the results with an updated context initialization with the same proposed method, and the results are shown below.

Table 1.b: Average BD rate results for CTC with updated context initialization.

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | -0.09% | -0.11% | 100% | 99% |
| RA | 0.00% | -0.24% | -0.30% | 99% | 100% |
| LD | 0.02% | 0.00% | -0.35% | 100% | 103% |

# Introduction

For chroma intra mode coding, the current VVC specification text associates DM_CHROMA mode with intra_chroma_pred_mode=4 for sps_cclm_enabled_flag=0 and intra_chroma_pred_mode=7 for sps_cclm_enabled_flag= 1, as described below in Table 2 and Table 3 (Table 8-5 and 8-6 in the draft) [1].

Table 2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Secondly, two different binarization tables are described for intra chroma mode coding, depending on sps_cclm_enabled_flag value, as described in Table 4 and Table 5 (Table 9-14 and 9-15 of the draft) [1].

Table 4 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |

Table 5 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 7 | 0 |
| 4 | 10 |
| 5 | 1110 |
| 6 | 1111 |
| 0 | 11000 |
| 1 | 11001 |
| 2 | 11010 |
| 3 | 11011 |

# Proposal 1

It is proposed to have intra_chroma_pred_mode=4 irrespective of sps_cclm_enabled_flag value. Thus the modified Table 8-6 of VVC draft would be as specified in Table 6.

Table 6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |
| 5 | 81 | 81 | 81 | 81 | 81 |
| 6 | 82 | 82 | 82 | 82 | 82 |
| 7 | 83 | 83 | 83 | 83 | 83 |

# Proposal 2

On top of proposal 1, we propose to use a single binarization table for sps_cclm_enabled_flag=0 and 1. The modified binarization table is shown below in Table 7.

Table 7 - unified binarization table.

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 00 |
| 0 | 0100 |
| 1 | 0101 |
| 2 | 0110 |
| 3 | 0111 |
| 5 | 10 |
| 6 | 110 |
| 7 | 111 |

Here, the first bin indicates whether it is regular (0) or LM modes (1). If it is regular mode, then we derive the next bins the same way as described in Table 4. If it is LM mode, then the next bin indicates whether it is LM_CHROMA (0) or not. If it is not LM_CHROMA, next 1 bin indicates whether it is LM_L (0) or LM_A (1).  For this case, when sps_cclm_enabled_flag is 0, the first bin of the binarization table for the corresponding intra_chroma_pred_mode can be discarded prior to the entropy coding. Or, in other words, the first bin is inferred to be 0 and hence not to be coded. It also implies that for sps_cclm_enabled_flag=0, the binarization is unchanged. This single binarization table works for both sps_cclm_enabled_flag 0 and 1 case.

Please note that, with this new binarization, DM_CHROMA (4) uses 1 bin more than the original binarization, LM_CHROMA uses the same number of bins, while the other 6 remaining modes uses 1 less bin.

For our current implementation, we have used three contexts (Table 8.a): red: context 1, blue: context 2, green: context 3, black: bypass coded. Please note that the original chroma mode coding uses the same number of contexts (i.e., 3) as our proposal.

Table 8.a - Contexts for new binarization.

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 00 |
| 0 | 0100 |
| 1 | 0101 |
| 2 | 0110 |
| 3 | 0111 |
| 5 | 10 |
| 6 | 110 |
| 7 | 111 |

Accordingly, In the VTM5 draft [1], in the Table 9.17, for intra chroma mode coding, we propose the following changes for the context selections of bin (Table 8.b).

Table 8.b - Context selection for bins related to intra chroma mode coding

| binIdx | 0 | 1 | 2 | 3 | 4 | >=5 |
| --- | --- | --- | --- | --- | --- | --- |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 0 = = 0 | 0 | 1 | bypass | bypass | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 0 = = 1 | 0 | 2 | bypass | na | na | na |

# Experimental results

Proposal 2 has been implemented on top of VTM5.0. The experimental results under the AI, RA and LDB configurations are reported in Table-3 for QP values {22, 27, 32, 37}, as described for CTC [2].

Table 9.a - BD rate results for CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.18% | -0.25% | 100% | 101% |
| Class A2 | -0.01% | -0.03% | -0.04% | 100% | 102% |
| Class B | 0.00% | -0.09% | -0.12% | 100% | 101% |
| Class C | 0.00% | -0.11% | -0.18% | 100% | 97% |
| Class E | 0.00% | 0.02% | -0.07% | 100% | 96% |
| Overall | -0.01% | -0.08% | -0.13% | 100% | 100% |
| Class D | 0.00% | -0.01% | -0.16% | 100% | 102% |
| Class F | -0.03% | -0.07% | -0.12% | 100% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.35% | -0.44% | 100% | 100% |
| Class A2 | 0.05% | 0.07% | 0.09% | 102% | 102% |
| Class B | 0.07% | -0.39% | -0.64% | 99% | 99% |
| Class C | 0.05% | -0.62% | -0.70% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.35% | -0.47% | 101% | 101% |
| Class D | 0.12% | -0.85% | -0.36% | 103% | 103% |
| Class F | 0.04% | -0.03% | -0.09% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.17% | -0.74% | 101% | 103% |
| Class C | 0.11% | -0.41% | -0.91% | 101% | 108% |
| Class E | 0.08% | -0.68% | 0.27% | 100% | 103% |
| Overall | 0.08% | -0.38% | -0.55% | 101% | 104% |
| Class D | 0.10% | -0.48% | -0.20% | 101% | 95% |
| Class F | -0.01% | -0.94% | -1.08% | 98% | 96% |

Table 9.b – Updated BD rate results for CTC with updated context initialization

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.13% | -0.28% | 100% | 100% |
| Class A2 | 0.00% | -0.09% | -0.03% | 100% | 102% |
| Class B | 0.00% | -0.12% | -0.11% | 100% | 101% |
| Class C | 0.00% | -0.13% | -0.03% | 100% | 96% |
| Class E | 0.00% | 0.04% | -0.12% | 101% | 98% |
| Overall | 0.00% | -0.09% | -0.11% | 100% | 99% |
| Class D | -0.01% | 0.04% | -0.05% | 99% | 101% |
| Class F | -0.01% | -0.05% | -0.12% | 99% | 96% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.47% | -0.31% | 98% | 100% |
| Class A2 | 0.01% | 0.14% | 0.04% | 101% | 103% |
| Class B | 0.03% | -0.27% | -0.50% | 97% | 97% |
| Class C | -0.04% | -0.33% | -0.29% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.24% | -0.30% | 99% | 100% |
| Class D | 0.06% | -0.50% | 0.08% | 102% | 103% |
| Class F | -0.02% | -0.03% | 0.03% | 100% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.16% | -0.29% | 101% | 101% |
| Class C | -0.01% | -0.16% | -0.41% | 100% | 107% |
| Class E | 0.10% | -0.05% | -0.35% | 100% | 103% |
| Overall | 0.02% | 0.00% | -0.35% | 100% | 103% |
| Class D | 0.00% | -0.69% | 0.39% | 101% | 93% |
| Class F | 0.02% | -0.19% | -0.30% | 97% | 93% |

For AI configuration, the proposal brings minor gain, particularly for ClassA1 sequences. For Table 9.a., for RA and LD configurations, minor luma loss and considerable chroma gain is observed. With the updated context initialization, in Table 9.b., for RA, there is no luma loss, and for LD, the luma loss is also reduced compared to Table 9.a. We propose to use the configuration with updated contexts (Table 9.b).

Please note that although the main motivation here is to simplify the specification text related to intra chroma mode coding, the method provides competitive performance compared to the original chroma mode coding method of VTM-5.0.

# Acknowlegements

# The authors would like to thank ATEME for performing the crosscheck.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” 14th JVET Meeting, Geneva, CH, Mar. 2019, JVET-N1001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).