| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0672 |
| --- | --- |

| Title: | Non-CE3: MRL intra prediction for chroma DM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Chang Kuo, Ching-Chieh Lin1, Chun-Lung Lin2 | Email: | JackLin@itri.com1 chunlung@itri.com2 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

Chroma direct mode (DM) is a mode to directly use the intra prediction mode of the corresponding luma block. This contribution proposed to apply multiple reference line (MRL) intra prediction for chroma DM if its corresponding luma block uses non-zero reference line index. CTC results of the proposed method show 0.00% (Y), -0.06% (U), -0.01% (V) and 0.00% (Y), -0.06% (U), -0.09% (V) in AI and RA test conditions respectively. This contribution also tested YUV 4:4:4 and 4:2:2 contents. For YUV 4:4:4 content, the results show -0.02% (Y), 0.01% (U), 0.03% (V) and 0.02% (Y), -0.01% (U), 0.04% (V) in AI and RA test conditions respectively. For YUV 4:2:2 content, the results show 0.00% (Y), 0.00% (U), 0.00% (V) and 0.00% (Y), 0.00% (U), -0.05% (V) in AI and RA test conditions respectively.

Introduction

Multiple reference line (MRL) uses extended reference lines to predict a CU block in the intra prediction as shown in Fig. 1. A CU selects one of the available reference lines for predictor generation and the reference line index is signalled to indicate which reference line is used. In the current VVC draft [1], MRL is only applied to luma coding block, and the reference line 0 is always used for intra prediction of chroma block.

A direct mode (DM) is designed in VVC to explore the correlation between the chroma and luma blocks. Chroma DM directly uses the intra prediction mode of the corresponding luma block when the texture direction of the chroma block is similar to the luma block. However, the current DM only can use the nearest reference line. Therefore, this contribution extended the MRL tool to chroma DM if its corresponding luma block uses a non-zero reference line index.

Fig. 1. Concept of multiple reference line.

Proposed Method

Currently in VVC [1], the reference line index (refIdx) is constantly set equal to 0 for chroma intra prediction. The proposed method derives the reference line index of chroma by the following pseudo-code.

refIdxL = reference line index of the corresponding luma block (in DM position) for current chroma block isChromaDM = (intra prediction mode of current chroma block is DM)? 1: 0 if ( isChromaDM ) refIdx = (chroma format is 4:4:4) ? refIdxL : (refIdxL>>1) else refIdx = 0

Experimental Results

The implementation was on top of the VTM 5.0 software. The simulations were launched under common test condition (CTC) [2]. Table 1 shows the experimental results of the proposed method. It reports 0.00% (Y), -0.06% (U), -0.01% (V) and 0.00% (Y), -0.06% (U), -0.09% (V) in AI and RA test conditions respectively. There is no impact on encoding and decoding time.

Evaluation of YUV 4:4:4 and YUV 4:2:2 compression performance was done by using the JCT-VC Range Extension (RExt) CTC [3] test set. Table 2 shows the experimental results of the proposed method for YUV 4:4:4 and YUV 4:2:2 content. For YUV 4:4:4 content, the results show -0.02% (Y), 0.01% (U), 0.03% (V) and 0.02% (Y), -0.01% (U), 0.04% (V) in AI and RA test conditions respectively. For YUV 4:2:2 content, the results show 0.00% (Y), 0.00% (U), 0.00% (V) and 0.00% (Y), 0.00% (U), -0.05% (V) in AI and RA test conditions respectively.

Table 1: Summary results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | -0.04% | 100% | 100% |
| Class B | 0.00% | -0.05% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.11% | 0.04% | 100% | 100% |
| Class E | -0.01% | -0.06% | -0.05% | 100% | 100% |
| Overall | 0.00% | -0.06% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.16% | 0.00% | 100% | 100% |
| Class F | -0.01% | -0.03% | 0.01% | 100% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | 0.13% | 101% | 100% |
| Class A2 | 0.03% | -0.01% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.01% | -0.22% | 101% | 101% |
| Class C | -0.02% | -0.12% | -0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.09% | 101% | 100% |
| Class D | 0.01% | -0.09% | 0.00% | 100% | 100% |
| Class F | 0.03% | 0.07% | 0.10% | 100% | 100% |

Table 2: Summary results of the proposed method for YUV 4:4:4 and YUV 4:2:2 content

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.02% | 0.01% | 0.03% | 0.99% | 1.00% |
| YUV 4:2:2 | 0.00% | 0.00% | 0.00% | 0.99% | 1.01% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.02% | -0.01% | 0.04% |  |  |
| YUV 4:2:2 | 0.00% | 0.00% | -0.05% |  |  |

Conclusions

This contribution inherited the reference lilne index of intra prediction for chroma DM from its corresponding luma block. The simulation results are reported. It is suggested to further study the chroma MRL.

Working Draft Changes

The proposed method can be represented as following working draft(v8) changes. Changes are highlighted in green.

8.4.4 Derivation process for chroma intra prediction mode … In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] and the chroma reference line index IntraChromaRefLineIdx[ x ][ y ] for x = xCb.. xCb + cbWidth − 1 and y = yCb.. yCb + cbHeight − 1 are derived. … The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6. The variable isChromaDM is derived as follows: - If one of the following conditions is true, isChromaDM is set equal to 1: - sps_cclm_enabled_flag is equal to 0 and intra_chroma_pred_mode[ xCb ][ yCb ] is equal to 4 - sps_cclm_enabled_flag is equal to 1 and intra_chroma_pred_mode[ xCb ][ yCb ] is equal to 7 - Otherwise, isChromaDM is set equal to 0 The chroma reference line index IntraChromaRefLineIdx[ x ][ y ] for x = xCb.. xCb + cbWidth − 1 and y = yCb.. yCb + cbHeight – 1 is derived as follows: if ( isChromaDM ) IntraChromaRefLineIdx[ x ][ y ] = ( chroma_format_idc = = 3 ) ? IntraLumaRefLineIdx[ xCb ][ yCb ] : (IntraLumaRefLineIdx[ xCb ][ yCb ]>>1) else IntraChromaRefLineIdx[ x ][ y ] = 0 … 8.4.5.2.5 General intra sample prediction … refIdx = ( cIdx = = 0 ) ? IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ] : IntraChromaRefLineIdx[ xTbCmp * SubWidthC ][ yTbCmp * SubHeightC ] (8-96) …

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, the 14th JVET meeting, Geneva, CH, 19–27 Mar. 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Geneva, CH, 19–27 Mar. 2019.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, the 16th JCT-VC Meeting, San Jose, CA, January 2014.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).