| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0677 |
| --- | --- |

| Title: | Crosscheck of JVET-O0191 (CE1-related: Simplified CABAC initialization method) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong | Tel: Email: | +1 408.533.9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the crosscheck of JVET-O0191. The implementation in VTM5.0 and standalone SIMD code were inspected and confirmed to be consistent with the technical description. The coding efficiency results provided by the proponent were verified.

# Comments

JVET-O0191 proposes a simplified CABAC initialization method, of which the initialization values have 6-bit precision and the output probability states are in the linear domain. The details are shown as below.

a = (initValue >> 3) * 9

b = (initValue & 7) * 9

q = Clip3(8, 24, QP >> 1) – 8

s = 16 * a + (b – a) * q + 8

pStateIdx0 = s

pStateIdx1 = s << 4

The implementation in VTM5.0 matches the technical description, and the initialization values used can be reproduced using the training tool provided by CE1.

In JVET-O0191, the compression efficiency is measured according to the criteria defined in CE1, including a full test under CTC and two RA tests unnder low and high QPs, respectively. The results provided by the proponent match those generated by the crosschecker (see the enclosed Excel spreadsheets).

The SIMD code provided in Section 3.2 is confirmed to be consistent with the technical description. However, the running time comparison with other CE1 tests cannot be verified, because the SIMD codes for other CE1 tests are not provided.