| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0681 |
| --- | --- |

| Title: | CE9-related: Disabled DMVR, BDOF and BCW for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the Core Experiment (CE) 9-2.2 on complexity reduction on combined inter and intra prediction (CIIP), it’s tested to disable decoder-side motion vector derivation (DMVR) and bi-directional optical flow (BDOF) when generating the inter prediction samples in the CIIP mode. In this contribution, it’s proposed to further disable Bi-prediction with CU-level weight (BCW) when generating the inter prediction samples in the CIIP mode. The average BD-rate change of the proposed method is {-0.01%, -0.08%, -0.08%} for RA and {0.03%, 0.09%, -0.32%} for LDB, respectively. The average BD-rate change of the proposed method 2 is {-0.01%, -0.07%, -0.04%} for RA and {-0.02%, 0.11%, 0.04%} in LDB, respectively.

# Introduction

In the Core Experiment (CE) 9-2.2 on complexity reduction on combined inter and intra prediction (CIIP), it’s tested to disable decoder-side motion vector derivation (DMVR) and bi-directional optical flow (BDOF) when generating the inter prediction samples in the CIIP mode. In this contribution, it’s proposed to further disable Bi-prediction with CU-level weight (BCW) when generating the inter prediction samples in the CIIP mode.

In method 1, the BCW index in the merge candidate for CIIP is set to 0.

In method 2, when generating the inter prediction samples in the CIIP mode, equal weighted averaging is applied in bi-prediction regardless the BCW index.

# Proposed text changes

Added texts are highlight in green.

#### 8.5.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when general_merge_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index bcwIdx.

- the merging candidate list mergeCandList.

The bi-prediction weight index bcwIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for spatial merging candidates from neighbouring coding units as specified in clause 8.5.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices bcwIdxA0, bcwIdxA1, bcwIdxB0, bcwIdxB1, bcwIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index bcwIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-263)

predFlagL0Col = availableFlagL0Col		(8-264)

predFlagL1Col = 0		(8-265)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-266)

predFlagL1Col = availableFlagL1Col		(8-267)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-268)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than MaxNumMergeCand and greater than 1, the following applies:

- The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index bcwIdx of candidate avgCand being added into mergeCandList is set equal to 0.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for zero motion vector merging candidates specified in clause 8.5.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index bcwIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-269)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-270)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-271)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-272)

bcwIdx = ciip_flag[ xCb ][ yCb ] == 1 ? 0 : bcwIdxN 		(8-273)

- When mmvd_merge_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.5.2.7 is invoked with the luma location ( xCb, yCb ), the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-274)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-275)

mvLX[ 0 ][ 0 ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 0 ] )	(8-276)

mvLX[ 0 ][ 0 ] [ 1 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 1 ] ) 	(8-277)

### 8.5.1 General decoding process for coding units coded in inter prediction mode

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- ciip_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

### 8.5.6.1 General

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

- ciip_flag[ xCb ][ yCb ] is equal to 0

- BcwIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

- cbHeight is greater than or equal to 8

- cIdx is equal to 0.

Test change for method 2:

##### 8.5.6.5.2 Default weighted sample prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bi-prediction weight index bcwIdx.

- the sample bit depth, bitDepth.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

Variables shift1, shift2, offset1, offset2, and offset3 are derived as follows:

- The variable shift1 is set equal to Max( 2, 14 − bitDepth ) and the variable shift2 is set equal to Max( 3, 15 − bitDepth ).

- The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

- The variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

- The variable offset3 is set equal to 1  <<  ( shift2 + 2 ).

Depending on the values of predFlagL0 and predFlagL1, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 )  >>  shift1 )	(8-787)

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 )  >>  shift1 )	(8-788)

- Otherwise (predFlagL0 is equal to 1 and predFlagL1 is equal to 1), the following applies:

- If bcwIdx is equal to 0 or ciip_flag[ xCb ][ yCb ] is equal to 1, the prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, 	(8-789)
					( predSamplesL0[ x ][ y ] + predSamplesL1[ x ][ y ] + offset2 )  >>  shift2 )

- Otherwise (bcwIdx is not equal to 0 and ciip_flag[ xCb ][ yCb ] is equal to 0), the following applies:

- The variable w1 is set equal to bcwWLut[ bcwIdx ] with bcwWLut[ k ] = { 4, 5, 3, 10, −2 }.

- The variable w0 is set equal to ( 8 − w1 ).

- The prediction sample values are derived as follows.

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, 	(8-790)
		( w0*predSamplesL0[ x ][ y ] + w1*predSamplesL1[ x ][ y ] + offset3 )  >>  (shift2+3) )

# Test results

The proposed method is implemented on top of CE9-2.2 software. The average BD-rate change of the proposed method 1 is {-0.01%, -0.08%, -0.08%} for RA and {0.03%, 0.09%, -0.32%} for LDB, respectively. The average BD-rate change of the proposed method 2 is {-0.01%, -0.07%, -0.04%} for RA and {-0.02%, 0.11%, 0.04%} in LDB, respectively. Details of the test results are shown in the following tables.

Table 3-1 Test result of method 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.20% | 0.08% | 106% | 105% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 101% |
| Class B | -0.01% | 0.02% | -0.22% | 102% | 103% |
| Class C | -0.03% | -0.16% | -0.07% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.08% | -0.08% | 102% | 101% |
| Class D | 0.00% | 0.01% | 0.22% | 102% | 101% |
| Class F | 0.02% | 0.04% | 0.04% | 103% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.31% | -0.07% | 101% | 106% |
| Class C | 0.04% | -0.10% | -0.15% | 100% | 102% |
| Class E | 0.05% | -0.03% | -0.96% | 100% | 106% |
| Overall | 0.03% | 0.09% | -0.32% | 101% | 105% |
| Class D | 0.00% | 0.26% | 0.29% | 102% | 100% |
| Class F | -0.03% | -0.16% | -0.48% | 104% | 106% |

Table 3-2 Test result of method 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.08% | 105% | 104% |
| Class A2 | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.05% | -0.12% | 101% | 103% |
| Class C | -0.02% | -0.15% | -0.07% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.04% | 101% | 100% |
| Class D | 0.00% | -0.02% | 0.27% | 101% | 101% |
| Class F | 0.02% | 0.03% | 0.05% | 102% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.25% | 0.34% | 101% | 104% |
| Class C | -0.04% | -0.04% | -0.10% | 100% | 99% |
| Class E | -0.01% | 0.06% | -0.26% | 100% | 104% |
| Overall | -0.02% | 0.11% | 0.04% | 101% | 102% |
| Class D | 0.03% | -0.06% | 0.66% | 101% | 100% |
| Class F | -0.09% | -0.05% | -0.57% | 103% | 104% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).