| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0682 |
| --- | --- |

| Title: | CE8-related: An Intra-Affine mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Cao, Zhengren Li, Jun Wang, Fan Liang, Yuanfang Yu, Yang Liu | Email: | caoj33@mail2.sysu.edu.cn lizhr7@mail2.sysu.edu.cn wangj387@mail.sysu.edu.cn isslf@mail.sysu.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | NELVT Guangzhou Research and Industrialization Center, Sun Yat-Sen University, OPPO |  |  |

_____________________________

- Abstract

In this contribution, a new intra-affine mode is proposed. In this new mode, affine model is introduced to intra prediction for SCC. The current partially encoded picture is considered as the only reference picture. The use of intra-affine mode is signaled by using the IBC mode flag [1] and the Affine flag. These two flags are both true when the proposed mode is used. The proposed intra-affine mode is implemented on VTM-5.0. The simulations show BD-rate gains for VTM-5.0 anchor (IBC mode enabled) are:

- In AI, -2.03% / -1.99% / -2.03% for Y, U, V component of SCC 1080p, separately.

- In AI, -0.19% / -0.24% / -0.18% for Y, U, V component of Class F, separately.

- In RA, -0.17% / -0.15% / -0.23%  for Y, U, V component of SCC 1080p, separately.

- In RA, -0.61% / -0.61% / -0.67%  for Y, U, V component of Class F, separately.

Introduction

Intra block copy (IBC) or current picture referencing (CPR), as a tool for SCC, is proved to be efficient when there are many repeating areas within the same picture. However, IBC is based on translational motion model, which can not describe some complicated movement accurately, such as rotation and zoom.

In JVET-M0634 [2], we preliminary proposed a method which combined the affine motion mode with CPR (CPRAffine). However, the improvement of coding efficiency of CPRAffine was not significant. In this contribution, we further improve the process and propose the intra-affine mode.

Proposed methods

Encoder aspect:

The intra-affine mode is an independent mode rather than embedded in the IBC mode in [2]. The encoder performs RD check for blocks whose sizes are equal to or larger than  and finds the best mode among three modes (regular intra mode, IBC mode and intra-affine mode).

As shown in Fig. 1, the process of the proposed mode can be divided into two main steps: selecting initial BVAffis (block-affine vectors) and iterative searching for the best BVAffi. Initial BVAffis can be obtained by inheritance and local search. The way of iterative searching is similar to the affine motion inter prediction. The outstanding differences are that the reference picture is the current picture itself and boundary checking is applied in the proposed mode.

- Fig. 1  The overall algorithm flowchart

Decoder aspect:

If the affine flag and the IBC mode flag are both true, the proposed intra-affine mode can be inferred.

- Fig. 2  Inference of the intra-affine mode

Experimental Results

The proposed intra-affine mode is implemented on VTM-5.0. QP values are chosen as 22, 27, 32 and 37 in AI and RA case. Some other test configurations and test sequences follow the requirement of reference [3]. VTM-5.0 (IBC mode enabled) is used as the anchor. Results are shown in the following tables:

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.19% | -0.24% | -0.18% | 129% | 99% |
| Class SCC 1080p | -2.03% | -1.99% | -2.03% | 120% | 100% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.17% | -0.15% | -0.23% | 102% | 99% |
| Class SCC 1080p | -0.61% | -0.61% | -0.67% | 101% | 101% |

Conclusions

In this contribution, a new intra-affine mode is proposed and tested. Sigificant coding gains for screen content materials are observed from the experimental results. It is recommended to include this method into the test model for further consideration.

References

[1] W. Chien, V. Seregin and M. Karczewicz, " CE8-related: CPR mode signaling and interaction with inter coding tools," JVET-M0483, Marrakech, MA, Jan. 2019.

[2]	S. Cao, H. Han, J. Wang, F. Liang, Y. Yu and Y. Liu, "Affine motion mode in intra coding," JVET-M0634, Marrakech, MA, Jan. 2019.

[3]	X. X, Y. Sun, Y. Chao and J. Xu, " Description of Core Experiment 8: Screen Content Coding Tools," JVET-L1028, Macao, CN, Oct. 2018.

Patent rights declaration(s)

OPPO and NELVT Guangzhou Research and Industrialization Center may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sun Yat-Sen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).