| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0693 |
| --- | --- |

| Title: | Crosscheck of JVET-O0598 (CE3-related: Simplification on MIP Look-up Table) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liang Zhao | Email: | leolzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-O0598, which removes three tables in MIP module: g_intraMode65to33AngMapping, g_mapMipToAngular65 and g_mapAngular33ToMip. The software implementation as provided by the proponent of JVET-O0598 has been studied and confirmed to be aligned with the proposal description. Simulation results as provided in this contribution are reportedly matching with the results provided by proponent of JVET-O0598.

Crosscheck report

In JVET-O0598, a method is proposed to remove these three LUTs, g_intraMode65to33AngMapping, g_mapMipToAngular65, and g_mapAngular33ToMip. Instead of performing mode conversion through LUT operations, a neighboring block with MIP mode is simply considered as Planar mode when constructing conventional intra MPM list, and a neighboring block with a conventional intra mode is considered as “Not Available” when constructing MIP MPM list. In deriving an intra chroma mode where the corresponding luma intra mode is needed, the corresponding luma intra mode is also considered as Planar if it is a MIP mode.

The SW implementation was provided by the proponent of JVET-O0598 and simulations are performed under common test condition (CTC) [1]. Two tests in JVET- O0598 have been crosschecked, which are listed in Table 1 and Table 2.

Table 1: Results of Test 1 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.10% | 99% | 100% | 0.01% | 0.00% | 0.14% | 99% | 101% |
| Class A2 | 0.03% | -0.01% | -0.02% | 99% | 100% | 0.05% | 0.19% | 0.12% | 99% | 100% |
| Class B | 0.01% | 0.01% | 0.05% | 99% | 102% | 0.03% | 0.03% | 0.08% | 100% | 101% |
| Class C | 0.00% | -0.01% | 0.07% | 99% | 102% | -0.05% | 0.03% | -0.07% | 100% | 101% |
| Class E | 0.02% | -0.07% | -0.08% | 100% | 105% |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.03% | 99% | 102% | 0.01% | 0.06% | 0.06% | 100% | 101% |
| Class D | -0.01% | 0.04% | 0.02% | 99% | 101% | 0.03% | 0.09% | 0.10% | 100% | 104% |
| Class F | 0.00% | 0.00% | 0.04% | 100% | 105% | -0.01% | -0.09% | 0.20% | 100% | 104% |

Table 2: Results of Test 2 over JVET-O0084

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.16% | 99% | 100% | 0.02% | 0.06% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.04% | 0.02% | 99% | 100% | 0.06% | 0.27% | 0.14% | 99% | 100% |
| Class B | 0.01% | 0.04% | 0.03% | 99% | 100% | 0.01% | 0.11% | 0.07% | 99% | 99% |
| Class C | 0.03% | -0.01% | -0.01% | 99% | 95% | 0.01% | -0.07% | 0.09% | 100% | 97% |
| Class E | 0.02% | -0.02% | 0.06% | 99% | 98% |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.05% | 99% | 98% | 0.02% | 0.08% | 0.07% | 99% | 99% |
| Class D | 0.02% | 0.13% | -0.12% | 100% | 100% | 0.04% | 0.35% | 0.06% | 100% | 102% |
| Class F | -0.02% | -0.02% | 0.04% | 100% | 100% | 0.01% | 0.18% | -0.03% | 100% | 100% |

References

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.