| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0699 |
| --- | --- |

| Title: | Non-CE6: Modified DCT8/DST7 Transform Matrices for Faster Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser*, Louis Kerofsky+, Fabrice Le Léannec*, Tangi Poirier* *975 Avenue des champs blancs 35576 Cesson-Sévigné, France +9276 Scranton Road Suite 300 San Diego, CA 92121 | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a slight modification to the DCT8/DST7 matrices so that faster implementation can be achieved. Similar to the current fast transform design, the proposed modification allows dual implementation support, in which the transform can be either implemented by matrix multiplication or a fast algorithm to yield exactly the same results.

Experimental results reportedly show 0.00% and 0.01% RD loss in RA and AI respectively when the fast algorithm for 8x8 DCT8/DST7 is used, while the number of multiplications is reduced by 61% compared to matrix multiplications.

# Introduction

In VTM5.0 decoder, DCT8 and DST7 are supported for residual blocks of sizes 4x4 to 32x32. The fast algorithm of 4x4 is inherited from HEVC, while the 16x16 and 32x32 fast algorithm is the one adopted in Marrakesh meeting (JVET-M0497). However, VTM5.0 doesn’t support any fast algorithm for 8x8 DCT8/DST7.

With the recent adoption of LFNST and MIP, it is proposed in JVET-O0544 and JVET-O0547 to modify the matrices in order to reduce the number of multiplications. This contribution proposes to apply the same concept for DCT8/DST7. By doing so, a fast algorithm for size 8x8 can be obtained and the number of operations for DCT8/DST7 of sizes 16x16 and 32x32 is reduced.

# Proposition

The contribution proposes fast algorithms for the modified versions of DCT8/DST7 transform matrices. The fast algorithms have the following properties:

- Dual implementation support: the transform can be implemented by both fast algorithm and matrix multiplication with exactly the same results

- No cascading of operations is required to perform the fast algorithm

- Independent computation of transform coefficients, which enables parallel computation

- DCT8/DST7 of size NxN can be represented by N coefficients, exactly as the current design

- DCT8/DST7 relationship is maintained: DST7 matrix is obtained by flipping DCT8 matrix with negating odd rows functions

To show the advantage of the proposed method, we compare the number of multiplications for computing DCT8/DST7 in VTM5.0 and the proposed method in the table below:

Table 1 Number of multiplications required for the DCT8/DST7 transforms

| DCT8/DST7 | JVET-O0544 | Proposed |
| --- | --- | --- |
| 8x8 | 64 | 25 |
| 16x16 | 127 | 121 |
| 32x32 | 620 | 544 |

As indicated in Table 1, the number of operations for computing the transform is reduced: 6%, 15% and 12% for 8x8, 16x16 and 32x32.

# Implementation details

The fast algorithm is obtained by modifying the transform matrices such that the number following features (F1,F2,F3,F4 and F5) is maximized:

- F1: a=1<<w

- F2: b=a<<w

- F3: c=a<<w + b<<x

- F4: d=a<<w + b<<x + c<<y

- F5: e=a<<w + b<<x + c<<y + d<<z

Where a,b,c,d and e are the absolute value of the coefficients of one row of the transform matrix, and w,x,y and z are shift values. It should be noted the F2 and F3 are same feature that are used for the design the fast algorithm of VTM software (JVET-M0497). However, the shift is not considered. Therefore, this contribution proposes an extension of the feature list so that the number of multiplication is further reduced. Additionally, the method is applicable to 8x8 transform, where no fast algorithm is currently available.

To have a clear idea of the fast algorithm, the implementation of DCT8 of size 8x8 is presented. Below is the code used for the fast algorithm:

dst[line*0]=(71*(-src[0]*(6)+src[1]*(3)+(src[2]<<1)+src[3])+46*((src[4]<<1)+src[5])+(src[0]<<9)-(src[1]<<7)-(src[2]<<6)-(src[4]<<5)+(src[6]<<5)+(src[7]<<4) + add ) >> shift;

dst[line*1]=(-71*(src[0]+src[4])+-86*(src[5])+-78*(-(src[0]<<1)+src[6])+-46*(-(src[1]<<1)+src[7])-(src[1]<<5)+(src[2]<<4)-(src[3]<<5) + add ) >> shift;

dst[line*2]=(85*(src[0]+(src[2]<<3)+(src[3]<<1)-src[4]+src[6])+71*(-src[0]+(src[2]<<2)+src[4]+src[7])+(src[0]<<6)+(src[1]<<4)-(src[2]<<10)-(src[3]<<8)-(src[4]<<5)+(src[5]<<5) + add ) >> shift;

dst[line*3]=(60*(src[5])+-46*((src[0]<<2)+(src[4]<<1)+src[6])+-85*(-src[0]*(3)-(src[2]<<1)-(src[4]<<1)+src[7])-(src[1]<<5)-(src[2]<<8)-(src[3]<<4) + add ) >> shift;

dst[line*4]=(78*(-(src[0]<<3)-(src[1]<<1)-src[2]+src[3])+-85*(-(src[0]<<2)-src[1]+src[5])+86*((src[0]<<2)+src[7])+(src[2]<<5)+(src[4]<<5)-(src[6]<<4) + add ) >> shift;

dst[line*5]=(71*(-(src[1]<<1)+(src[3]<<2)+src[4]+src[6])+-78*(-src[0]-(src[1]<<2)+(src[3]<<4)+(src[4]<<1)+src[7])-(src[0]<<5)-(src[1]<<8)+(src[2]<<5)+(src[3]<<10)+(src[5]<<4) + add ) >> shift;

dst[line*6]=(71*(-(src[1]<<1)-src[2]+(src[3]<<1)+src[5])+-86*(-(src[2]<<1)+(src[3]<<1)+src[6])+60*(src[7])+(src[0]<<5)+(src[1]<<6)-(src[2]<<4)-(src[3]<<4)-(src[4]<<4) + add ) >> shift;

# dst[line*7]=(-85*(src[2]+src[3])+-78*(src[1]-(src[2]<<1)-src[4]+src[5])+60*(-(src[4]<<1)+src[6])+(src[0]<<4)+(src[1]<<5)+(src[4]<<7)-(src[7]<<5) + add ) >> shift;

This fast algorithm requires 25 multiplications, 91 additions and 61 shifts. The default matrix multiplication implementation requires 64 multiplications and 64 additions. Thus, the fast algorithm reduces the number of multiplications by 61% of its original value, while adding more additions and shifts.

The algorithm can also be implemented with without increasing the number of additions. This is by considering up to F2. This results in the following algorithm:

dst[line*0]= (86*(src[0])+85*(src[1])+78*(src[2])+71*(src[3])+60*(src[4])+46*(src[5])+(src[6]<<5)+(src[7]<<4) + add ) >> shift;

dst[line*1]= (85*(src[0])+60*(src[1])+-71*(src[4])+-86*(src[5])+-78*(src[6])+-46*(src[7])+(src[2]<<4)-(src[3]<<5) + add ) >> shift;

dst[line*2]= (78*(src[0])+-60*(src[2])+-86*(src[3])+-46*(src[4])+85*(src[6])+71*(src[7])+(src[1]<<4)+(src[5]<<5) + add ) >> shift;

dst[line*3]= (71*(src[0])+-86*(src[2])+78*(src[4])+60*(src[5])+-46*(src[6])+-85*(src[7])-(src[1]<<5)-(src[3]<<4) + add ) >> shift;

dst[line*4]= (60*(src[0])+-71*(src[1])+-46*(src[2])+78*(src[3])+-85*(src[5])+86*(src[7])+(src[4]<<5)-(src[6]<<4) + add ) >> shift;

dst[line*5]= (46*(src[0])+-86*(src[1])+60*(src[3])+-85*(src[4])+71*(src[6])+-78*(src[7])+(src[2]<<5)+(src[5]<<4) + add ) >> shift;

dst[line*6]= (-78*(src[1])+85*(src[2])+-46*(src[3])+71*(src[5])+-86*(src[6])+60*(src[7])+(src[0]<<5)-(src[4]<<4) + add ) >> shift;

dst[line*7]= (-46*(src[1])+71*(src[2])+-85*(src[3])+86*(src[4])+-78*(src[5])+60*(src[6])+(src[0]<<4)-(src[7]<<5) + add ) >> shift;

This fast algorithm requires 48 multiplications, 56 additions and 16 shifts. Thus, the number of additions and shifts are reduced with increased multiplications. The two algorithms yield exactly the same results, which is the same as matrix multiplication.

In order to fully compare the number of operations, the number of multiplications, additions and subtractions for the fast algorithm and the one of VTM (JVET-M0497) are compared in the tables below, where Proposed1 is the fast algorithm that considers all the features, while Proposed2 considers up to the second feature (F2).

Table 2 Number of operations for Proposed1, Proposed2 and VTM for DCT8/DST7 of length 8

| Method |  | #Mutliplications |  | #Additions |  | #Shifts |
| --- | --- | --- | --- | --- | --- | --- |
| Proposed 1 |  | 25 |  | 91 |  | 61 |
| Proposed 2 | 48 | 56 | 16 |
| VTM | 64 | 64 | 0 |

Table 3 Number of operations for Proposed1, Proposed2 and VTM for DCT8/DST7 of length 16

| Method | #Mutliplications | #Additions | #Shifts |
| --- | --- | --- | --- |
| Proposed 1 | 121 | 376 | 173 |
| Proposed 2 | 166 | 230 | 30 |
| VTM | 127 | 155 | 0 |

Table 4 Number of operations for Proposed1, Proposed2 and VTM for DCT8/DST7 of length 32

| Method | #Mutliplications | #Additions | #Shifts |
| --- | --- | --- | --- |
| Proposed 1 | 544 | 1388 | 602 |
| Proposed 2 | 590 | 986 | 209 |
| VTM | 620 | 718 | 0 |

# Experimental Results

The simulations were run with VTM5.0 using the common testing conditions. The anchor is VTM5.0 and two test conditions are considered:

- Test1: New DCT8/DST7 for transform sizes of 8x8, 16x16 and 32x32

- Test2: New DCT8/DST7 for transform sizes of 16x16 and 32x32

- Test3: New DCT8/DST7 for transform sizes of 8x8 only

Table 5 Experimental results of Test 1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | U |  |  | V |  |  |  | EncT |  | DecT |
| Class A1 | 0.02% |  |  | 0.05% |  |  | 0.06% |  |  |  | 100% |  | 101% |
| Class A2 | 0.03% |  |  | 0.06% |  |  | 0.02% |  |  |  | 101% |  | 101% |
| Class B | 0.03% |  |  | -0.01% |  |  | 0.06% |  |  |  | 100% |  | 101% |
| Class C | 0.01% |  |  | -0.04% |  |  | 0.05% |  |  |  | 100% |  | 101% |
| Class E | 0.01% |  |  | 0.02% |  |  | -0.08% |  |  |  | 100% |  | 100% |
| Overall | 0.02% |  |  | 0.01% |  |  | 0.03% |  |  |  | 100% |  | 101% |
| Class D | 0.00% |  |  | 0.26% |  |  | 0.09% |  |  |  | 99% |  | 99% |
| Class F | 0.01% |  |  | 0.10% |  |  | 0.12% |  |  |  | 101% |  | 101% |
| Class TGM | 0.02% |  |  | 0.05% |  |  | 0.06% |  |  |  | 100% |  | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  |  | EncT |  | DecT |  |
| Class A1 | 0.00% | -0.13% |  |  | 0.16% |  |  | 100% |  | 101% |  |
| Class A2 | 0.04% | 0.05% |  |  | 0.07% |  |  | 101% |  | 100% |  |
| Class B | 0.02% | 0.02% |  |  | -0.20% |  |  | 100% |  | 100% |  |
| Class C | 0.00% | 0.03% |  |  | 0.04% |  |  | 100% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.00% |  |  | -0.01% |  |  | 100% |  | 100% |  |
| Class D | 0.02% | 0.20% |  |  | 0.18% |  |  | 100% |  | 100% |  |
| Class F | 0.01% | 0.02% |  |  | 0.16% |  |  | 100% |  | 100% |  |
| Class TGM | #VALUE! | #VALUE! |  |  | #VALUE! |  |  | #DIV/0! |  | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.26% |  |  | -0.02% |  |  | 101% |  | 101% |  |
| Class C | 0.03% |  | 0.10% |  |  | 0.37% |  |  | 101% |  | 100% |  |
| Class E | 0.04% |  | 0.08% |  |  | 2.15% |  |  | 100% |  | 100% |  |
| Overall | 0.02% |  | 0.16% |  |  | 0.65% |  |  | 101% |  | 101% |  |
| Class D | 0.03% |  | -0.37% |  |  | 1.13% |  |  | 100% |  | 101% |  |
| Class F | -0.04% |  | 0.04% |  |  | 0.18% |  |  | 101% |  | 100% |  |
| Class TGM | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  | #NUM! |  | #NUM! |  |

Table 6 Experimental results of Test 2

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  | U |  |  | V |  |  |  |  | EncT |  | DecT |
| Class A1 | 0.00% |  |  |  |  |  |  |  | 0.02% |  |  | 0.05% |  |  |  |  | 101% |  | 100% |
| Class A2 | 0.02% |  |  |  |  |  |  |  | 0.04% |  |  | 0.01% |  |  |  |  | 101% |  | 101% |
| Class B | 0.02% |  |  |  |  |  |  |  | 0.05% |  |  | 0.05% |  |  |  |  | 100% |  | 101% |
| Class C | 0.00% |  |  |  |  |  |  |  | -0.02% |  |  | 0.12% |  |  |  |  | 101% |  | 103% |
| Class E | 0.00% |  |  |  |  |  |  |  | -0.04% |  |  | -0.02% |  |  |  |  | 101% |  | 100% |
| Overall | 0.01% |  |  |  |  |  |  |  | 0.01% |  |  | 0.05% |  |  |  |  | 101% |  | 101% |
| Class D | 0.00% |  |  |  |  |  |  |  | 0.16% |  |  | -0.06% |  |  |  |  | 100% |  | 98% |
| Class F | 0.01% |  |  |  |  |  |  |  | 0.01% |  |  | 0.15% |  |  |  |  | 101% |  | 100% |
| Class TGM | #VALUE! |  |  |  |  |  |  |  | #VALUE! |  |  | #VALUE! |  |  |  |  | #DIV/0! |  | #DIV/0! |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | U |  |  | V |  |  | EncT |  |  | DecT |  |  |  |
| Class A1 |  |  |  |  | -0.01% |  | -0.18% |  |  | 0.13% |  |  | 101% |  |  | 101% |  |  |  |
| Class A2 |  |  |  |  | 0.04% |  | 0.22% |  |  | 0.12% |  |  | 101% |  |  | 101% |  |  |  |
| Class B |  |  |  |  | 0.04% |  | 0.07% |  |  | -0.13% |  |  | 100% |  |  | 100% |  |  |  |
| Class C |  |  |  |  | -0.04% |  | -0.02% |  |  | -0.05% |  |  | 100% |  |  | 100% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  | 0.01% |  | 0.03% |  |  | -0.01% |  |  | 100% |  |  | 100% |  |  |  |
| Class D |  |  |  |  | 0.02% |  | -0.15% |  |  | 0.06% |  |  | 100% |  |  | 101% |  |  |  |
| Class F |  |  |  |  | -0.02% |  | -0.12% |  |  | 0.20% |  |  | 101% |  |  | 101% |  |  |  |
| Class TGM |  |  |  |  | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |  |
|  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Y |  |  |  | U |  |  | V |  |  | EncT |  | DecT |  |  |  |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  | 0.00% |  |  |  | 0.15% |  |  | -0.28% |  |  | 101% |  | 101% |  |  |  |  |
| Class C |  |  |  | 0.01% |  |  |  | 0.10% |  |  | -0.01% |  |  | 100% |  | 100% |  |  |  |  |
| Class E |  |  |  | 0.08% |  |  |  | 0.76% |  |  | 0.35% |  |  | 101% |  | 101% |  |  |  |  |
| Overall |  |  |  | 0.03% |  |  |  | 0.28% |  |  | -0.03% |  |  | 101% |  | 101% |  |  |  |  |
| Class D |  |  |  | 0.01% |  |  |  | -0.01% |  |  | 0.47% |  |  | 100% |  | 100% |  |  |  |  |
| Class F |  |  |  | 0.01% |  |  |  | -0.55% |  |  | -0.20% |  |  | 101% |  | 100% |  |  |  |  |
| Class TGM |  |  |  | #VALUE! |  |  |  | #VALUE! |  |  | #VALUE! |  |  | #DIV/0! |  | #DIV/0! |  |  |  |  |

Table 7 Experimental results of Test 3

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | EncT | DecT |
| Class A1 | 0.01% |  | 0.03% |  | 0.06% |  |  | 100% | 101% |
| Class A2 | 0.01% |  | -0.02% |  | -0.05% |  |  | 101% | 101% |
| Class B | 0.01% |  | -0.04% |  | 0.05% |  |  | 101% | 101% |
| Class C | 0.01% |  | -0.06% |  | 0.08% |  |  | 101% | 101% |
| Class E | 0.01% |  | -0.02% |  | -0.04% |  |  | 101% | 101% |
| Overall | 0.01% |  | -0.03% |  | 0.03% |  |  | 101% | 101% |
| Class D | -0.01% |  | 0.05% |  | -0.11% |  |  | 101% | 100% |
| Class F | 0.00% |  | -0.04% |  | 0.09% |  |  | 100% | 101% |
| Class TGM | 0.01% |  | 0.03% |  | 0.06% |  |  | 100% | 101% |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |
|  |  | Over VTM-5.0 |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  | EncT | DecT |  |
| Class A1 |  | 0.01% | -0.21% |  | 0.02% |  | 101% | 100% |  |
| Class A2 |  | 0.03% | 0.05% |  | 0.09% |  | 101% | 101% |  |
| Class B |  | 0.00% | 0.09% |  | -0.10% |  | 101% | 100% |  |
| Class C |  | -0.02% | -0.18% |  | -0.28% |  | 101% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.00% | -0.05% |  | -0.09% |  | 101% | 100% |  |
| Class D |  | -0.04% | 0.13% |  | -0.14% |  | 101% | 100% |  |
| Class F |  | -0.01% | -0.06% |  | 0.17% |  | 100% | 100% |  |
| Class TGM |  | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! | #DIV/0! |  |

The experimental results show that the reduction in number of multiplications is associated with negligible loss (0.03% for test1 and 0.03% for test 2).

# Additional Results

# The low QP results are provided in the tables below:

Table 8 Experimental results of Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.07% | 0.09% | 100% | 101% |
| Class A2 | 0.24% | 0.17% | 0.16% | 100% | 100% |
| Class B | 0.14% | 0.12% | 0.14% | 101% | 100% |
| Class C | 0.16% | 0.15% | 0.14% | 101% | 101% |
| Class E | 0.23% | 0.21% | 0.22% | 101% | 100% |
| Overall | 0.16% | 0.14% | 0.15% | 101% | 100% |
| Class D | 0.19% | 0.13% | 0.09% | 100% | 100% |
| Class F | 0.15% | 0.09% | 0.07% | 101% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 9 Experimental results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.07% | 100% | 101% |
| Class A2 | 0.15% | 0.14% | 0.13% | 100% | 101% |
| Class B | 0.06% | 0.08% | 0.10% | 101% | 100% |
| Class C | 0.04% | 0.05% | 0.06% | 101% | 101% |
| Class E | 0.08% | 0.12% | 0.08% | 100% | 101% |
| Overall | 0.07% | 0.08% | 0.09% | 101% | 101% |
| Class D | 0.04% | 0.02% | 0.07% | 100% | 101% |
| Class F | 0.04% | 0.03% | 0.07% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 10 Experimental results of Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.02% | 101% | 101% |
| Class A2 | 0.11% | 0.03% | 0.04% | 100% | 101% |
| Class B | 0.08% | 0.04% | 0.05% | 101% | 101% |
| Class C | 0.12% | 0.09% | 0.13% | 100% | 100% |
| Class E | 0.15% | 0.11% | 0.08% | 101% | 101% |
| Overall | 0.10% | 0.05% | 0.06% | 100% | 101% |
| Class D | 0.14% | 0.06% | 0.06% | 100% | 101% |
| Class F | 0.11% | 0.04% | 0.06% | 101% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Spec change

# Test 1

–	Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =                        (8-979)

{

{   16   32   46   60   71   78   85   86 }

{   46   78   86   71   32  -16  -60  -85 }

{   71   85   32  -46  -86  -60   16   78 }

{   85   46  -60  -78   16   86   32  -71 }

{   86  -16  -85   32   78  -46  -71   60 }

{   78  -71  -16   85  -60  -32   86  -46 }

{   60  -86   71  -16  -46   85  -78   32 }

{   32  -60   78  -86   85  -71   46  -16 }

}

–	Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =                        (8-980)

{

{    9   17   25   33   41   50   56   62   66   72   77   81   83   87   89   90 }

{   25   50   66   81   89   89   81   66   50   25    0  -25  -50  -66  -81  -89 }

{   41   72   89   83   62   25  -17  -56  -81  -90  -77  -50   -9   33   66   87 }

{   56   87   81   41  -17  -66  -90  -72  -25   33   77   89   62    9  -50  -83 }

{   66   89   50  -25  -81  -81  -25   50   89   66    0  -66  -89  -50   25   81 }

{   77   77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77 }

{   83   56  -50  -87   -9   81   62  -41  -89  -17   77   66  -33  -90  -25   72 }

{   89   25  -81  -50   66   66  -50  -81   25   89    0  -89  -25   81   50  -66 }

{   90   -9  -89   17   87  -25  -83   33   81  -41  -77   50   72  -56  -66   62 }

{   87  -41  -66   72   33  -89    9   83  -50  -62   77   25  -90   17   81  -56 }

{   81  -66  -25   89  -50  -50   89  -25  -66   81    0  -81   66   25  -89   50 }

{   72  -83   25   56  -90   50   33  -87   66    9  -77   81  -17  -62   89  -41 }

{   62  -90   66   -9  -56   89  -72   17   50  -87   77  -25  -41   83  -81   33 }

{   50  -81   89  -66   25   25  -66   89  -81   50    0  -50   81  -89   66  -25 }

{   33  -62   81  -90   83  -66   41   -9  -25   56  -77   89  -87   72  -50   17 }

{   17  -33   50  -62   72  -81   87  -90   89  -83   77  -66   56  -41   25   -9 }

}

–	Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-981)

transMatrix[ m ][ n ] =                        (8-982)

{

{    4    9   13   17   21   26   30   34   38   42   45   50   53   56   60   63 }

{   13   26   38   50   60   68   77   82   86   89   90   88   86   80   74   66 }

{   21   42   60   74   84   89   89   84   74   60   42   21    0  -21  -42  -60 }

{   30   56   77   88   89   80   63   38    9  -21  -50  -72  -86  -90  -84  -68 }

{   38   68   86   88   74   45    9  -30  -63  -84  -90  -78  -53  -17   21   56 }

{   45   78   90   77   42   -4  -50  -80  -90  -74  -38    9   53   82   89   72 }

{   53   86   86   53    0  -53  -86  -86  -53    0   53   86   86   53    0  -53 }

{   60   89   74   21  -42  -84  -84  -42   21   74   89   60    0  -60  -89  -74 }

{   66   90   56  -13  -74  -88  -45   26   80   84   34  -38  -86  -78  -21   50 }

{   72   86   34  -45  -89  -63   13   78   82   21  -56  -90  -53   26   84   77 }

{   77   80    9  -72  -84  -17   66   86   26  -60  -88  -34   53   90   42  -45 }

{   80   72  -17  -86  -60   34   90   45  -50  -89  -30   63   86   13  -74  -78 }

{   84   60  -42  -89  -21   74   74  -21  -89  -42   60   84    0  -84  -60   42 }

{   86   45  -63  -78   21   90   26  -77  -66   42   88    4  -86  -50   60   80 }

{   88   30  -78  -56   60   77  -34  -88    4   89   26  -80  -53   63   74  -38 }

{   90   13  -88  -26   84   38  -78  -50   72   60  -63  -68   53   77  -42  -82 }

}

transMatrix[ m ][ n ] = transMatrixCol16to31[ m ][ n ] with m = 16..31, n = 0..15	(8-983)

transMatrix[ m ][ n ] =                        (8-984)

{

{   66   68   72   74   77   78   80   82   84   86   86   88   88   89   90   90 }

{   56   45   34   21    9   -4  -17  -30  -42  -53  -63  -72  -78  -84  -88  -90 }

{  -74  -84  -89  -89  -84  -74  -60  -42  -21    0   21   42   60   74   84   89 }

{  -45  -17   13   42   66   82   90   86   74   53   26   -4  -34  -60  -78  -88 }

{   80   90   82   60   26  -13  -50  -77  -89  -86  -66  -34    4   42   72   88 }

{   34  -13  -56  -84  -88  -68  -30   17   60   86   88   66   26  -21  -63  -86 }

{  -86  -86  -53    0   53   86   86   53    0  -53  -86  -86  -53    0   53   86 }

{  -21   42   84   84   42  -21  -74  -89  -60    0   60   89   74   21  -42  -84 }

{   88   72    9  -60  -90  -63    4   68   89   53  -17  -77  -86  -42   30   82 }

{    9  -66  -88  -42   38   88   68   -4  -74  -86  -30   50   90   60  -17  -80 }

{  -90  -50   38   89   56  -30  -88  -63   21   86   68  -13  -82  -74    4   78 }

{    4   82   68  -21  -88  -56   38   90   42  -53  -88  -26   66   84    9  -77 }

{   89   21  -74  -74   21   89   42  -60  -84    0   84   60  -42  -89  -21   74 }

{  -17  -90  -30   74   68  -38  -88   -9   84   53  -56  -82   13   89   34  -72 }

{  -86    9   90   21  -82  -50   66   72  -42  -86   13   90   17  -84  -45   68 }

{   30   86  -17  -89    4   90    9  -88  -21   86   34  -80  -45   74   56  -66 }

}

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-985)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

–	Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =                        (8-979)

{

{   86   85   78   71   60   46   32   16 }

{   85   60   16  -32  -71  -86  -78  -46 }

{   78   16  -60  -86  -46   32   85   71 }

{   71  -32  -86  -16   78   60  -46  -85 }

{   60  -71  -46   78   32  -85  -16   86 }

{   46  -86   32   60  -85   16   71  -78 }

{   32  -78   85  -46  -16   71  -86   60 }

{   16  -46   71  -85   86  -78   60  -32 }

}

–	Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =                        (8-980)

{

{   90   89   87   83   81   77   72   66   62   56   50   41   33   25   17    9 }

{   89   81   66   50   25    0  -25  -50  -66  -81  -89  -89  -81  -66  -50  -25 }

{   87   66   33   -9  -50  -77  -90  -81  -56  -17   25   62   83   89   72   41 }

{   83   50   -9  -62  -89  -77  -33   25   72   90   66   17  -41  -81  -87  -56 }

{   81   25  -50  -89  -66    0   66   89   50  -25  -81  -81  -25   50   89   66 }

{   77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77  -77 }

{   72  -25  -90  -33   66   77  -17  -89  -41   62   81   -9  -87  -50   56   83 }

{   66  -50  -81   25   89    0  -89  -25   81   50  -66  -66   50   81  -25  -89 }

{   62  -66  -56   72   50  -77  -41   81   33  -83  -25   87   17  -89   -9   90 }

{   56  -81  -17   90  -25  -77   62   50  -83   -9   89  -33  -72   66   41  -87 }

{   50  -89   25   66  -81    0   81  -66  -25   89  -50  -50   89  -25  -66   81 }

{   41  -89   62   17  -81   77   -9  -66   87  -33  -50   90  -56  -25   83  -72 }

{   33  -81   83  -41  -25   77  -87   50   17  -72   89  -56   -9   66  -90   62 }

{   25  -66   89  -81   50    0  -50   81  -89   66  -25  -25   66  -89   81  -50 }

{   17  -50   72  -87   89  -77   56  -25   -9   41  -66   83  -90   81  -62   33 }

{    9  -25   41  -56   66  -77   83  -89   90  -87   81  -72   62  -50   33  -17 }

}

–	Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-981)

transMatrix[ m ][ n ] =                        (8-982)

{

{   90   90   89   88   88   86   86   84   82   80   78   77   74   72   68   66 }

{   90   88   84   78   72   63   53   42   30   17    4   -9  -21  -34  -45  -56 }

{   89   84   74   60   42   21    0  -21  -42  -60  -74  -84  -89  -89  -84  -74 }

{   88   78   60   34    4  -26  -53  -74  -86  -90  -82  -66  -42  -13   17   45 }

{   88   72   42    4  -34  -66  -86  -89  -77  -50  -13   26   60   82   90   80 }

{   86   63   21  -26  -66  -88  -86  -60  -17   30   68   88   84   56   13  -34 }

{   86   53    0  -53  -86  -86  -53    0   53   86   86   53    0  -53  -86  -86 }

{   84   42  -21  -74  -89  -60    0   60   89   74   21  -42  -84  -84  -42   21 }

{   82   30  -42  -86  -77  -17   53   89   68    4  -63  -90  -60    9   72   88 }

{   80   17  -60  -90  -50   30   86   74    4  -68  -88  -38   42   88   66   -9 }

{   78    4  -74  -82  -13   68   86   21  -63  -88  -30   56   89   38  -50  -90 }

{   77   -9  -84  -66   26   88   53  -42  -90  -38   56   88   21  -68  -82   -4 }

{   74  -21  -89  -42   60   84    0  -84  -60   42   89   21  -74  -74   21   89 }

{   72  -34  -89  -13   82   56  -53  -84    9   88   38  -68  -74   30   90   17 }

{   68  -45  -84   17   90   13  -86  -42   72   66  -50  -82   21   90    9  -86 }

{   66  -56  -74   45   80  -34  -86   21   88   -9  -90   -4   89   17  -86  -30 }

}

transMatrix[ m ][ n ] = transMatrixCol16to31[ m ][ n ] with m = 16..31, n = 0..15	(8-983)

transMatrix[ m ][ n ] =                        (8-984)

{

{   63   60   56   53   50   45   42   38   34   30   26   21   17   13    9    4 }

{  -66  -74  -80  -86  -88  -90  -89  -86  -82  -77  -68  -60  -50  -38  -26  -13 }

{  -60  -42  -21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{   68   84   90   86   72   50   21   -9  -38  -63  -80  -89  -88  -77  -56  -30 }

{   56   21  -17  -53  -78  -90  -84  -63  -30    9   45   74   88   86   68   38 }

{  -72  -89  -82  -53   -9   38   74   90   80   50    4  -42  -77  -90  -78  -45 }

{  -53    0   53   86   86   53    0  -53  -86  -86  -53    0   53   86   86   53 }

{   74   89   60    0  -60  -89  -74  -21   42   84   84   42  -21  -74  -89  -60 }

{   50  -21  -78  -86  -38   34   84   80   26  -45  -88  -74  -13   56   90   66 }

{  -77  -84  -26   53   90   56  -21  -82  -78  -13   63   89   45  -34  -86  -72 }

{  -45   42   90   53  -34  -88  -60   26   86   66  -17  -84  -72    9   80   77 }

{   78   74  -13  -86  -63   30   89   50  -45  -90  -34   60   86   17  -72  -80 }

{   42  -60  -84    0   84   60  -42  -89  -21   74   74  -21  -89  -42   60   84 }

{  -80  -60   50   86   -4  -88  -42   66   77  -26  -90  -21   78   63  -45  -86 }

{  -38   74   63  -53  -80   26   89    4  -88  -34   77   60  -56  -78   30   88 }

{   82   42  -77  -53   68   63  -60  -72   50   78  -38  -84   26   88  -13  -90 }

}

## Test 2

–	Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =                        (8-980)

{

{    9   17   25   33   41   50   56   62   66   72   77   81   83   87   89   90 }

{   25   50   66   81   89   89   81   66   50   25    0  -25  -50  -66  -81  -89 }

{   41   72   89   83   62   25  -17  -56  -81  -90  -77  -50   -9   33   66   87 }

{   56   87   81   41  -17  -66  -90  -72  -25   33   77   89   62    9  -50  -83 }

{   66   89   50  -25  -81  -81  -25   50   89   66    0  -66  -89  -50   25   81 }

{   77   77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77 }

{   83   56  -50  -87   -9   81   62  -41  -89  -17   77   66  -33  -90  -25   72 }

{   89   25  -81  -50   66   66  -50  -81   25   89    0  -89  -25   81   50  -66 }

{   90   -9  -89   17   87  -25  -83   33   81  -41  -77   50   72  -56  -66   62 }

{   87  -41  -66   72   33  -89    9   83  -50  -62   77   25  -90   17   81  -56 }

{   81  -66  -25   89  -50  -50   89  -25  -66   81    0  -81   66   25  -89   50 }

{   72  -83   25   56  -90   50   33  -87   66    9  -77   81  -17  -62   89  -41 }

{   62  -90   66   -9  -56   89  -72   17   50  -87   77  -25  -41   83  -81   33 }

{   50  -81   89  -66   25   25  -66   89  -81   50    0  -50   81  -89   66  -25 }

{   33  -62   81  -90   83  -66   41   -9  -25   56  -77   89  -87   72  -50   17 }

{   17  -33   50  -62   72  -81   87  -90   89  -83   77  -66   56  -41   25   -9 }

}

–	Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-981)

transMatrix[ m ][ n ] =                        (8-982)

{

{    4    9   13   17   21   26   30   34   38   42   45   50   53   56   60   63 }

{   13   26   38   50   60   68   77   82   86   89   90   88   86   80   74   66 }

{   21   42   60   74   84   89   89   84   74   60   42   21    0  -21  -42  -60 }

{   30   56   77   88   89   80   63   38    9  -21  -50  -72  -86  -90  -84  -68 }

{   38   68   86   88   74   45    9  -30  -63  -84  -90  -78  -53  -17   21   56 }

{   45   78   90   77   42   -4  -50  -80  -90  -74  -38    9   53   82   89   72 }

{   53   86   86   53    0  -53  -86  -86  -53    0   53   86   86   53    0  -53 }

{   60   89   74   21  -42  -84  -84  -42   21   74   89   60    0  -60  -89  -74 }

{   66   90   56  -13  -74  -88  -45   26   80   84   34  -38  -86  -78  -21   50 }

{   72   86   34  -45  -89  -63   13   78   82   21  -56  -90  -53   26   84   77 }

{   77   80    9  -72  -84  -17   66   86   26  -60  -88  -34   53   90   42  -45 }

{   80   72  -17  -86  -60   34   90   45  -50  -89  -30   63   86   13  -74  -78 }

{   84   60  -42  -89  -21   74   74  -21  -89  -42   60   84    0  -84  -60   42 }

{   86   45  -63  -78   21   90   26  -77  -66   42   88    4  -86  -50   60   80 }

{   88   30  -78  -56   60   77  -34  -88    4   89   26  -80  -53   63   74  -38 }

{   90   13  -88  -26   84   38  -78  -50   72   60  -63  -68   53   77  -42  -82 }

}

transMatrix[ m ][ n ] = transMatrixCol16to31[ m ][ n ] with m = 16..31, n = 0..15	(8-983)

transMatrix[ m ][ n ] =                        (8-984)

{

{   66   68   72   74   77   78   80   82   84   86   86   88   88   89   90   90 }

{   56   45   34   21    9   -4  -17  -30  -42  -53  -63  -72  -78  -84  -88  -90 }

{  -74  -84  -89  -89  -84  -74  -60  -42  -21    0   21   42   60   74   84   89 }

{  -45  -17   13   42   66   82   90   86   74   53   26   -4  -34  -60  -78  -88 }

{   80   90   82   60   26  -13  -50  -77  -89  -86  -66  -34    4   42   72   88 }

{   34  -13  -56  -84  -88  -68  -30   17   60   86   88   66   26  -21  -63  -86 }

{  -86  -86  -53    0   53   86   86   53    0  -53  -86  -86  -53    0   53   86 }

{  -21   42   84   84   42  -21  -74  -89  -60    0   60   89   74   21  -42  -84 }

{   88   72    9  -60  -90  -63    4   68   89   53  -17  -77  -86  -42   30   82 }

{    9  -66  -88  -42   38   88   68   -4  -74  -86  -30   50   90   60  -17  -80 }

{  -90  -50   38   89   56  -30  -88  -63   21   86   68  -13  -82  -74    4   78 }

{    4   82   68  -21  -88  -56   38   90   42  -53  -88  -26   66   84    9  -77 }

{   89   21  -74  -74   21   89   42  -60  -84    0   84   60  -42  -89  -21   74 }

{  -17  -90  -30   74   68  -38  -88   -9   84   53  -56  -82   13   89   34  -72 }

{  -86    9   90   21  -82  -50   66   72  -42  -86   13   90   17  -84  -45   68 }

{   30   86  -17  -89    4   90    9  -88  -21   86   34  -80  -45   74   56  -66 }

}

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-985)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-986)

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

–	Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =                        (8-980)

{

{   90   89   87   83   81   77   72   66   62   56   50   41   33   25   17    9 }

{   89   81   66   50   25    0  -25  -50  -66  -81  -89  -89  -81  -66  -50  -25 }

{   87   66   33   -9  -50  -77  -90  -81  -56  -17   25   62   83   89   72   41 }

{   83   50   -9  -62  -89  -77  -33   25   72   90   66   17  -41  -81  -87  -56 }

{   81   25  -50  -89  -66    0   66   89   50  -25  -81  -81  -25   50   89   66 }

{   77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77  -77 }

{   72  -25  -90  -33   66   77  -17  -89  -41   62   81   -9  -87  -50   56   83 }

{   66  -50  -81   25   89    0  -89  -25   81   50  -66  -66   50   81  -25  -89 }

{   62  -66  -56   72   50  -77  -41   81   33  -83  -25   87   17  -89   -9   90 }

{   56  -81  -17   90  -25  -77   62   50  -83   -9   89  -33  -72   66   41  -87 }

{   50  -89   25   66  -81    0   81  -66  -25   89  -50  -50   89  -25  -66   81 }

{   41  -89   62   17  -81   77   -9  -66   87  -33  -50   90  -56  -25   83  -72 }

{   33  -81   83  -41  -25   77  -87   50   17  -72   89  -56   -9   66  -90   62 }

{   25  -66   89  -81   50    0  -50   81  -89   66  -25  -25   66  -89   81  -50 }

{   17  -50   72  -87   89  -77   56  -25   -9   41  -66   83  -90   81  -62   33 }

{    9  -25   41  -56   66  -77   83  -89   90  -87   81  -72   62  -50   33  -17 }

}

–	Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-981)

transMatrix[ m ][ n ] =                        (8-982)

{

{   90   90   89   88   88   86   86   84   82   80   78   77   74   72   68   66 }

{   90   88   84   78   72   63   53   42   30   17    4   -9  -21  -34  -45  -56 }

{   89   84   74   60   42   21    0  -21  -42  -60  -74  -84  -89  -89  -84  -74 }

{   88   78   60   34    4  -26  -53  -74  -86  -90  -82  -66  -42  -13   17   45 }

{   88   72   42    4  -34  -66  -86  -89  -77  -50  -13   26   60   82   90   80 }

{   86   63   21  -26  -66  -88  -86  -60  -17   30   68   88   84   56   13  -34 }

{   86   53    0  -53  -86  -86  -53    0   53   86   86   53    0  -53  -86  -86 }

{   84   42  -21  -74  -89  -60    0   60   89   74   21  -42  -84  -84  -42   21 }

{   82   30  -42  -86  -77  -17   53   89   68    4  -63  -90  -60    9   72   88 }

{   80   17  -60  -90  -50   30   86   74    4  -68  -88  -38   42   88   66   -9 }

{   78    4  -74  -82  -13   68   86   21  -63  -88  -30   56   89   38  -50  -90 }

{   77   -9  -84  -66   26   88   53  -42  -90  -38   56   88   21  -68  -82   -4 }

{   74  -21  -89  -42   60   84    0  -84  -60   42   89   21  -74  -74   21   89 }

{   72  -34  -89  -13   82   56  -53  -84    9   88   38  -68  -74   30   90   17 }

{   68  -45  -84   17   90   13  -86  -42   72   66  -50  -82   21   90    9  -86 }

{   66  -56  -74   45   80  -34  -86   21   88   -9  -90   -4   89   17  -86  -30 }

}

transMatrix[ m ][ n ] = transMatrixCol16to31[ m ][ n ] with m = 16..31, n = 0..15	(8-983)

transMatrix[ m ][ n ] =                        (8-984)

{

{   63   60   56   53   50   45   42   38   34   30   26   21   17   13    9    4 }

{  -66  -74  -80  -86  -88  -90  -89  -86  -82  -77  -68  -60  -50  -38  -26  -13 }

{  -60  -42  -21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{   68   84   90   86   72   50   21   -9  -38  -63  -80  -89  -88  -77  -56  -30 }

{   56   21  -17  -53  -78  -90  -84  -63  -30    9   45   74   88   86   68   38 }

{  -72  -89  -82  -53   -9   38   74   90   80   50    4  -42  -77  -90  -78  -45 }

{  -53    0   53   86   86   53    0  -53  -86  -86  -53    0   53   86   86   53 }

{   74   89   60    0  -60  -89  -74  -21   42   84   84   42  -21  -74  -89  -60 }

{   50  -21  -78  -86  -38   34   84   80   26  -45  -88  -74  -13   56   90   66 }

{  -77  -84  -26   53   90   56  -21  -82  -78  -13   63   89   45  -34  -86  -72 }

{  -45   42   90   53  -34  -88  -60   26   86   66  -17  -84  -72    9   80   77 }

{   78   74  -13  -86  -63   30   89   50  -45  -90  -34   60   86   17  -72  -80 }

{   42  -60  -84    0   84   60  -42  -89  -21   74   74  -21  -89  -42   60   84 }

{  -80  -60   50   86   -4  -88  -42   66   77  -26  -90  -21   78   63  -45  -86 }

{  -38   74   63  -53  -80   26   89    4  -88  -34   77   60  -56  -78   30   88 }

{   82   42  -77  -53   68   63  -60  -72   50   78  -38  -84   26   88  -13  -90 }

}

## Test 3

–	Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =                        (8-979)

{

{   16   32   46   60   71   78   85   86 }

{   46   78   86   71   32  -16  -60  -85 }

{   71   85   32  -46  -86  -60   16   78 }

{   85   46  -60  -78   16   86   32  -71 }

{   86  -16  -85   32   78  -46  -71   60 }

{   78  -71  -16   85  -60  -32   86  -46 }

{   60  -86   71  -16  -46   85  -78   32 }

{   32  -60   78  -86   85  -71   46  -16 }

}

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-980)

{

{   8   17   25   33   40   48   55   62   68   73   77   81   85   87   88   88 }

{  25   48   68   81   88   88   81   68   48   25    0  −25  −48  −68  −81  −88 }

{  40   73   88   85   62   25  −17  −55  −81  −88  −77  −48   −8   33   68   87 }

{  55   87   81   40  −17  −68  −88  −73  −25   33   77   88   62    8  −48  −85 }

{  68   88   48  −25  −81  −81  −25   48   88   68    0  −68  −88  −48   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  85   55  −48  −87   −8   81   62  −40  −88  −17   77   68  −33  −88  −25   73 }

{  88   25  −81  −48   68   68  −48  −81   25   88    0  −88  −25   81   48  −68 }

{  88   −8  −88   17   87  −25  −85   33   81  −40  −77   48   73  −55  −68   62 }

{  87  −40  −68   73   33  −88    8   85  −48  −62   77   25  −88   17   81  −55 }

{  81  −68  −25   88  −48  −48   88  −25  −68   81    0  −81   68   25  −88   48 }

{  73  −85   25   55  −88   48   33  −87   68    8  −77   81  −17  −62   88  −40 }

{  62  −88   68   −8  −55   88  −73   17   48  −87   77  −25  −40   85  −81   33 }

{  48  −81   88  −68   25   25  −68   88  −81   48    0  −48   81  −88   68  −25 }

{  33  −62   81  −88   85  −68   40   −8  −25   55  −77   88  −87   73  −48   17 }

{  17  −33   48  −62   73  −81   87  −88   88  −85   77  −68   55  −40   25   −8 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-981)

transMatrixCol0to15 =	(8-982)

{

{   4    9   13   17   21   26   30   34   38   42   46   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   87   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   46    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  46   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −87  −46   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −46  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −46 }

{  80   72  −17  −86  −60   34   90   46  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   46  −63  −78   21   90   26  −77  −66   42   87    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −87    4   89   26  −80  −53   63   74  −38 }

{  90   13  −87  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15	(8-983)

transMatrixCol16to31 =	(8-984)

{

{  66   68   72   74   77   78   80   82   84   85   86   87   88   89   90   90 }

{  56   46   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −87  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −46  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   87 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   87   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   87   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −87  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −87  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −46   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −46   74   56  −66 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-985)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-985)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

–	Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =                        (8-979)

{

{   86   85   78   71   60   46   32   16 }

{   85   60   16  -32  -71  -86  -78  -46 }

{   78   16  -60  -86  -46   32   85   71 }

{   71  -32  -86  -16   78   60  -46  -85 }

{   60  -71  -46   78   32  -85  -16   86 }

{   46  -86   32   60  -85   16   71  -78 }

{   32  -78   85  -46  -16   71  -86   60 }

{   16  -46   71  -85   86  -78   60  -32 }

}

# Conclusion

This contribution proposes new fast algorithms of DCT8/DST7 with dual support. The new design reduces the number of multiplications. When testing length-8 DCT8 fast algorithm, Experimental results show 0.01% loss in AI and 0.00% loss in RA, while 0.05% chroma gain is achieved. This comes with 61% reduction of number of multiplications.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).