| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0700 |
| --- | --- |

| Title: | AHG12: On sub-picture info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes SEI message to signal position information of independently coded picture region.

# Introduction

JVET-N0826 proposed a harmonized sub-picture-based coding design for VVC, a picture may be divided into sub-pictures, and each sub-picture may refer to its own PPS. The sub-picture indication and position information such as location and size are signaled in SPS.

JVET-O0394 proposed to exclude the positioning information of independently coded picture regions from the syntax of parameter sets and place it in the decoding control NAL unit proposed in JVET-O0391.

As each interpedently coded picture region, or sub-picture, may refer to its own PPS, signaling the position information of individual sub-picture may not present the layout information of the entire composite picture. While signaling position information of all sub-pictures referring to the PPS may be redundant for each individual sub-picture.

Signaling the position information in SPS may prohibit sub-picture layout change during CVS which is not desirable for the applications such as viewport dependent streaming.

To facilitate dynamic sub-picture position change on frame basis, this contribution proposes a SEI message to carry the size of composite picture and the position information of the sub-picture associated with the picture.

# Sub-picture info SEI message

Table 1 sub-picture info SEI message

| sub_picture_info( payloadSize ) { | Descriptor |
| --- | --- |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| num_sub_pics_minus1 | ue(v) |
| sub_pic_id_len_minus1 | ue(v) |
| for ( i = 0; i <= num_sub_pics_minus1; i++ ) { |  |
| sub_pic_id[ i ] | u(v) |
| if( num_sub_pics_minus1 ) { |  |
| sub_pic_x_offset[ i ] | ue(v) |
| sub_pic_y_offset[ i ] | ue(v) |
| sub_pic_width_in_luma_samples[ i ] | ue(v) |
| sub_pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

pic_width_in_luma_samples specifies the deafult width of picture associated with SEI message in units of luma samples. A picture may consist of a single or multiple sub-pictures.

layer_pic_height_in_luma_samples specifies the default height of pictures associated with SEI message in units of luma samples. A picture may consist of a single or multiple sub-pictures.

num_sub_pics_minus1 plus 1 specifies the number of sub-pictures of pictures in the picture. The value of num_sub_pics_minus1  shall be in the range of 0 to 1024, inclusive.

sub_pic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sub_pic_id[ i ]. The value of sub_pic_id_len_minus1 shall be in the range of Ceil( Log2( num_sub_pic_minus1 + 1 ) − 1 to 9, inclusive.

sub_pic_id[ i ] specifies the sub-picture ID of the j-th sub-picture in the picture associated with the SEI message. The length of vps_sub_pic_id[ i ]  is sub_pic_id_len_minus1 + 1 bits.

sub_pic_x_offset[ i ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the i-th sub-picture relative to the top-left corner luma sample of the picture associated with the SEI message. When not present, the value of sub_pic_x_offset[ i ] is inferred to be equal to 0.

sub_pic_y_offset[ i ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the i-th sub-picture relative to the top-left corner luma sample of the picture associated with the SEI message. When not present, the value of sub_pic_y_offset[ i ] is inferred to be equal to 0.

sub_pic_width_in_luma_samples[ i ] specifies the width, in units of luma samples, of the i-th sub-picture of the picture associated with the SEI message. When not present, the value of sub_pic_width_in_luma_samples[ i ] is inferred to be equal to pic_width_in_luma_samples.

sub_pic_height_in_luma_samples[ i ] specifies the height, in units of luma samples, of the i-th sub-picture for the picture associated with the SEI message. When not present, the value of sub_pic_height_in_luma_samples[ i ][ j ] is inferred to be equal to pic_height_in_luma_samples.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).