# Draft Specification Changes

## Test 1: Chroma 4x4 change on VTM-5.0

### 8.6.1 General decoding process for coding units coded in IBC prediction mode

…

…

…

The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps:

- The motion vector components of the current coding unit are derived as follows:

    - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

- The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output.

- When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

    - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies:

        - If either cbWidth or cbHeight is equal to 4, useChroma2x2 is set to TRUE. Otherwise, useChroma2x2 is set to FALSE.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

numSbX = useChroma2x2 ? ( cbWidth >> 2 ) : ( cbWidth >> 3 )	(8-873)

numSbY = useChroma2x2 ? ( cbHeight >> 2 ) : ( cbHeight >> 3 )	(8-874)

- The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1:

- The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows:

- The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows:

xCuY = useChroma2x2 ? ( xCb + xSbIdx*4 ) : ( xCb + xSbIdx*8 + 4 ) 	(8-875)

yCuY = useChroma2x2 ? ( yCb + ySbIdx*4 ) : ( yCb + ySbIdx*8 + 4 ) 	(8-876)

- If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies.

mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0	(8-877)

mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0	(8-878)

predFlagL0[ xSbIdx ][ ySbIdx ] = 0	(8-879)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-880)

- Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies:

mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] 	(8-881)

mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] 	(8-882)

predFlagL0[ xSbIdx ][ ySbIdx ] = 1	(8-883)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-884)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output.

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, the output shall be equal to TRUE.

- One or both of the following conditions shall be true:

- ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + useChroma2x2 ? ( xSbIdx * 2 + 2 ) : ( xSbIdx * 4 + 4 ) is less than or equal to 0.

- ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + useChroma2x2 ? ( ySbIdx * 2 + 2 ) : ( ySbIdx * 4 + 4 ) is less than or equal to 0.

## Test 2 : Chroma 4x4 change on CE8-1.5.b (i.e. CU center BV as default BV)

### 8.6.1 Decoding process for coding units coded in IBC prediction mode

8.6.1.1 General

…

…

…

The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps:

- The motion vector components of the current coding unit are derived as follows:

    - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

- The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output.

- When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

    - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies:

        - If either cbWidth or cbHeight is equal to 4, useChroma2x2 is set to TRUE. Otherwise, useChroma2x2 is set to FALSE.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

numSbX = useChroma2x2 ? ( cbWidth >> 2 ) : ( cbWidth >> 3 )	(8-873)

numSbY = useChroma2x2 ? ( cbHeight >> 2 ) : ( cbHeight >> 3 )	(8-874)

- The luma motion vector mvLDefault is derived as follows:

- The location ( xCenterY, yCenterY ) of the center position of the collocated luma coding unit is dervied as follows:

xCenterY = xCb + cbWidth / 2 	(8-875)

yCenterY = yCb + cbHeight / 2	(8-876)

- If CuPredMode[ xCenterY ][ yCenterY ] is equal to MODE_INTRA, the following applies.

mvLDefault[ 0 ] = 0		(8-877)

mvLDefault[ 1 ] = 0		(8-878)

predFlagL0Default = 0			(8-879)

predFlagL1Default = 0		(8-880)

- Otherwise ( CuPredMode[ xCenterY ][ yCenterY ] is equal to MODE_IBC ), the following applies:

mvLDefault[ 0 ]=MvL0[ xCenterY ][ yCenterY ][ 0 ] 	(8-881)

mvLDefault[ 1 ]=MvL0[ xCenterY ][ yCenterY ][ 1 ] 	(8-882)

predFlagL0Default = 1			(8-883)

predFlagL1Default = 0		(8-884)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvLDefault as inputs, and mvCDefault as output.

- It is a requirement of bitstream conformance that the chroma motion vector mvCDefault shall be valid. That is the chroma IBC motion vector validation process in clause 8.6.1.2 is invoked with the luma location ( xCb, yCb ) set equal to (xCb, yCb), cbWidth, cbHeight set to cbWidth, cbHeight, chroma sub-block index ( xSbIdx, ySbIdx ) set to ( xSbIdx, ySbIdx ), the flag useChroma2x2 set to useChroma2x2, mvC set to mvCDefault. The output shall be equal to TRUE.

- The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1:

- The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows:

- The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows:

xCuY = useChroma2x2 ? ( xCb + xSbIdx*4 ) : ( xCb + xSbIdx*8 + 4 ) 	(8-885)

yCuY = useChroma2x2 ? ( yCb + ySbIdx*4 ) : ( yCb + ySbIdx*8 + 4 ) 	(8-886)

- If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies.

mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0	(8-887)

mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0	(8-888)

predFlagL0[ xSbIdx ][ ySbIdx ] = 0	(8-889)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-890)

- Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies:

mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] 	(8-891)

mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] 	(8-892)

predFlagL0[ xSbIdx ][ ySbIdx ] = 1	(8-893)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-894)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output.

- The chroma IBC motion vector validation process in clause 8.6.1.2 is invoked with the luma location ( xCb, yCb ) set equal to (xCb, yCb), cbWidth, cbHeight set to cbWidth, cbHeight, chroma sub-block index ( xSbIdx, ySbIdx ) set to ( xSbIdx, ySbIdx ), the flag useChroma2x2 set to useChroma2x2, mvC set to mvC[ xSbIdx ][ ySbIdx ]. If the output is equal to FALSE, mvC[ xSbIdx ][ ySbIdx ] is set to mvCDefault.

- The prediction samples of the current coding unit are derived as follows:

  - If  treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the prediction samples of the current coding unit are derived as follows:

…

…

…

********* [Note: The following section is borrowed and modified from clause 8.6.2.1 on IBC luma motion vector conformance check to fit for the chroma IBC motion vector validation check.] *********

8.6.1.2 IBC motion vector validation process for chroma

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- a chroma sub-block index ( xSbIdx, ySbIdx ) identifying the chroma sub-block in the current chroma coding block

- a flag useChroma2x2 indicating whether used chroma sub-block size is 2x2 or not

- a chroma motion vector mvC

Outputs of this process are:

- whether the motion vector is valid or not.

The top-left location inside the reference block ( xRefTL, yRefTL ) and the bottom-right location inside the reference block ( xRefBR, yRefBR ) are derived as follows:

( xRefTL, yRefTL ) = ( xCb / SubWidthC + ( mvC[ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ 1 ] >> 5 ) ) 	(8-895)

( xRefBR, yRefBR ) = ( xRefTL + cbWidth / SubWidthC − 1, yRefTL + cbHeight / SubHeightC − 1 ) 	(8-896)

It any of the following condidtions is not true, the chroma motion vector is considered not valid and the output of this process is FALSE. Otherwise, the output of this process is TRUE.

- The motion vector mvC points to a reference block that is available, i.e. for ( xRefTL, yRefTL ) and ( xRefBR, yRefBR ), all of the following conditions shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xRefTL, yRefTL ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xRefBR, yRefBR ) as inputs, the output shall be equal to TRUE.

- The motion vector mvC points to a reference block that is outside (left and/or above) and not overlapping with the current coding bock, i.e. one or both the following conditions shall be true:

  - ( mvC[ 0 ] >> 5 ) + useChroma2x2 ? ( xSbIdx * 2 + 2 ) : ( xSbIdx * 4 + 4 ) is less than or equal to 0.

  - ( mvC[ 1 ] >> 5 ) + useChroma2x2 ? ( ySbIdx * 2 + 2 ) : ( ySbIdx * 4 + 4 ) is less than or equal to 0.

- The motion vector mvC points to a reference block that is fully contained in the same coding tree block as the current block or fully contained in a block to the left with the same height of the current coding tree block and a width equal to 128 / SubWidthC chroma samples, i.e. all of the following conditions shall be true:

yRefTL / CtbHeightC = yCb >> CtbLog2SizeY 		(8-897)

yRefBR / CtbHeightC = yCb >> CtbLog2SizeY	 	(8-898)

xRefTL / CtbWidthC >= ( xCb >> CtbLog2SizeY ) + Min( 1, 7 − CtbLog2SizeY )	(8-899)
	− ( 1 << ( (7 − CtbLog2SizeY ) << 1) ) )

xRefBR / CtbWidthC <= ( xCb >> CtbLog2SizeY )	(8-900)

- When the motion vector mvC points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) / CtbWidthC is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvC does not point to a reference block where the top-left sample is located left to or inside the same (64 / SubWidthC )x( 64 / SubHeightC ) area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( ( xRefTL + 128 / SubWidthC ) / ( 64 / SubWidthC ) * ( 64 / SubWidthC ), yRefTL / ( 64 / SubHeightC ) * ( 64 / SubHeightC ) ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 / SubWidthC ) / ( 64 / SubWidthC ) * ( 64 / SubWidthC ), yRefTL / ( 64 / SubHeightC ) * (64 / SubHeightC ) ) shall not be equal to ( xCb / SubWidthC, yCb / SubHeightC ).