| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0713 |
| --- | --- |

| Title: | Crosscheck of JVET-O0531: Non-CE4: On LIC for Affine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu | Tel: Email: | 1-858-209-3851 xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Verification

The document reports the results of crosschecking JVET-O0531 where two modifications are proposed for the LIC. In the first modification, the LIC flag derivation for constructed affine merge by enabling LIC for all constructed affine merge candidates is simplified. The second modification proposes to disable PROF when LIC is enabled for an affine coded block. The software was provided by the proponent and it is verified that the software implementation is matched with what is described in JVET-O0531. After the cross-checking, all the BD-rates results match exactly with those in JVET-O0531. Due to heterogeneous cores that are used, the encoding and decoding time may not be reliable for the cross-check results.

- Test One

|  | Over VTM-5.0, RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.54% | -0.59% | 123% | 99% |
| Class A2 | -0.38% | -0.32% | -0.41% | 113% | 100% |
| Class B | -0.48% | -0.28% | -0.49% | 117% | 102% |
| Class C | -0.37% | -0.57% | -0.61% | 119% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.42% | -0.52% | 118% | 101% |
| Class D | -0.11% | -0.07% | 0.03% | 116% | 107% |
| Class F | -0.69% | -0.41% | -0.44% | 113% | 105% |

|  | Over VTM-5.0, LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.07% | -0.58% | 123% | 98% |
| Class C | -0.47% | -0.38% | -0.70% | 127% | 104% |
| Class E | -0.11% | 0.03% | -0.63% | 103% | 103% |
| Overall | -0.41% | -0.15% | -0.63% | 119% | 101% |
| Class D | -0.06% | -0.85% | 0.14% | 119% | 102% |
| Class F | -1.90% | -1.68% | -1.74% | 119% | 102% |

- Test Two

|  | Over VTM-5.0, RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.67% | -0.53% | 125% | 100% |
| Class A2 | -1.20% | -0.06% | -0.26% | 115% | 101% |
| Class B | -1.02% | -0.50% | -0.58% | 117% | 101% |
| Class C | -0.67% | -0.52% | -0.73% | 119% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.91% | -0.45% | -0.54% | 119% | 101% |
| Class D | -0.68% | -0.32% | -0.26% | 115% | 104% |
| Class F | -1.51% | -0.81% | -0.88% | 112% | 103% |

|  | Over VTM-5.0, LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.87% | -0.39% | -0.46% | 125% | 100% |
| Class C | -0.66% | -0.36% | -0.64% | 126% | 104% |
| Class E | -0.38% | 0.02% | -0.78% | 105% | 101% |
| Overall | -0.68% | -0.28% | -0.60% | 120% | 102% |
| Class D | -0.51% | -0.78% | -0.02% | 119% | 100% |
| Class F | -2.95% | -2.45% | -3.02% | 119% | 101% |