| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0720 |
| --- | --- |

| Title: | Cross-check of JVET-O0449 Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-6103-766-240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This is a cross-check of contribution JVET-O0449 on Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar.

# Test Results

The software provided by the proponent was evaluated according the common test conditions. The software is based on VTM-5.0 with modifications to implement the technology described in JVET-O0449. The modifications are implemented under the macro ATM_DIMD.

The software as provided by the proponents was run on an Intel Xeon machine running with Ubuntu 18.04. The software was compiled with gcc v7.3.0.

The performance of the technology is shown below. The results below perfectly match the ones obtained by the proponent. The full results are available in the attached excel sheet.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.30% | -0.36% | 120% | 105% |
| Class A2 | -0.31% | -0.21% | -0.25% | 119% | 105% |
| Class B | -0.37% | -0.27% | -0.25% | 122% | 106% |
| Class C | -0.40% | -0.33% | -0.20% | 120% | 108% |
| Class E | -0.51% | -0.27% | -0.27% | 121% | 105% |
| Overall | -0.43% | -0.28% | -0.26% | 121% | 106% |
| Class D | -0.48% | -0.19% | -0.27% | 118% | 108% |
| Class F | -0.26% | -0.10% | -0.11% | 113% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.46% | -0.34% | 104% | 105% |
| Class A2 | -0.13% | -0.29% | -0.27% | 103% | 102% |
| Class B | -0.18% | -0.26% | -0.44% | 104% | 102% |
| Class C | -0.24% | -0.49% | -0.50% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.37% | -0.40% | 104% | 103% |
| Class D | -0.16% | -0.41% | -0.33% | 105% | 103% |
| Class F | -0.20% | -0.23% | -0.01% | 104% | 103% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.05% | -0.42% | 104% | 110% |
| Class C | -0.03% | -0.36% | -0.56% | 101% | 100% |
| Class E | -0.09% | 0.50% | 0.02% | 100% | 100% |
| Overall | -0.05% | 0.02% | -0.36% | 102% | 104% |
| Class D | -0.05% | -0.11% | -0.16% | 101% | 101% |
| Class F | -0.01% | 0.03% | -0.18% | 101% | 103% |

# Analysis of software and specification text

The software and specification text provided in contribution JVET-O0449 were checked. The software mostly implements the technology as described in JVET-O0449 and only minor differences and issues were detected. Many of the detected issues and differences have been addressed by the proponent in a second version of their contribution.

The minor issues detected with the specification text are:

- The specification text should be adapted to be more consistent with the style of the text in VVC draft 5, for example in the way new processes are invoked or when using global vs local variables in the intra prediction processes.

- The binarization of the syntax element dimd_flag, as well as its CABAC context derivation and initialization are missing in Tables 9-5, 9-11 and 9-17.

In addition, the cross-checker would like to bring to the attention of the group the following aspects.

- Divisions are currently used in the software for the purpose of calculating the angle of the edges in the neighboring blocks and for calculating the weights used for blending of the DIMD modes in the final prediction.

- The DIMD mode is currently always added to the searched modes in the encoder Rate-Distortion search, which explains the additional encoder runtime.

- The combination with PDPC is rather unclear, but some of it comes from the complicated implementation of PDPC in VTM-5.0. The current understanding of the combination of DIMD and PDPC by the cross-checker is summarized below.
As far as the cross-checker understands, PDPC is currently applied in IntraPrediction::xPredIntraAng for the angular modes using PDPC, but it is applied in IntraPrediction::predIntraAng for Planar, DC, horizontal and vertical modes. 
Since DIMD is calculated in function IntraPrediction::predIntraAng before applying the PDPC process, it therefore seems at first sight that PDPC is applied BEFORE blending of the three predictions when the first DIMD mode is an angular mode not equal to pure horizontal and vertical and AFTER blending otherwise.