| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0733 |
| --- | --- |

| Title: | Crosscheck of JVET-O0123: CE4-related: Alignment of BDOF refinement process with PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | Wei.Chen@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-O0123. The simulation results do not change CTC simulation results, which match with that reported by the proponents.

# Introduction

JVET-O0123 [1] proposes to align BDOF refinement process with that of PROF. Specifically, this proposal proposes to align BDOF’s refinement process (1) with PROF’s process (2) so that both refinement processes could share same processing logics to reduce hardware costs.

I'i,j=

Clip 0, 1023, (I0i,j+I1i,j+((( gx1i,j- gx0i,j)*Δvxi,j+(gy1i,j-gy0i,j)*Δvyi,j+1)≫1) +16400)≫5         (1)

I'i,j= Clip 0, 1023,  (I0i,j+Clip(-213, 213-1,  (((-gx0i,j)*Δvxi,j+(-gy0i,j)*Δvyi,j+1)≫1))+I1i,j+Clip(-213, 213-1, ((gx1i,j*Δvxi,j+gy1i,j*Δvyi,j+1)≫1)+16400)≫5 (2)

# Simulation results

The simulations were performed using VTM-5.0 with those changes proposed in [1], according to common test conditions. The encoding results compared to VTM-5.0 anchor are summarized in Table 1.

Table 1. Results compared to VTM-5.0 anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | -0.03% | 100% | 101% |
| Class A2 | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class B | 0.01% | 0.00% | -0.04% | 100% | 101% |
| Class C | -0.02% | 0.03% | 0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.01% | 0.05% | 100% | 101% |
| Class F | -0.01% | 0.04% | 0.02% | 100% | 101% |

# Reference

- J. Li, C. Lim, “CE4-related: Alignment of BDOF refinement process with PROF”, JVET-O0123, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.