| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0734 |
| --- | --- |

| Title: | Crosscheck of JVET-O0313: CE4-related: On Conditions for enabling PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | Wei.Chen@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-O0313. The simulation results do not change CTC simulation results, which match with that reported by the proponents.

# Introduction

JVET-O0313 [1] proposes a method to disabling PROF [2] on the affine blocks which may not benefit of applying PROF. According to [1], if one of the following conditions is true, PROF is disabled.

Condition 1: Control point motion vectors are all equal.

Condition 2: The size of reference block bounding box exceed a pre-defined threshold.

Condition 3 (Proposed by JVET-O0313 [1]): None of ΔmaxVx and ΔmaxVy is greater than the rounding threshold.

# Simulation results

There are two tests defined as below to be performed:

Test 1: Apply Condition 3 on top of CE4-2.1.a and remove Condition 1.

Test 2: Apply Condition 3 on top of CE4-2.1.b and remove Condition 1.

The simulations were performed using VTM-5.0 with those changes proposed in [1], according to common test conditions. The encoding results compared to CE4-2.1 are summarized in Table 1 and Table 2.

Table 1. Test 1 Results compared to CE4-2.1.a

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1.a |  |  |  |  | Over CE4-2.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 103% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 96% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |

Table 2 Test 2 Results compared to CE4-2.1.b

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1.b |  |  |  |  | Over CE4-2.1.b |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.03% | 101% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.08% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.04% | 101% | 100% | 0.03% | 0.01% | 0.06% | 100% | 100% |
| Class C | -0.02% | 0.02% | -0.02% | 101% | 100% | -0.03% | -0.18% | 0.30% | 100% | 99% |
| Class E |  |  |  |  |  | 0.06% | -0.47% | -0.62% | 101% | 99% |
| Overall | -0.01% | 0.00% | -0.02% | 101% | 100% | 0.02% | -0.17% | -0.03% | 100% | 99% |
| Class D | -0.01% | 0.01% | -0.01% | 100% | 101% | 0.03% | -0.44% | 0.28% | 101% | 99% |
| Class F | -0.01% | 0.03% | -0.04% | 101% | 100% | -0.02% | -0.01% | 0.37% | 101% | 100% |

# Reference

- Y.-C. Yang, P.-H. Lin, “CE4-related: On Conditions for enabling PROF”, JVET-O0313, Gothenburg, SE, July 2019.

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode,” JVET-N0236, Geneva, CH, March 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.