| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0741-v4 |
| --- | --- |

| Title: | Context memory reduction via selective adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Ted Hsieh, Jie Dong 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a proposal reduce the cost of supporting a large number of CABAC contexts by removing the adaptability of the least important contexts. It is shown that this can be done with a minimal change to the context implementation (a single “if” line added), no changes to the CABAC engine, and (unlike converting to bypass) no changes to the any other parts of VTM or the standard’s text. The change can yield significant savings in memory and hardware implementation costs because it changes the context memory requirement from 28 bits to 6 bits, and a part of the context-state total RAM can be replaced by cheaper ROM. Simulation results show that between 40% to 60% percent of the contexts can be converted with very small losses. Under CTC and 4 bits of precision for probabilities, conversion of 100 contexts from adaptive to non-adaptive results in 0.01% and 0.00% luma losses in AI and RA, and conversion of 200 contexts results in 0.06% and 0.xx% luma losses in AI and RA.

# Problem definition

In hardware implementations the cost of the CABAC contexts depends heavily on the amount of memory required for all contexts. This happens because entropy coding is a performance-critical part of the decoder, and thus needs to have its data readily available in very fast and expensive memory.

The type of memory is also very important: when using RAM, it is necessary to guarantee the minimum bandwidths for both reading and writing. ROM, on the other hand, needs only logic for addressing fixed information, and no bandwidth is required for writing (i.e., half the bandwidth is saved).

HEVC uses 202 contexts, and 8 bits to store the state of each context, while VTM-5.0 uses 424 contexts, and each context needs 24 bits of RAM to store the probability estimation state, and 4 bits of ROM to store the adaptation parameters.

A straightforward method to reduce complexity is to replace a CABAC context with bypass coding, but this has important disadvantages:

- For most contexts the replacement leads to significant coding loss (as expected, since implementers commonly measure bypass loss);

- The identification of which contexts can be replaced by bypass cannot be done automatically, because some changes can destroy the structure employed for coding (it can create exceptions within exceptions, etc.).  Thus, each case must be individually analyzed and discussed.

- Each conversion is laborious, as it requires changing several parts of the code, and of the standard text.

Thus, it is desirable to have a method that reduces the memory costs without the disadvantages listed above.

# Proposed solution

A simple modification to CABAC is to selectively disable adaptation of some contexts. Fig. 1 shows the main concept for the encoder (the decoder is defined by simply reversing input and output).

Fig. 1 – Binary arithmetic coding with selective adaptation.

As indicated in Fig. 1, one clear advantage is that the information needed for non-adaptive contexts can be store in inexpensive ROM.

Since the CABAC engine does not need to be modified, the changes in the coding implementation are very small. In an experimental code, like VTM, it can be done simply by

- Identifying (using 1 bit) the non-adaptive contexts in their probability initialization parameter and changing the initialization function to read a probability initializer with Nb bits (done once per slice);

- Adding one Boolean variable to each context; and testing its value before adaptation.

Methods for optimizing performance and storing the mode selection bits can be developed when the VVC standard is finished, and the sets of adaptive and non-adaptive contexts fully defined. For instance, the adaptation bit may be stored separately, or as a special code in the 4 bits used to define the context adaptation parameters.

While the implementation modifications are quite small, the most important remaining question is how many contexts can be converted to non-adaptive without incurring in significant coding loss.

Contexts can be replaced with bypass with little loss when we have the right combination of two factors:

- The context is rarely used;

- The average bit rate of the context is nearly 1 (nearly equiprobable bin values).

For best results it is necessary to measure how those two factors combine, and then decide on which context to modify.

Changing to non-adaptive context, on the other hand, can also exploit those factors, and in addition can be used for contexts that

- Code bin values with probabilities that are relatively stable over different video types and quality settings;

- Code bins that have low entropy (bin value probabilities very near 0 or 1).

Again, best results are obtained when the full effects are measured. Thanks to the tools that have been developed for re-training the CABAC initializers for each new VTM version, it is possible to use the following procedure

- Optimize the probability values for the non-adaptive contexts.

- Estimate the loss incurred when converting from adaptive to non-adaptive.

- Rank the contexts according to the estimated loss.

- Modify the Cn contexts with smallest loss from adaptive to non-adaptive.

The effectiveness of this approach depends on the accuracy of the loss estimates, which have been shown to be quite good by the CEs on arithmetic coding.

In the following sections the simulation results obtained with different values of probability representation bits Nb and modified contexts Cn. are presented.

# Summary of simulation results

Table I shows the average results for all simulations, under CTC conditions. Full results are in in Section 6, and the accompanying Excel files (which also include results for low delay P simulations).

Table I – Summary of simulation results: Cn contexts converted to non-adaptive, using Nb bits for bin value probabilities.

| Nb | Cn | AI |  |  | RA |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| 4 | 100 | 0.01% | 0.00% | -0.01% | 0.00% | 0.00% | -0.05% | -0.03% | -0.10% | -0.12% |
| 4 | 150 | 0.03% | -0.02% | -0.01% | 0.03% | 0.06% | 0.00% | 0.01% | -0.16% | -0.05% |
| 4 | 175 | 0.03% | 0.03% | 0.03% | 0.06% | 0.09% | 0.00% |  |  |  |
| 4 | 200 | 0.06% | -0.02% | -0.01% | 0.10% | 0.07% | 0.08% | 0.05% | -0.02% | -0.12% |
| 4 | 250 | 0.14% | 0.11% | 0.09% | 0.22% | 0.35% | 0.27% | 0.17% | 0.49% | -0.11% |
| 6 | 150 | 0.01% | -0.02% | 0.00% | 0.01% | -0.02% | 0.04% | 0.03% | -0.25% | -0.01% |
| 6 | 175 | 0.02% | -0.05% | -0.03% | 0.03% | -0.03% | -0.06% | -0.01% | 0.00% | 0.27% |
| 6 | 200 | 0.03% | 0.00% | 0.03% | 0.05% | 0.12% | 0.09% | 0.01% | 0.20% | 0.42% |
| 6 | 250 | 0.06% | 0.02% | -0.01% | 0.15% | 0.17% | 0.21% | 0.07% | 0.45% | 0.16% |

# Memory requirements and usage ratios

Table II shows the amount of memory needed for different values of Nb and Cn. Table III shows the ratio of bins that are decoded using the non-adaptive contexts.

Table II – Memory required for entropy coding (in bits) when Cn contexts converted are to non-adaptive and use Nb bits for bin value probabilities.

| Nb | Cn | RAM | Prob. ROM | Adap. ROM | Total ROM |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 10,176 | 0 | 1,696 | 1,696 |
| 4 | 100 | 7,776 | 400 | 1,296 | 1,696 |
| 4 | 150 | 6,576 | 600 | 1,096 | 1,696 |
| 4 | 175 | 5,976 | 700 | 996 | 1,696 |
| 4 | 200 | 5,376 | 800 | 896 | 1,696 |
| 4 | 250 | 4,176 | 1,000 | 696 | 1,696 |
| 6 | 150 | 6,576 | 900 | 1,096 | 1,996 |
| 6 | 175 | 5,976 | 1,050 | 996 | 2,046 |
| 6 | 200 | 5,376 | 1,200 | 896 | 2,096 |
| 6 | 250 | 4,176 | 1,500 | 696 | 2,196 |

Table III – Ratio of bins decoded with non-adaptive coding contexts while decoding sequence “BQMall” (8 frames for AI runs, and 17 frames for RA runs), Nb = 6 bits.

| Cn | AI |  | RA |  |
| --- | --- | --- | --- | --- |
|  | QP = 27 | QP = 32 | QP = 27 | QP = 32 |
| 100 | 1.5% | 0.9% | 1.6% | 1.0% |
| 150 | 6.1% | 4.7% | 5.9% | 4.7% |
| 200 | 11.9% | 9.5% | 11.7% | 9.8% |
| 250 | 22.6% | 18.75 | 22.6% | 19.4% |

# Conclusions and recommendations

The simulation results and memory analysis demonstrate that conversion of a set of adaptive arithmetic coding contexts from adaptive to non-adaptive can reduce RAM by 50% or more, with very minor coding loss. It is recommended to have this technique further studied in a CE.

# Full simulation results

The following results were obtained with VTM-5.0 code, CTC conditions. Since the simulations were run on a heterogeneous cluster, execution times are not accurate.

## Nb = 4 bits, Cn = 100 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.02% | 101% | 104% |
| Class A2 | 0.01% | -0.05% | -0.04% | 101% | 101% |
| Class B | 0.00% | -0.03% | -0.04% | 99% | 98% |
| Class C | 0.02% | 0.02% | 0.04% | 100% | 103% |
| Class E | 0.02% | -0.04% | -0.03% | 101% | 107% |
| Overall | 0.01% | 0.00% | -0.01% | 100% | 102% |
| Class D | -0.01% | 0.07% | -0.03% | 95% | 86% |
| Class F | 0.00% | 0.10% | 0.04% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.15% | 0.26% | 100% | 99% |
| Class A2 | 0.05% | 0.09% | 0.03% | 100% | 100% |
| Class B | 0.02% | -0.04% | -0.21% | 100% | 98% |
| Class C | -0.07% | -0.12% | -0.14% | 101% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.05% | 100% | 101% |
| Class D | -0.01% | 0.32% | 0.52% | 104% | 107% |
| Class F | 0.01% | -0.04% | 0.18% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.32% | -0.30% | 100% | 102% |
| Class C | -0.01% | -0.25% | -0.41% | 97% | 91% |
| Class E | -0.12% | 0.49% | 0.58% | 96% | 93% |
| Overall | -0.03% | -0.10% | -0.12% | 98% | 96% |
| Class D | 0.01% | 0.14% | 1.07% | 98% | 89% |
| Class F | 0.02% | -0.12% | -0.53% | 101% | 103% |

## Nb = 4 bits, Cn = 150 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.01% | -0.03% | 99% | 100% |
| Class A2 | 0.03% | 0.02% | 0.00% | 99% | 101% |
| Class B | 0.02% | -0.03% | -0.05% | 99% | 96% |
| Class C | 0.04% | -0.04% | 0.07% | 100% | 101% |
| Class E | 0.02% | -0.04% | -0.06% | 100% | 107% |
| Overall | 0.03% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.02% | 0.23% | -0.22% | 99% | 90% |
| Class F | 0.01% | -0.02% | -0.01% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.35% | 0.25% | 99% | 98% |
| Class A2 | 0.06% | 0.23% | 0.09% | 99% | 99% |
| Class B | 0.03% | 0.01% | -0.08% | 99% | 97% |
| Class C | -0.01% | -0.20% | -0.17% | 103% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.06% | 0.00% | 100% | 101% |
| Class D | 0.06% | 0.23% | 0.53% | 99% | 97% |
| Class F | -0.01% | -0.11% | -0.07% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.08% | -0.16% | 100% | 102% |
| Class C | 0.06% | -0.10% | -0.26% | 96% | 87% |
| Class E | -0.03% | -0.36% | 0.42% | 96% | 93% |
| Overall | 0.01% | -0.16% | -0.05% | 97% | 94% |
| Class D | 0.03% | -0.59% | 0.50% | 98% | 88% |
| Class F | 0.02% | 0.05% | -0.34% | 100% | 99% |

## Nb = 4 bits, Cn = 175 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.10% | 0.07% | 101% | 101% |
| Class A2 | 0.06% | 0.05% | 0.07% | 100% | 103% |
| Class B | 0.02% | -0.03% | -0.02% | 100% | 97% |
| Class C | 0.03% | 0.05% | 0.07% | 100% | 100% |
| Class E | 0.02% | 0.03% | -0.05% | 100% | 102% |
| Overall | 0.03% | 0.03% | 0.03% | 100% | 100% |
| Class D | 0.03% | 0.17% | -0.14% | 93% | 85% |
| Class F | 0.00% | 0.05% | -0.10% | 101% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.18% | 0.26% | 98% | 99% |
| Class A2 | 0.09% | 0.21% | -0.04% | 99% | 100% |
| Class B | 0.05% | 0.12% | -0.03% | 99% | 97% |
| Class C | 0.00% | -0.10% | -0.11% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.09% | 0.00% | 99% | 100% |
| Class D | 0.06% | 0.63% | 0.31% | 99% | 96% |
| Class F | 0.06% | -0.12% | 0.05% | 99% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.02% | 0.06% | -0.49% | 99% | 98% |
| Class E | -0.13% | -0.22% | -0.18% | 97% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.04% | 0.18% | 0.87% | 95% | 86% |
| Class F | 0.07% | -0.49% | -0.47% | 101% | 100% |

## Nb = 4 bits, Cn = 200 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.04% | 0.10% | 101% | 103% |
| Class A2 | 0.08% | 0.16% | 0.09% | 101% | 103% |
| Class B | 0.04% | -0.05% | -0.08% | 100% | 97% |
| Class C | 0.04% | -0.12% | 0.00% | 99% | 99% |
| Class E | 0.04% | -0.08% | -0.11% | 101% | 103% |
| Overall | 0.06% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.04% | 0.22% | -0.01% | 94% | 85% |
| Class F | 0.06% | 0.09% | -0.06% | 100% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.11% | 0.30% | 98% | 99% |
| Class A2 | 0.14% | 0.22% | 0.18% | 99% | 100% |
| Class B | 0.09% | 0.08% | -0.11% | 99% | 97% |
| Class C | 0.03% | -0.10% | 0.07% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.07% | 0.08% | 99% | 100% |
| Class D | 0.03% | 0.12% | 0.38% | 99% | 96% |
| Class F | 0.06% | 0.05% | -0.10% | 99% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.29% | 0.14% | 99% | 100% |
| Class C | 0.05% | 0.03% | -0.38% | 98% | 97% |
| Class E | -0.01% | -0.61% | -0.22% | 97% | 101% |
| Overall | 0.05% | -0.02% | -0.12% | 98% | 99% |
| Class D | 0.11% | -0.57% | 1.24% | 95% | 87% |
| Class F | 0.10% | -0.18% | 0.19% | 101% | 103% |

## Nb = 4 bits, Cn = 250 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.21% | 0.20% | 100% | 101% |
| Class A2 | 0.14% | 0.23% | 0.20% | 101% | 101% |
| Class B | 0.11% | 0.08% | 0.03% | 101% | 97% |
| Class C | 0.12% | 0.08% | 0.16% | 99% | 94% |
| Class E | 0.09% | 0.00% | -0.10% | 100% | 99% |
| Overall | 0.14% | 0.11% | 0.09% | 100% | 98% |
| Class D | 0.11% | 0.14% | 0.09% | 94% | 85% |
| Class F | 0.13% | 0.05% | 0.04% | 100% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.44% | 0.59% | 0.60% | 100% | 100% |
| Class A2 | 0.23% | 0.48% | 0.45% | 100% | 101% |
| Class B | 0.19% | 0.33% | 0.13% | 99% | 97% |
| Class C | 0.10% | 0.09% | 0.06% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.22% | 0.35% | 0.27% | 99% | 99% |
| Class D | 0.13% | 0.48% | 0.56% | 98% | 95% |
| Class F | 0.18% | 0.17% | 0.17% | 99% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.74% | -0.44% | 99% | 97% |
| Class C | 0.13% | 0.15% | 0.04% | 96% | 97% |
| Class E | 0.24% | 0.50% | 0.17% | 95% | 95% |
| Overall | 0.17% | 0.49% | -0.11% | 97% | 94% |
| Class D | 0.20% | -0.26% | 1.03% | 95% | 87% |
| Class F | 0.21% | 0.21% | -0.04% | 102% | 100% |

## Nb = 6 bits, Cn = 150 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | -0.02% | 101% | 102% |
| Class A2 | 0.01% | 0.04% | -0.01% | 101% | 103% |
| Class B | 0.00% | -0.04% | -0.04% | 100% | 98% |
| Class C | 0.01% | -0.04% | 0.06% | 100% | 108% |
| Class E | 0.01% | -0.10% | -0.01% | 102% | 110% |
| Overall | 0.01% | -0.02% | 0.00% | 101% | 103% |
| Class D | 0.01% | 0.18% | -0.06% | 96% | 87% |
| Class F | 0.01% | 0.00% | -0.02% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | 0.35% | 98% | 97% |
| Class A2 | 0.02% | 0.08% | -0.03% | 99% | 98% |
| Class B | 0.02% | -0.07% | -0.18% | 99% | 98% |
| Class C | 0.00% | -0.22% | 0.14% | 103% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | 0.04% | 100% | 101% |
| Class D | 0.02% | -0.01% | 0.51% | 102% | 101% |
| Class F | -0.02% | -0.09% | 0.11% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.14% | 0.07% | 99% | 100% |
| Class C | 0.03% | -0.25% | -0.13% | 98% | 96% |
| Class E | 0.11% | -0.42% | 0.03% | 97% | 97% |
| Overall | 0.03% | -0.25% | -0.01% | 98% | 98% |
| Class D | 0.05% | -0.20% | 0.67% | 96% | 87% |
| Class F | -0.12% | -0.63% | -0.46% | 102% | 103% |

## Nb = 6 bits, Cn = 175 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.02% | -0.01% | 103% | 105% |
| Class A2 | 0.03% | 0.00% | 0.02% | 100% | 103% |
| Class B | 0.01% | -0.09% | -0.04% | 99% | 94% |
| Class C | 0.01% | -0.09% | 0.00% | 101% | 108% |
| Class E | 0.02% | 0.01% | -0.11% | 102% | 111% |
| Overall | 0.02% | -0.05% | -0.03% | 101% | 103% |
| Class D | 0.00% | 0.09% | -0.04% | 93% | 84% |
| Class F | 0.00% | 0.03% | 0.06% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.14% | 0.20% | 98% | 99% |
| Class A2 | 0.08% | -0.01% | 0.07% | 99% | 99% |
| Class B | 0.01% | -0.04% | -0.25% | 98% | 95% |
| Class C | -0.02% | -0.14% | -0.10% | 103% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.03% | -0.06% | 99% | 100% |
| Class D | 0.02% | 0.35% | 0.35% | 98% | 96% |
| Class F | 0.03% | -0.01% | 0.18% | 101% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.27% | 0.35% | 99% | 99% |
| Class C | 0.02% | 0.13% | -0.02% | 99% | 98% |
| Class E | -0.12% | -0.63% | 0.51% | 98% | 101% |
| Overall | -0.01% | 0.00% | 0.27% | 99% | 99% |
| Class D | 0.01% | -0.34% | 0.55% | 96% | 88% |
| Class F | -0.02% | -0.51% | 0.24% | 100% | 99% |

## Nb = 6 bits, Cn = 200 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.08% | 0.05% | 102% | 101% |
| Class A2 | 0.04% | 0.07% | 0.10% | 101% | 102% |
| Class B | 0.02% | -0.04% | 0.03% | 100% | 97% |
| Class C | 0.04% | -0.04% | 0.01% | 99% | 104% |
| Class E | 0.02% | 0.01% | -0.03% | 101% | 108% |
| Overall | 0.03% | 0.00% | 0.03% | 100% | 102% |
| Class D | 0.02% | 0.20% | -0.05% | 95% | 86% |
| Class F | 0.03% | 0.10% | 0.20% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.28% | 0.34% | 0.12% | 0.28% |
| Class A2 | 0.06% | 0.25% | 0.27% | 98% | 98% |
| Class B | 0.05% | 0.11% | -0.05% | 99% | 97% |
| Class C | 0.00% | -0.06% | -0.05% | 101% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.12% | 0.09% | 99% | 100% |
| Class D | 0.06% | 0.10% | 0.16% | 99% | 95% |
| Class F | 0.01% | 0.00% | 0.28% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.56% | 0.23% | 99% | 101% |
| Class C | 0.03% | -0.19% | -0.08% | 98% | 95% |
| Class E | -0.08% | 0.11% | 1.39% | 97% | 96% |
| Overall | 0.01% | 0.20% | 0.42% | 98% | 98% |
| Class D | 0.03% | 0.04% | 1.39% | 96% | 86% |
| Class F | 0.08% | -0.42% | 0.05% | 102% | 104% |

## Nb = 6 bits, Cn = 250 contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.13% | 0.12% | 101% | 101% |
| Class A2 | 0.07% | 0.22% | 0.07% | 100% | 100% |
| Class B | 0.04% | -0.01% | -0.06% | 102% | 97% |
| Class C | 0.03% | -0.11% | -0.08% | 101% | 104% |
| Class E | 0.04% | -0.04% | -0.02% | 102% | 108% |
| Overall | 0.06% | 0.02% | -0.01% | 101% | 102% |
| Class D | 0.05% | 0.08% | 0.05% | 102% | 103% |
| Class F | 0.06% | 0.08% | 0.01% | 102% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.36% | 0.40% | 0.56% | 99% | 98% |
| Class A2 | 0.16% | 0.40% | 0.43% | 98% | 98% |
| Class B | 0.11% | 0.10% | 0.04% | 100% | 98% |
| Class C | 0.05% | -0.08% | -0.01% | 102% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.17% | 0.21% | 100% | 100% |
| Class D | 0.12% | 0.18% | 0.03% | 106% | 111% |
| Class F | 0.13% | 0.18% | 0.28% | 101% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | 0.68% | 0.46% | 99% | 100% |
| Class C | 0.05% | 0.03% | -0.24% | 98% | 95% |
| Class E | -0.02% | 0.61% | 0.19% | 97% | 97% |
| Overall | 0.07% | 0.45% | 0.16% | 98% | 98% |
| Class D | 0.09% | -0.11% | 0.05% | 98% | 88% |
| Class F | 0.10% | 0.23% | -0.12% | 103% | 103% |

# Patent rights declaration(s)

Qualcomm Incorporated, may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Needed after the CABAC’s context state is updated.

A non-adaptive context needs at most 6 bits for probability representation, since that is the number of bits used in computing the product range × probability.