| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0746-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0262 (Non-CE2: Removal of division operations in LMCS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Taoran Lu, Peng Yin 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | tlu@dolby.com |
| Source: | Dolby Laboratories, Inc., |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-O0262 (Non-CE2: Removal of division operations in LMCS). The simulation results reportedly match those provided by the proponents. Testing results on both SDR and HDR-H1 suggest the proposed method has insignificant impact on coding performance.

# Introduction

JVET-O0262 [1] proposes to replace all division operations in LMCS by a lookup table. The lookup table has 64 entries and each stores an 8-bit integer.

# Experimental Results

The simulations were performed based on VTM5.0 software following the CTC.

Table 1. BD-rate Results for SDR

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 104% | 101% |
| Class A2 | 0.00% | -0.03% | 0.00% | 98% | 97% |
| Class B | 0.01% | 0.02% | -0.13% | 101% | 99% |
| Class C | -0.02% | -0.08% | -0.15% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.08% | 101% | 100% |
| Class D | 0.05% | 0.00% | 0.04% | 102% | 103% |
| Class F | 0.01% | 0.00% | 0.01% | 101% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.17% | 0.21% | 102% | 102% |
| Class C | 0.03% | -0.20% | -0.24% | 99% | 99% |
| Class E | 0.11% | 0.26% | -0.15% | 91% | 98% |
| Overall | 0.04% | 0.07% | -0.03% | 98% | 100% |
| Class D | 0.02% | -0.04% | 0.70% | 96% | 101% |
| Class F | 0.00% | 0.02% | 0.15% | 96% | 99% |

Table 2. BD-rate Results for HDR over. HDR LMCS1 (LQP0)

|  | All Intra: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Class HDR-H1 | 0.03% | -0.01% | 0.00% | 0.04% | 0.02% |
|  | Random Access: BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Class HDR-H1 | -0.06% | -0.04% | -0.02% | -0.04% | -0.16% |

# References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, “Non-CE2: Removal of division operations in LMCS”, JVET-O0262, July 2019, Gothenburg, SE.