| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0747-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0627 (CE2-related: luma-chroma dependency reduction on chroma scaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Taoran Lu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | tlu@dolby.com |
| Source: | Dolby Laboratories, Inc., |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-O0627 (CE2-related: luma-chroma dependency reduction on chroma scaling). The simulation results reportedly match those provided by the proponents.

# Introduction

JVET-O0627 [1] proposes to derive chroma scaling factor at the CTU level instead of at the chorma block level. For each CTU, the left and top neighboring reconstructed luma samples are used.

# Experimental Results

There are four parameters settings in the proposal: M16-dualsingle, M16-single, M4-dualsingle and M4-single. Test M16-dualsingle is fully crosschecked under SDR CTC and all four cases have been tested under HDR CTC as additional results. All the tests are performed with the CRS lambda encoder bugfix [2] applied.

Table 1. BD-rate Results of SDR, M16-dualsingle

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.96% | 0.64% | 92% | 97% |
| Class A2 | -0.18% | 1.70% | 1.90% | 93% | 98% |
| Class B | 0.06% | -0.31% | -0.49% | 103% | 102% |
| Class C | 0.27% | -2.21% | -2.40% | 106% | 98% |
| Class E | 0.21% | -2.37% | -3.10% | 106% | 103% |
| Overall | 0.05% | -0.53% | -0.76% | 100% | 100% |
| Class D | 0.16% | -1.19% | -1.47% | 104% | 98% |
| Class F | 0.46% | -2.12% | -2.25% | 103% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.96% | -0.49% | 93% | 102% |
| Class A2 | 0.03% | -0.14% | -0.20% | 96% | 93% |
| Class B | 0.04% | -0.88% | -1.00% | 99% | 100% |
| Class C | 0.14% | -2.86% | -2.93% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -1.27% | -1.25% | 98% | 100% |
| Class D | 0.15% | -1.86% | -2.18% | 105% | 95% |
| Class F | 0.40% | -2.45% | -2.45% | 107% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.03% | -0.77% | 106% | 101% |
| Class C | 0.01% | -0.68% | -0.46% | 101% | 95% |
| Class E | -0.10% | -1.58% | -1.08% | 89% | 101% |
| Overall | -0.01% | -0.61% | -0.74% | 100% | 99% |
| Class D | 0.08% | -0.79% | 0.68% | 105% | 98% |
| Class F | 0.10% | -0.88% | -0.96% | 100% | 102% |

Table 2. BD-rate Results of HDR, M16-single

|  | O0627 M16ST Over vtm5 HDR Anchor (LMCS0 LQP1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra: BD-rate |  |  |  |  | Random Access: BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| HDR-H1 |  |  |  |  |  | 6.73% | -1.75% | -1.57% | 14.78% | 8.43% |

Table 3. BD-rate Results of HDR, M16-dualsingle

|  | O0627 M16DTST Over vtm5 HDR Anchor (LMCS0 LQP1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra: BD-rate |  |  |  |  | Random Access: BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| HDR-H1 | 2.70% | -1.49% | -1.03% | 5.41% | 1.20% | 2.82% | -1.32% | -1.12% | 7.02% | 3.21% |

Table 4. BD-rate Results of HDR, M4-single

|  | O0627 M16ST Over vtm5 HDR Anchor (LMCS0 LQP1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra: BD-rate |  |  |  |  | Random Access: BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| HDR-H1 |  |  |  |  |  | 6.81% | -1.76% | -1.56% | 14.74% | 8.75% |

Table 5. BD-rate Results of HDR, M4-dualsingle

|  | O0627 M4DTST Over vtm5 HDR Anchor (LMCS0 LQP1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra: BD-rate |  |  |  |  | Random Access: BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| HDR-H1 | 3.10% | -1.44% | -0.99% | 5.92% | 2.22% | 3.32% | -1.29% | -1.11% | 7.35% | 4.48% |

# References

- J. Chen, R.-L. Liao, Y. Ye, J. Luo, “CE2-related: luma-chroma dependency reduction on chroma scaling”, JVET-O0627, July 2019, Gothenburg, SE.

- T. Lu, P. Yin, E. François, F. Hiron, “Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree”, JVET-O0429, July 2019, Gothenburg, SE.