| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0748_v3 |
| --- | --- |

| Title: | Non-CE3: On signalling for intra coding mode in the MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, aramasub, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to improve the signalling performance for the Planar flag and the first bin of the MPM index. In the first test, the Planar flags of the ISP mode and regular mode are encoded using the same context; hence, only one context is used for the Planar flag. In the second test, the Planar flag is encoded using three contexts with Planar check of its neighbour which unified with the Planar check in the MPM list derivation. Test 3 is built on top of Test 2 where the first bin of MPM index is encoded using 3 contexts. Under AI and RA CTC condition, the average (Y, U, V) BD rate gains of the proposed tests are reported as follows:

Test 1: AI(0.00%, -0.02%, 0.01%) and RA(0.00%, -0.02%, -0.01xx%)

Test 2: AI(-0.04%, -0.06%, -0.10%) and RA(-0.01%, -0.04%, -0.06%).

Test 3: AI(-0.09%, -0.11%, -0.09%) and RA(-0.03%, -0.08%, -0.09%).

Additional tests: In this proposal, the performance of Test 2 and Test 3 combined with JVET-O0925 is also reported as:

Test 4: Test 2 + MPM modification JVET-O0484: AI(-0.17%, -0.17%, -0.16%) and RA(-0.01%, -0.02%, -0.03%)

Test 5: Test 3 + MPM modification JVET-O0484: AI(-0.21%, -0.22%, -0.23%) and RA(-0.02%, -0.02%, -0.03%).

# Introduction

The current design of VTM5.0 [1] unifies the MPM list of regular, ISP, and MRL intra coding modes. This unified MPM list includes 6 modes [2], in which the first mode of the list is always Planar while the modes of the remaining indices is dependent on the intra prediction mode of the neighbour blocks.

The Planar mode of the MPM list is signalled using a two-context CABAC encoder. These two contexts are decided based on whether the current intra block is encoded using the ISP mode or the regular intra mode. The index of the other modes in MPM list is encoded using a bypass encoder.

# Proposed Method

In our observation on the current VTM5.0, the probability (P) that a block is code using the regular intra mode and ISP mode is summarized in Table 1. From this result, the probability that a block is encoded using the Planar mode given Regular or ISP is calculated as: P(Planar | Regular) = 46% and P(Planar | ISP) = 56%. It is obvious that these probabilities are not notably different, and the probability of the ISP mode is not significant. Therefore, it is desired to encode the Planar flag with regardless of whether the block is encoded using ISP mode or the regular mode.

Table 1. Probability of intra coding mode in VTM5.0

| Regular |  |  | ISP |  |
| --- | --- | --- | --- | --- |
| 72.8% |  |  | 5.2% |  |
| Planar | Others |  | Planar | Others |
| 33.5% | 39.3% |  | 3.0% | 3.2% |

This proposal proposes three tests:

Test 1: MPM flag is encoded using 1 context without checking whether the block is ISP or Regular

The Planar flag is encoded using one context with regardless of whether the block is encoded using ISP mode or the regular mode.

Test 2: MPM flag is encoded using 3 contexts with checking Planar mode of its neighbours

To improve the performance of signalling, three contexts are used to encode the Planar flag. Compared to VTM5.0, this test has only 1 additional context.

The context index of encoding the Planar flag based on the coding mode information of the neighbour blocks. Particularly, the context index derivation is dependent on the Planar check of the neighbour blocks (left and top with the position depicted in Figure 1). The Planar check is unified with the check in MPM list derivation where a block is considered as Planar if one of the following conditions is true:

  - It is a MIP block

  - It is encoded using non intra prediction mode (Inter or IBC)

  - it is intra coded with the Planar mode.

The context index (ctxIdx ranging from 0 to 2) is calculated as:

ctxIdx = (left is Planae) + (top is Planar)

When the top block and the current block are not sharing the same CTU, the top block is not considered in context index derivation.

Figure 1. The position of the top and left block for context derivation.

Test 3: Test 2 + first bin of MPM index is CABAC encoded using 3 contexts.

This test is established on Test 2 where 3 contexts is used to encode the Planar flag. In this test, the signalling is further improved by encoding the first bin of MPM index with 3 more contexts. The contexts of the first bin and the Planar flag are identical so there is no additional context index derivation compared to Test 2.

# Experimental Results

The proposed tests have been implemented on top of VTM5.0. The experimental results under the AI and RA CTC configurations are summarized in Table 2, Table 3, Table 4, Table 5 and Table 6. It should be noted that, the context initialization values of Planar flag are copied form VTM5.0 where ones for the first bin of MPM is set equal to default values (154 and 8 for context and reset window size). Therefore, the performance of the proposed techniques could be improved further by retraining for the context initialization values.

It can be seen from these tables that, the average (Y, U, V) BD rate performances of these above alternatives are reported as follows with negligible encoding and decoding time change:

Test 1: AI(0.00%, -0.02%, 0.01%) and RA(0.00%, -0.02%, -0.01xx%)

Test 2: AI(-0.04%, -0.06%, -0.10%) and RA(-0.01%, -0.04%, -0.06%).

Test 3: AI(-0.09%, -0.11%, -0.09%) and RA(-0.03%, -0.08%, -0.09%).

Additional tests: In this proposal, the performance of Test 2 and Test 3 combined with the MPM list modification proposed in JVET-O0484 is also reported as:

Test 4: Test 2 + JVET-O0925: AI(-0.17%, -0.17%, -0.16%) and RA(-0.01%, -0.02%, -0.03%)

Test 5: Test 3 + JVET-O0925: AI(-0.21%, -0.22%, -0.23%) and RA(-0.02%, -0.02%, -0.03%).

Table 2 Performance of Test 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% |  | -0.02% |  | 100% |  | 98% |  |
| Class A2 | 0.01% | -0.04% |  | -0.05% |  | 98% |  | 97% |  |
| Class B | 0.00% | 0.01% |  | 0.07% |  | 101% |  | 100% |  |
| Class C | -0.01% | -0.07% |  | 0.05% |  | 101% |  | 104% |  |
| Class E | -0.01% | -0.07% |  | -0.07% |  | 99% |  | 103% |  |
| Overall | 0.00% | -0.02% |  | 0.01% |  | 100% |  | 101% |  |
| Class D | -0.01% | 0.05% |  | -0.07% |  | 100% |  | 98% |  |
| Class F | 0.01% | -0.01% |  | 0.05% |  | 100% |  | 104% |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% |  | 0.15% |  | 100% |  | 100% |  |
| Class A2 | 0.03% | 0.04% |  | 0.09% |  | 97% |  | 95% |  |
| Class B | -0.01% | -0.05% |  | -0.12% |  | 101% |  | 101% |  |
| Class C | -0.02% | -0.05% |  | -0.05% |  | 100% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.02% |  | -0.01% |  | 100% |  | 100% |  |
| Class D | 0.05% | -0.04% |  | 0.41% |  | 100% |  | 101% |  |
| Class F | 0.01% | 0.01% |  | 0.14% |  | 100% |  | 100% |  |

Table 3. Performance of Test 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.01% | -0.09% | 101% | 98% |
| Class A2 | -0.03% | -0.02% | -0.08% | 99% | 97% |
| Class B | -0.03% | -0.08% | -0.06% | 102% | 104% |
| Class C | -0.03% | -0.06% | -0.02% | 102% | 104% |
| Class E | -0.07% | -0.13% | -0.29% | 100% | 103% |
| Overall | -0.04% | -0.06% | -0.10% | 101% | 102% |
| Class D | -0.02% | 0.10% | -0.13% | 101% | 100% |
| Class F | -0.02% | 0.01% | -0.01% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.14% | 0.16% | 100% | 101% |
| Class A2 | 0.01% | 0.09% | -0.04% | 97% | 95% |
| Class B | 0.00% | -0.04% | -0.15% | 101% | 101% |
| Class C | -0.04% | -0.07% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | -0.06% | 100% | 100% |
| Class D | 0.03% | -0.13% | 0.38% | 100% | 101% |
| Class F | 0.02% | -0.03% | 0.08% | 101% | 101% |

Table 4. Performance of Test 3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.01% | -0.03% | -0.10% | -0.01% |
| Class A2 | -0.08% | -0.09% | -0.09% | -0.08% | -0.09% |
| Class B | -0.09% | -0.16% | -0.09% | 102% | 88% |
| Class C | -0.07% | -0.08% | -0.09% | 78% | 87% |
| Class E | -0.11% | -0.18% | -0.13% | 79% | 85% |
| Overall | -0.09% | -0.11% | -0.09% | 89% | 103% |
| Class D | -0.06% | 0.13% | -0.02% | 79% | 86% |
| Class F | -0.11% | -0.08% | -0.09% | 82% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.29% | 0.08% | 98% | 99% |
| Class A2 | -0.02% | 0.12% | -0.01% | 100% | 98% |
| Class B | -0.03% | -0.03% | -0.28% | 101% | 100% |
| Class C | -0.03% | -0.15% | -0.05% | 83% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.08% | -0.09% | 95% | 99% |
| Class D | 0.01% | 0.00% | 0.05% | 86% | 96% |
| Class F | -0.05% | 0.00% | 0.02% | 86% | 98% |

Table 5. Performance of Test 4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.18% | -0.20% | 90% | 97% |
| Class A2 | -0.12% | -0.14% | -0.18% | 92% | 101% |
| Class B | -0.15% | -0.19% | -0.14% | 77% | 85% |
| Class C | -0.15% | -0.07% | -0.07% | 69% | 102% |
| Class E | -0.27% | -0.27% | -0.25% | 67% | 97% |
| Overall | -0.17% | -0.17% | -0.16% | 78% | 95% |
| Class D | -0.09% | 0.10% | -0.25% | 77% | 77% |
| Class F | -0.10% | 0.05% | 0.01% | 79% | 86% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.24% | -0.14% | 92% | 93% |
| Class A2 | 0.01% | 0.10% | 0.19% | 97% | 98% |
| Class B | 0.03% | 0.08% | -0.02% | 74% | 77% |
| Class C | -0.03% | -0.06% | -0.13% | 78% | 89% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.03% | 83% | 87% |
| Class D | 0.09% | 0.26% | 0.23% | 81% | 79% |
| Class F | -0.02% | 0.04% | 0.08% | 76% | 79% |

Table 6. Performance of Test 5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.21% | -0.25% | 91% | 97% |
| Class A2 | -0.17% | -0.24% | -0.23% | 92% | 96% |
| Class B | -0.20% | -0.23% | -0.25% | 93% | 103% |
| Class C | -0.17% | -0.18% | -0.12% | 94% | 103% |
| Class E | -0.29% | -0.27% | -0.30% | 93% | 104% |
| Overall | -0.21% | -0.22% | -0.23% | 93% | 101% |
| Class D | -0.10% | 0.09% | -0.10% | 94% | 101% |
| Class F | -0.22% | 0.03% | -0.16% | 98% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.23% | -0.07% | 95% | 100% |
| Class A2 | 0.00% | 0.31% | 0.21% | 95% | 96% |
| Class B | -0.01% | -0.05% | -0.14% | 96% | 100% |
| Class C | -0.01% | -0.06% | -0.06% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.03% | 95% | 99% |
| Class D | 0.08% | 0.13% | 0.07% | 96% | 101% |
| Class F | -0.07% | -0.07% | 0.04% | 97% | 101% |

# Conclusions

This proposal proposes to improve the performance of signalling the intra modes in MPM list.

# References

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen, “ CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, the 14th JVET meeting, Mar. 2019, Geneva, CH.

- B. Wang, A. M. Kotra, S. Esenlik, H. Gao, E. Alshina, “Non-CE3: Modified CABAC Coding for Intra Prediction Mode”, JVET-O0111, the 1fth JVET meeting, Jul. 2019, Gothenburg, SE.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).