| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0755-v3 |
| --- | --- |

| Title: | Non-CE3: On signalling of MIP parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Thibaud Biatek Luong Pham Van Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a signalling mechanism for MIP parameters. It is asserted that the proposal has two major benefits: ability to define/add new MIP modes, and marking one or more pre-defined MIP modes as unavailable The signalling allows for parameters that are associated with one or more new MIP modes to be signalled independently or based on a previously signalled MIP mode parameter. The signalling also allows for certain pre-defined matrices to be marked as unavailable so as to arguably reduce the signalling cost of mode indices. It is remarked that the main advantage of MIP parameter signalling comes from the ability to design MIP modes that are suited to particular types of content. It is asserted that providing the flexibility to encoders to signal MIP mode in addition to those specified in the codec increases the usability of this intra prediction mode.

In version 2 of this document, some results are provided to indicate the potential signalling cost/efficiency of the proposed methods.

In version 3 of this document, more results are updated. For signalling unavailable modes, it is reported that BD-rate gains of 0.05% to 0.1% may be obtained.

# Introduction

Matrix intra prediction (MIP) was adopted in the 14th JVET meeting. Under MIP, prediction samples for a block are derived from a subset of the reference samples of the block using a matrix multiplication. The block sizes are classified into three categories, and under each category several MIP modes are defined. The encoder signals the mode from the category associated with the current block: the signalling mechanism may either use an MPM index (three MPMs are defined for MIP) or a remaining mode index. Table 1 shows the different categories of blocks and number of MIP modes defined for each category.

Table 1: Number of MIP modes and parameters for various block sizes

| Category | Block sizes | Number of MIP modes | Number of parameters |
| --- | --- | --- | --- |
| 0 | 4x4 | 35 | 1440 |
| 1 | 4x8, 8x4, 8x8 | 19 | 1440 |
| 2 | Other block sizes | 11 | 3456 |

Several variants and simplifications of the MIP tool have been proposed in this meeting, and it is likely that more such modifications are likely to occur. Although the MIP matrices are reportedly trained on a large set of image/sequence databases, it is still likely that for some content the MIP tool may not be useful. A safer way to design the tool would be to allow for some flexibility in the specification of the MIP parameters, e.g., by allowing signalling of some MIP parameters so that new matrices/modes may be defined for the codec.

# Proposal

This document proposes a signalling mechanism for MIP parameters. The mechanism assumes that a set of MIP modes are pre-defined in the codec. The signalling allows for new parameters to be signalled for new modes (signalling may be independent of other modes or may be dependent on one or more pre-defined modes.)

## Definitions

For ease of description, certain terms are defined:

MIP parameter set (MPS): a set of parameters that are associated with a MIP mode which is used to predict a block coded with the MIP mode. The MPS collectively refers to all the parameters that are associated with a MIP mode (e.g., matrix coefficients, offset vectors, etc.).

MPS subset: a subset of parameters of an MPS that is used to predict one sample in the predicted block. Under current MIP design, this sample may belong to a downsampled grid in the predicted block. MPS subset could refer to one row of the matrix used by a MIP mode and one entry of its offset vector.

Default MIP parameter set (DMPS): an MPS that is associated with a pre-defined MIP mode in the codec.

Additional MIP parameter set (AMPS): an MPS that is signalled in the bitstream and is associated with a new MIP mode. An AMPS may be signalled independently or signalled dependent on one or more MPS. The AMPS is signalled in the sequence parameter set.

Available MIP parameter set (Available MPS): a MIP parameter set that is available for a particular block. In the current design the available MIP parameter set is the same for all the blocks of pictures in a sequence. The available MIP parameter set is a union of all AMPS signalled in the SPS and a subset of all the DMPS.

## Signalling of set of AMPS

For simplicity, and for ease of specification, the MPS A associated with a MIP mode is defined as a two-dimensional array of parameters – one dimension representing the downsampled reference array and the other dimension representing the predicted sample array. Let NumRefSamples[ m ] and NumPredSamples[ m ] refer to the number of reference samples and number of predicted samples, respectively, in the m-th category of MIP modes; in this case the MPS A is a two-dimensional array of size NumPredSamples[ m ]xNumRefSamples[ m ] when MPS A belongs to category m. The parameters in A may be used to define a matrix and an offset vector. If there are more than NumRefSamples[ ] parameters that are specified for the MPS subset, the corresponding dimension of MPS A will be expanded to include those parameters (e.g., if there is an offset parameter, the size of A would be NumPredSamples[ m ] x ( NumRefSamples[ m ] + 1 ).)

## Signalling of independent and dependent AMPS

Let NDMPS[ m ] be the number of DMPS belonging to the m-th category specified in the codec. When an AMPS is specified to be dependent on a previously defined MPS, an index to the reference DMPS is signalled and the parameters are signalled as a delta value from the parameters of the reference DMPS.

It was considered whether the signalling of AMPS should be considered to be also dependent on previously signalled AMPS in the sequence parameter set. In some instances, such flexibility may provide some coding benefit. However, the authors were unable to justify such a signalling flexibility at this point of time and hence reference MPS are restricted to be DMPS in this proposal. Disallowing AMPS to be dependent on other AMPS also allows the parallel derivation/generation of AMPS.

## Derivation of available MPS

The available MPS for each picture is determined by an indication in the bitstream. In addition to the AMPS signalled in the SPS, the signalling also indicates which pre-defined DMPS are not to be considered as available for the sequence. This determination of available MPS is performed once every sequence, and does not change within a sequence.

## MPS for each category

When there is more than one category of blocks, a different set of DMPS is specified for each category. The AMPS are also specified for each category separately.

# Syntax and semantics

The syntax structure for the AMPS is specified in the sequence parameter set.

## Syntax

| sequence_parameter_set( ) | Descriptor |
| --- | --- |
| … |  |
| available_mps_present_flag | u(1) |
| if( available_mps_present_flag ) |  |
| available_mps_syntax( ) |  |
| … |  |

| available_mps_syntax( ) { | Descriptor |
| --- | --- |
| num_amps | ue(v) |
| for( i = 0; i < num_amps; i++ ) { |  |
| amps_type_idc[ i ] | u(2) |
| pred_amps_flag[ i ] | u(1) |
| num_bits_for_amps_param_val[ i ] | ue(v) |
| if( pred_amps_flag[ i ] ) |  |
| ref_mps_idx[ i ] | u(v) |
| for( j = 0; j < NumPredSamples[ amps_type_idc[ i ] ]; j++ ) { |  |
| if( pred_amps_flag[ i ] ) |  |
| pred_amps_subset_flag[ i ][ j ] | u(1) |
| for( k = 0; k < NumRefSamples[ amps_type_idc[ i ] ]; k++ ) // Index k may be allowed to exceed NumRefSamples[ ] if there are more parameters specified for a category |  |
| amps_param_val[ i ][ j ][ k ] | s(v) |
| } |  |
| } |  |
| modify_available_dmps_flag | u(1) |
| if( modifiy_available_dmps_flag ) |  |
| for( m = 0; m <= 2; m++ ) |  |
| for( n = 0; n < NDMPS[ m ]; n++ ) |  |
| dmps_included_flag[ m ][ n ] | u(1) |
| } |  |

## Semantics

The variables NumPredSamples[ ] and NumRefSamples[ ] are dependent on the category of the MIP mode.

available_mps_present_flag equal to 1 specifies that the available_mps_syntax( ) syntax structure is signalled in the SPS. available_mps_present_flag equal to 0 specifies that the available_mps_syntax( ) structure is not signalled in the SPS.

num_amps specifies the number of additional MPS signalled in the amps_syntax( ) structure. The value of num_amps shall be in the range of 0 to 15, inclusive.

amps_type_idc[ i ] specifies the MIP size ID associated with the i-th AMPS. The value of amps_type_idc[ i ] shall be in the range of 0 to 2, inclusive.

pred_amps_flag[ i ] equal to 1 specifies that i-th AMPS is dependent on another MPS that is specified and ref_mps_idx[ i ] is signalled. pred_amps_flag[ i ] equal to 0 specifies that the i-th AMPS is signalled independent of other MPS and ref_mps_idx[ i ] is not signalled.

num_bits_for_amps_param_val_minus1[ i ] specifies the number of bits used to signal the syntax element amps_param_val[ i ][ j ][ k ]. The value of num_bits_for_amps_param_val_minus1[ i ] shall be in the range of 0 to 5, inclusive.

ref_mps_idx[ i ] specifies the index of the MPS used to derive the parameters of the i-th MPS. The value of ref_mps_idx[ i ] shall be in the range of 0 to NDMPS[ amps_type_idc[ i ] ] – 1, inclusive. The MPS indicated by ref_mps_idx[ i ] is referred to as the reference MPS for the i-th AMPS.

When pred_amps_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that the value of MipSizeId[ ][ ] associated with the reference MPS of the i-th AMPS shall be equal to amps_type_idc[ i ].

pred_amps_subset_flag[ i ][ j ] equal to 1 specifies that the j-th subset of the i-th MPS is predicted from the j-th subset of the reference MPS. pred_amps_subset_flag[ i ][ j ] equal to 0 specifies that the j-th subset of the i-th MPS is not predicted from the reference MPS. When pred_amps_flag[ i ] is equal to 0, the value of pred_amps_subset_flag[ i ][ j ] is set inferred to be equal to 0 for all j in the range of 0 to NumPredSamples[ amps_type_idc[ i ] ] – 1, inclusive.

When pred_amps_flag[ i ] is equal to 1, the value of pred_amps_subset_flag[ i ][ j ] shall be equal to 1 for at least one value of j in the range of 0 to NumPredSamples[ amps_type_idc[ i ] ] – 1, inclusive.

amps_param_val[ i ][ j ][ k ] is used to derive the k-th parameter of the j-th subset of the i-th AMPS. The number of bits used to signal amps_param_val[ i ][ j ][ k ] is equal to num_bits_for_amps_param_val_minus1[ i ].

Let DmpsParamVal[ i ][ j ][ k ] specify the k-th parameter of the j-th subset of the i-th DMPS, for i in the range of 0 to NDMPS[ amps_type_idc[ i ] ] – 1, inclusive.

Let AmpsParamVal[ i ][ j ][ k ] specify the k-th parameter of the j-th subset of the i-th AMPS, for i in the range of 0 to num_amps – 1, inclusive. The variable AmpsParamVal[ i ][ j ][ k ] is derived as follows:

if( pred_amps_subset_flag[ i ][ j ] )
	AmpsParamVal[ i ][ j ][ k ] = DmpsParamVal[ ref_mps_idx[ i ] ][ j ][ k ] + 												amps_param_val[ i ][ j ][ k ]
else
	AmpsParamVal[ i ][ j ][ k ] = amps_param_val[ i ][ j ][ k ]

modify_available_dmps_flag equal to 1 specifies that the syntax element dmps_included_flag[ ][ ] is signalled. modify_available_dmps_flag equal to 0 specifies that the syntax element dmps_included_flag[ ][ ] is not signalled and all the DMPS are considered available for pictures that refer to the SPS.

dmps_included_flag[ m ][ n ] equal to 1 specifies that the n-th DMPS corresponding to the m-th category of MPS may be used for prediction by pictures that refer to the SPS. dmps_included_flag[ m ][ n ] equal to 0 specifies that the n-th DMPS corresponding to the m-th category of MPS shall not be used for prediction by pictures that refer to the SPS. When not signalled, the value of dmps_included_flag[ m ][ n ] is inferred to be equal to 1.

The available MPS for the pictures that refer to the SPS is derived as follows:

for( m = 0; m < 2; m++ ) {
	NumAvailableMps[ m ] = 0
	mpsCtr[ m ] = 0
	for( n = 0; n < NDMPS[ m ]; n++ )
		if( dmps_included_flag[ m ][ n ] )
			AvailableMpsIdx[ m ][ mpsCtr[ m ] ++ ] = n
	NumAvailableDmps[ m ] = mpsCtr[ m ]
}

for( i = 0; i < num_amps; i++ ) {
	m = amps_type_idc[ i ]
	AvailableMpsIdx[ m ][ mpsCtr[ m ] ++ ] = i
}

NumAvailableMps[ m ] = mpsCtr[ m ], for m = 0..2

The signalling MIP mode index will be modified to be dependent on the variable NumAvailableMps[ m ], e.g., the maximum value of MPS in category m will now be NumAvailableMps[ m ] instead of NDMPS[ m ]. If an mpsIdx is specified for indicating an MIP mode for category m, when mpsIdx < NumAvailableDmps[ m ], the DMPS with index equal to AvailableMpsIdx[ m ][ mpsIdx ] is chosen; otherwise, the AMPS with index equal to AvailableMpsIdx[ m ][ mpsIdx ] is chosen.

# Discussion

The proposed signalling has two major benefits: ability to define/add new MIP modes, and marking one or more pre-defined MIP modes as unavailable. This mechanism supports multiple use cases, some of which are listed below:

- Encoders that have the capability to train MIP mode parameters for particular content would now have the flexibility to signal their newly trained parameters. The signalled parameters would be defined as a new mode which could be used to more efficiently compress the content.

- For some content, some of the pre-defined matrices may not be used at all or may not provide sufficient performance. The pre-defined modes, however, are currently included and negatively affect the signalling cost of the MIP mode. If the modes that are not used are specified in the bitstream, the encoder would be able to specify the MIP mode more efficiently and the decoder would be able to identify the right mode. E.g., JVET-O0139 shows that several pre-defined matrices in the VVC may not be useful for the CTC. Instead of completely removing these matrices, a robust solution may be to indicate that some matrices are not used for a particular bitstream. This would keep the pre-defined matrices for non-CTC content (where it may be beneficial) but also allow for more efficient/less complex MIP mode signalling for some cases (e.g., CTC).

- Some encoders may not have the resources to train entirely new matrices but may have the capability to re-tune/modify existing mode parameters; if these modified parameters do not vary significantly from the pre-defined parameters, the parameter prediction of available_mps_syntax() using the pred_amps_flag and ref_mps_idx could be used to signal the modified mode parameters. This allows some flexibility in adding new modes, while not incurring the cost of signalling parameters of an independent new mode.

The authors would also like the group to consider the possibility of signalling the AMPS at finer granularity than the SPS, e.g., PPS or the APS. Some more restrictions may be needed in such cases to keep the complexity to a minimum.

# Results

This section provides some simulation results for the proposed methods. In all these tests, it is assumed that the MIP mode index is coded using truncated binary coding without an MPM specified for MIP.

## Test 1

In this test, the benefit of indicating unavaibility of MIP modes is tested. A subset of the MIP modes in the three MIP mode categories is chosen as available (subset chosen at random); the remaining MIP modes are considered unavailable. For the anchor of these tests, the unavailable modes are not chosen at the encoder; no other encoder changes are made. For the test, the available modes are signalled (using the proposed method) and the encoder and decoder only use the available modes to code the data; again no other encoder changes are made. The test is designed to show the potential benefit of indicating unavailable modes: the cost of signalling the parameters in the SPS vs ability to code the MIP mode index more efficiently.

As the subset of MIP modes available are chosen at random, three simulations were run to show that the gain is consistent across simulations. The results show that there is an improvement of roughly 0.05% luma BD-rate in AI configuration and 0.05% luma BD-rate in the RA configuration.

Table 2: Results for simulation 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.06% | 0.10% | 100% | 99% |
| Class A2 | -0.06% | 0.03% | -0.06% | 99% | 100% |
| Class B | -0.06% | 0.00% | 0.00% | 98% | 97% |
| Class C | -0.08% | 0.18% | 0.04% | 99% | 97% |
| Class E | -0.04% | -0.06% | -0.12% | 99% | 95% |
| Overall | -0.07% | 0.04% | 0.00% | 99% | 97% |
| Class D | -0.01% | -0.04% | -0.03% | 100% | 99% |
| Class F | -0.04% | 0.01% | 0.01% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.01% | -0.13% | 95% | 96% |
| Class A2 | -0.02% | 0.19% | 0.08% | 99% | 100% |
| Class B | -0.06% | -0.01% | 0.05% | 95% | 94% |
| Class C | -0.04% | 0.05% | 0.08% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.01% | -0.13% | 95% | 96% |
| Class D | -0.05% | 0.25% | 0.03% | 98% | 96% |
| Class F | 0.02% | 0.10% | 0.00% | 96% | 92% |

Table 3: Results for simulation 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.06% | -0.10% | 100% | 102% |
| Class A2 | -0.08% | -0.09% | -0.05% | 101% | 101% |
| Class B | -0.08% | -0.02% | -0.05% | 99% | 98% |
| Class C | -0.09% | 0.05% | 0.04% | 100% | 100% |
| Class E | -0.09% | -0.06% | -0.17% | 100% | 98% |
| Overall | -0.10% | -0.03% | -0.06% | 100% | 100% |
| Class D | -0.06% | 0.03% | -0.15% | 100% | 101% |
| Class F | -0.05% | -0.10% | -0.02% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.04% | -0.17% | 100% | 100% |
| Class A2 | -0.07% | -0.22% | 0.04% | 100% | 100% |
| Class B | -0.07% | -0.09% | -0.30% | 100% | 100% |
| Class C | -0.08% | 0.04% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.06% | -0.09% | 100% | 100% |
| Class D | -0.02% | -0.29% | -0.06% | 104% | 104% |
| Class F | -0.01% | -0.16% | 0.08% | 100% | 101% |

Table 4: Results for simulation 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.16% | -0.08% | 101% | 100% |
| Class A2 | -0.09% | -0.08% | -0.08% | 100% | 100% |
| Class B | -0.10% | -0.10% | -0.02% | 101% | 100% |
| Class C | -0.10% | 0.05% | -0.03% | 99% | 99% |
| Class E | -0.08% | -0.04% | -0.01% | 100% | 99% |
| Overall | -0.10% | -0.06% | -0.04% | 100% | 100% |
| Class D | -0.05% | -0.08% | -0.16% | 99% | 99% |
| Class F | -0.02% | 0.01% | 0.03% | 100% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over Enc-only unavailable |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.11% | -0.02% | 101% | 100% |
| Class A2 | -0.06% | -0.09% | 0.00% | 100% | 100% |
| Class B | -0.09% | 0.00% | -0.04% | 100% | 99% |
| Class C | -0.07% | -0.23% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.06% | -0.01% | 100% | 100% |
| Class D | -0.03% | 0.32% | 0.07% | 102% | 102% |
| Class F | 0.01% | -0.03% | 0.01% | 100% | 100% |

## Test 2

In this test, the cost of signalling a new mode is tested. The new modes are signalled independently, and three tests are performed: in each test, one additional mode for each category is signalled, i.e., tests 2a, 2b and 2c add one more for MIP category 0, 1, and 2, respectively. To only consider the signalling cost, it is assumed that one of the pre-defined matrices is signalled as the new mode so that only the cost of signalling is reflected; no other changes are made to the encoder.

The total size of parameters for one mode for category 0 is less than that of category 1, which in turn is less than that of category 2. The cost of signalling is clearly less in the RA configuration because the SPS is not signalled for every picture. Morever, it is likely that in practical use cases of this feature of the proposal, it is more likely that new matrices would be signalled when the gain of the new mode is sufficient to compensate the signalling cost. It may also be more beneficial to signal these matrices for larger sequences, or larger bit rates where the cost of signalling the new modes is less. Furthermore, combinations with the other features of the proposed method (signalling the new mode parameters as delta-coded from pre-defined mode parameters, or indicating unavailable matrices) may reduce the cost of signalling the new modes.

Table 5: Cost of signalling one mode of category 0 in the SPS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.14% | 0.17% | 98% | 99% |
| Class A2 | 0.07% | -0.02% | 0.01% | 100% | 101% |
| Class B | 0.17% | 0.14% | 0.16% | 101% | 102% |
| Class C | 0.42% | 0.36% | 0.43% | 99% | 97% |
| Class E | 0.54% | 0.46% | 0.49% | 99% | 96% |
| Overall | 0.26% | 0.21% | 0.25% | 99% | 99% |
| Class D | 1.17% | 1.21% | 1.05% | 98% | 97% |
| Class F | 0.39% | 0.34% | 0.41% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.11% | 98% | 101% |
| Class A2 | 0.02% | 0.08% | 0.01% | 100% | 102% |
| Class B | 0.04% | 0.09% | -0.04% | 97% | 97% |
| Class C | 0.06% | -0.03% | -0.09% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | -0.01% | 99% | 101% |
| Class D | 0.24% | 0.43% | 0.38% | 95% | 93% |
| Class F | 0.14% | 0.12% | 0.12% | 99% | 98% |

Table 6: Cost of signalling one mode of category 1 in the SPS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.23% | 0.26% | 101% | 103% |
| Class A2 | 0.11% | 0.02% | 0.05% | 100% | 104% |
| Class B | 0.31% | 0.27% | 0.29% | 102% | 107% |
| Class C | 0.78% | 0.68% | 0.76% | 100% | 100% |
| Class E | 1.00% | 0.87% | 0.91% | 99% | 96% |
| Overall |  |  |  |  |  |
| Class D | 2.21% | 2.14% | 2.00% | 97% | 96% |
| Class F | 0.73% | 0.65% | 0.72% | 104% | 110% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.11% | -0.02% | 101% | 103% |
| Class C | 0.13% | 0.03% | -0.03% | 102% | 107% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.44% | 0.61% | 0.57% | 99% | 99% |
| Class F | 0.28% | 0.26% | 0.25% | 102% | 104% |

Table 7: Cost of signalling one mode of category 2 in the SPS

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.70% | 0.73% | 0.76% | 102% | 102% |
| Class A2 | 0.36% | 0.30% | 0.33% | 100% | 102% |
| Class B | 1.16% | 1.05% | 1.10% | 103% | 108% |
| Class C | 2.98% | 2.62% | 2.74% | 100% | 101% |
| Class E | 3.78% | 3.34% | 3.42% | 100% | 99% |
| Overall | 1.79% | 1.60% | 1.67% | 101% | 103% |
| Class D | 8.40% | 7.71% | 7.69% | 98% | 97% |
| Class F | 2.77% | 2.51% | 2.57% | 102% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.04% | 0.19% | 99% | 101% |
| Class A2 |  |  |  |  |  |
| Class B | 0.22% | 0.26% | 0.13% | 102% | 103% |
| Class C | 0.54% | 0.41% | 0.35% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 1.64% | 1.73% | 1.70% | 99% | 99% |
| Class F | 1.12% | 1.06% | 1.05% | 102% | 104% |

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).