| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0781 |
| --- | --- |

| Title: | Crosscheck of JVET-O0271: CE2-related: Chroma scaling for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu | Tel: Email: | 1-858-209-3851 xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Verification

The document reports the results of crosschecking JVET-O0271 which reduces the luma-chroma dependency of the chroma scaling for the CIIP mode. The software was provided by the proponent and it is verified that the software implementation is matched with what is described in JVET-O0271. After the cross-checking, all the BD-rates results match exactly with those in JVET-O0271. Due to heterogeneous cores that are used, the encoding and decoding time may not be reliable for the cross-check results.

Method 1.1 is built on top of method 1. In addition, CIIP is treated as inter prediction mode for chroma scaling. For each chroma TB of CIIP CU, the reference luma sample is the top-left inter prediction sample, rather than the final combined prediction sample, of the collocated luma TB. Therefore, the chroma scaling factor can be derived right after obtaining the inter prediction sample.

Method 1.2 is built on top of method 1. In addition, CIIP is treated as intra prediction mode for chroma scaling. For each chroma TB of CIIP CU, the reference luma smaple is the top-left reconstructed boundary sample of the collocated luma TB.

In method 3, regardless of the current CU mode, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB. Moreover, the chroma scaling factor of the first chroma TB in one VPDU (i.e., 64x64-L/32x32-C unit) is shared in the entire VPDU. In method 3, there is no additional latency between luma reconstruction and chroma reconstruction when applying chroma scaling, since the top-left reconstructed boundary sample of the VPDU is already derived before processing the current VPDU.