| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0783-v2 |
| --- | --- |

| Title: | AHG17: adding control of the transform skip coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add high-level syntax elements to enable or disable the alternative residual coding that is dedicated for transform skip (TS) and block difference pulse code modulation (BDPCM). When the alternative residual coding is disabled in the sequence parameter set (SPS) or picture parameter set (PPS), the original residual coding is applied.

# Introduction

In JVET-N1001-v9 [1], there are control flags in the PPS for the TS process. When TS or BDPCM (TS is always used when BDPCM is selected) is selected, an alternative residual coding is applied.

# Proposed method

This contribution proposes to add high-level control for TS, BDPCM, and the alternative residual coding.

## Option 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| … |  |
| if (sps_transform_skip_enabled_flag ) { |  |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) { |  |
| pps_bdpcm_enabled_flag | u(1) |
| pps_alternative_residual_coding_flag | u(1) |
| } |  |
| } |  |
| if(transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 && pps_bdpcm_enabled_flag ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ]) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else if( pps_alternative_residual_coding_flag ) |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

sps_transform_skip_enabled_flag equal to 1 specifies that transform_skip_enabled_flag may be present in the PPS; equal to 0 specifies that transform_skip_enabled_flag is not present in the PPS.

pps_bdpcm_enabled_flag equal to 1 specifies that BDPCM is allowed; equal to 0 specifies that BDPCM is disallowed.

pps_alternative_residual_coding_flag equal to 1 specifies that residual_ts_coding() is applied for TS and BDPCM; equal to 0 specifies that residual_coding() is applied for TS and BDPCM.

It is proposed to set both pps_bdpcm_enabled_flag and pps_alternative_residual_coding_flag to 1 for Class F and Class TGM and 0 for the rest test sequences in the common test conditions (CTC).

## Option 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| sps_bdpcm_enabled_flag | u(1) |
| sps_alternative_residual_coding_flag | u(1) |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| … |  |
| transform_skip_enabled_flag | u(1) |
| if(sps_transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 && sps_bdpcm_enabled_flag ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ]) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else if( sps_alternative_residual_coding_flag ) |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

sps_transform_skip_enabled_flag equal to 1 specifies that sps_transform_skip_flag may be present in the transform unit syntax. sps_transform_skip_enabled_flag equal to 0 specifies that sps_transform_skip_flag is not present in the transform unit syntax.

sps_bdpcm_enabled_flag equal to 1 specifies that BDPCM is allowed; equal to 0 specifies that BDPCM is disallowed.

sps_alternative_residual_coding_flag equal to 1 specifies that residual_ts_coding() is applied for TS and BDPCM; equal to 0 specifies that residual_coding() is applied for TS and BDPCM.

It is proposed to set both sps_bdpcm_enabled_flag and sps_alternative_residual_coding_flag to 1 for Class F and Class TGM and 0 for the rest test sequences in the CTC.

# References

- “Versatile Video Coding (Draft 5),” JVET-N1001-v9, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).