| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0787-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0282 (CE8-releated: Bug Fix in CE8-2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0282 proposed by Qualcomm. JVET-O0282 reports a bug fix in CE test 8-2.1. The verification task was done, and the BD-rate results of all of the tests match exactly with those in JVET-O0282.

# Verification

Qualcomm reported a bug fix [1] for CE test 8-2.1 [2]. Six verification tests were carried out. Verification 1 and verification 2 are conducted according to CE8 description [3] with 4:2:0 test sequences. Verification 1 was tested under common test conditions (CTC) [4], and verification 2 was tested under low QP conditions. Verification 3 and verification 4 were conducted with 4:4:4 test sequences and DualTree on for all classes. Verification 3 was tested under CTC, and verification 4 was tested under low QP conditions. Verification 5 and verification 6 were conducted with 4:4:4 test sequences and DualTree off for all classes. Verification 4 was tested under CTC, and verification 5 was tested under low QP conditions. For all tests, the all intra, random access, and low delay B configurations were tested, and the IBC, HashME, and RDPCM were turned on only for Class F and Class TGM. It is noted that the run time of all tests are not reliable.

Table 1 to Table 4 show the results of CE8.2.1 after bug fixing compared with VTM5.0 [5] under different test conditions. The BD-rates from our experimental results match exactly those in JVET-O0282.

Note: Table 2 does not contain full results. There are two simulation points not finished in Class A1 and Class A2 in RA.

Table 1. Results of the CE8.2.1 after bug fixing on test sequences 4:2:0 under CTC

|  | Over VTM-5.0, CTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.02% | 122% | 105% |
| Class A2 | 0.04% | 0.07% | 0.06% | 135% | 94% |
| Class B | 0.05% | 0.02% | 0.06% | 123% | 110% |
| Class C | 0.05% | 0.06% | 0.06% | 138% | 100% |
| Class E | 0.06% | 0.04% | -0.08% | 138% | 92% |
| Overall | 0.04% | 0.03% | 0.02% | 131% | 101% |
| Class D | 0.03% | 0.26% | -0.02% | 119% | 122% |
| Class F | -0.72% | -0.56% | -0.83% | 126% | 111% |
| Class TGM | -4.29% | -4.53% | -3.89% | 120% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.13% | 122% | 95% |
| Class A2 | 0.06% | -0.11% | 0.12% | 117% | 88% |
| Class B | 0.05% | -0.02% | -0.29% | 120% | 96% |
| Class C | 0.00% | -0.03% | -0.11% | 120% | 90% |
| Overall | 0.03% | -0.06% | -0.08% | 120% | 93% |
| Class D | 0.04% | -0.17% | 0.15% | 115% | 105% |
| Class F | -0.55% | -0.83% | -0.77% | 127% | 87% |
| Class TGM | -1.71% | -2.54% | -2.15% | 120% | 88% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | 0.60% | -0.12% | 147% | 111% |
| Class C | 0.04% | 0.23% | -0.12% | 114% | 89% |
| Class E | 0.06% | -0.05% | 0.02% | 119% | 72% |
| Overall | 0.04% | 0.31% | -0.09% | 128% | 92% |
| Class D | 0.06% | 0.26% | 0.36% | 116% | 105% |
| Class F | -0.28% | -0.75% | -0.72% | 123% | 81% |
| Class TGM | -0.43% | -0.57% | -0.47% | 121% | 82% |

Table 2. Results of the CE8.2.1 after bug fixing on test sequences 4:2:0 under low QP conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.03% | 120% | 117% |
| Class A2 | 0.01% | 0.02% | 0.01% | 133% | 103% |
| Class B | -0.15% | -0.16% | -0.16% | 123% | 115% |
| Class C | -0.10% | -0.17% | -0.14% | 134% | 104% |
| Class E | -0.86% | -1.69% | -2.10% | 137% | 99% |
| Overall | -0.21% | -0.36% | -0.43% | 129% | 108% |
| Class D | -0.02% | -0.27% | -0.31% | 116% | 119% |
| Class F | -0.80% | -1.06% | -1.06% | 127% | 118% |
| Class TGM | -11.07% | -10.22% | -9.79% | 124% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.03% | -0.03% | 0.00% | 119% | 121% |
| Class C | -0.04% | -0.08% | -0.04% | 117% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.05% | -0.01% | -0.02% | 110% | 115% |
| Class F | -0.69% | -0.90% | -1.08% | 123% | 98% |
| Class TGM | -5.30% | -4.79% | -4.71% | 115% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.01% | -0.02% | 144% | 126% |
| Class C | 0.00% | 0.03% | 0.00% | 117% | 104% |
| Class E | -0.03% | -0.11% | -0.25% | 117% | 99% |
| Overall | -0.01% | -0.02% | -0.07% | 128% | 111% |
| Class D | 0.00% | -0.06% | 0.01% | 114% | 119% |
| Class F | -0.46% | -0.58% | -0.62% | 124% | 104% |
| Class TGM | -3.64% | -2.86% | -2.66% | 120% | 87% |

Table 3. Results of the CE8.2.1 after bug fixing on test sequences 4:4:4 with CTC QPs, DualTree =1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.54% | -13.69% | -13.62% | 127% | 99% |
| TGM 720p | -1.58% | -7.03% | -8.55% | 126% | 102% |
| Animation | 0.06% | -1.10% | -0.75% | 126% | 115% |
| Mixed content | 0.11% | -5.74% | -5.58% | 128% | 106% |
| Camera-Captured content | 0.06% | 0.03% | 0.02% | 126% | 107% |
| Overall | -2.13% | -6.81% | -7.13% | 127% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.40% | -13.28% | -12.89% | 115% | 75% |
| TGM 720p | -0.74% | -7.07% | -8.29% | 123% | 70% |
| Animation | 0.09% | -0.46% | -0.36% | 125% | 87% |
| Mixed content | -0.45% | -6.84% | -6.12% | 126% | 72% |
| Camera-Captured content | 0.05% | 0.08% | -0.06% | 126% | 94% |
| Overall | -1.71% | -6.85% | -6.93% | 122% | 77% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.43% | -10.81% | -10.28% | 139% | 82% |
| TGM 720p | -1.13% | -5.67% | -6.86% | 141% | 66% |
| Animation | 0.03% | -0.22% | -0.26% | 136% | 90% |
| Mixed content | -0.24% | -6.25% | -5.92% | 146% | 76% |
| Camera-Captured content | 0.04% | -0.07% | 0.00% | 145% | 103% |
| Overall | -1.79% | -5.68% | -5.79% | 141% | 80% |

Table 4. Results of the CE8.2.1 after bug fixing on test sequences 4:4:4 with low QPs, DualTree =1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -25.71% | -28.07% | -28.07% | 134% | 80% |
| TGM 720p | -9.08% | -11.97% | -12.20% | 136% | 99% |
| Animation | -0.70% | -1.09% | -1.16% | 131% | 117% |
| Mixed content | -3.90% | -3.75% | -3.60% | 138% | 110% |
| Camera-Captured content | 0.01% | 0.01% | 0.02% | 126% | 121% |
| Overall | -10.15% | -11.57% | -11.61% | 134% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -22.87% | -26.72% | -25.72% | 106% | 71% |
| TGM 720p | -8.57% | -11.68% | -12.62% | 124% | 76% |
| Animation | -0.28% | -0.47% | -0.46% | 122% | 110% |
| Mixed content | -2.02% | -1.62% | -1.35% | 125% | 85% |
| Camera-Captured content | 0.01% | -0.01% | 0.01% | 124% | 117% |
| Overall | -8.82% | -10.63% | -10.55% | 119% | 85% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -22.05% | -25.57% | -24.79% | 137% | 71% |
| TGM 720p | -6.92% | -9.19% | -10.72% | 143% | 77% |
| Animation | -0.04% | -0.18% | -0.14% | 132% | 110% |
| Mixed content | -1.60% | -1.08% | -1.00% | 144% | 89% |
| Camera-Captured content | -0.01% | -0.01% | -0.01% | 141% | 121% |
| Overall | -8.05% | -9.51% | -9.69% | 140% | 86% |

Table 5. Results of the CE8.2.1 after bug fixing on test sequences 4:4:4 with CTC QPs, DualTree =0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -16.41% | -22.86% | -22.49% | 118% | 92% |
| TGM 720p | -4.64% | -10.72% | -12.74% | 120% | 100% |
| Animation | 0.04% | -0.24% | -0.19% | 124% | 117% |
| Mixed content | -0.76% | -4.48% | -5.21% | 120% | 107% |
| Camera-Captured content | 0.01% | 0.01% | 0.04% | 120% | 109% |
| Overall | -5.76% | -9.88% | -10.46% | 120% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.64% | -17.44% | -17.01% | 112% | 75% |
| TGM 720p | -4.62% | -9.89% | -12.21% | 122% | 68% |
| Animation | 0.05% | -0.16% | -0.17% | 125% | 85% |
| Mixed content | -0.42% | -3.11% | -4.17% | 127% | 72% |
| Camera-Captured content | -0.01% | 0.24% | 0.18% | 123% | 93% |
| Overall | -4.15% | -7.90% | -8.62% | 121% | 76% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.54% | -11.48% | -11.07% | 140% | 82% |
| TGM 720p | -1.72% | -6.34% | -7.97% | 142% | 68% |
| Animation | 0.06% | -0.36% | -0.03% | 138% | 98% |
| Mixed content | 0.52% | -1.99% | -3.20% | 142% | 76% |
| Camera-Captured content | -0.03% | 0.04% | -0.12% | 146% | 102% |
| Overall | -2.09% | -5.19% | -5.74% | 141% | 81% |

Table 6. Results of the CE8.2.1 after bug fixing on test sequences 4:4:4 with low QPs, DualTree =0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -34.09% | -38.68% | -38.24% | 117% | 76% |
| TGM 720p | -15.38% | -17.13% | -18.09% | 123% | 95% |
| Animation | -0.11% | -0.23% | -0.23% | 120% | 117% |
| Mixed content | -3.60% | -4.50% | -4.42% | 122% | 109% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 123% | 120% |
| Overall | -13.92% | -15.81% | -15.94% | 121% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -24.10% | -31.41% | -30.15% | 103% | 67% |
| TGM 720p | -12.94% | -15.09% | -16.70% | 119% | 75% |
| Animation | -0.02% | -0.11% | -0.07% | 125% | 111% |
| Mixed content | -2.30% | -2.83% | -3.31% | 122% | 80% |
| Camera-Captured content | 0.00% | 0.01% | -0.03% | 124% | 118% |
| Overall | -10.34% | -12.98% | -13.17% | 116% | 83% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -25.60% | -29.49% | -28.57% | 136% | 68% |
| TGM 720p | -10.03% | -11.79% | -13.22% | 137% | 73% |
| Animation | 0.01% | -0.04% | -0.08% | 133% | 114% |
| Mixed content | -1.58% | -2.22% | -2.93% | 142% | 86% |
| Camera-Captured content | 0.00% | 0.00% | -0.01% | 138% | 120% |
| Overall | -9.81% | -11.46% | -11.74% | 137% | 84% |

# References

[1] C.-H. Hung, Y.-H. Chao, W.-J. Chien, V. Seregin, and M. Karczewicz, “CE8-related: Bug fix in CE8-2.1,” Document of Joint Video Experts Team, JVET-O0282.

[2] Y.-H. Chao et al, “CE8-2.1: Palette mode in HEVC,” Document of Joint Video Experts Team, JVET-O0119.

[3] X. Xu, Y.-H. Chao, Y.-C. Sun, and J. Xu, “Description of Core Experiment 8 (CE8): Screen content coding tools,” Document of Joint Video Experts Team, JVET-N1028.

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[5] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).