| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0789-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0575 (CE7-related: Reduction for context coded bins in transform skip mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0575 proposed by Bytedance. JVET-O0575 proposed to limit the number of context coded bins for transform skip (TS) mode based on the block size, with less context coded bins for small blocks. The verification task was done, and the BD-rate results of all of the tests match exactly with those in JVET-O0575.

# Verification

Bytedance provided the source code of their proposed method described in JVET-O0575 [1]. A verification test was carried out.

The tests are conducted under the common test conditions (CTC) defined in [2] and low QP conditions requested in CE7 [3] using the VTM5.0 software [4]. Table 1 and Table 2 show the results of using VTM5.0 as the anchor and enabling the proposed method under CTC and low QP, respectively. The BD-rates from our experimental results match exactly those in JVET-O0575. Only Class F and TGM are tested with AI and RA. It is noted that the run time data are not reliable.

Table 1. The results of test using VTM5.0 as anchor under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC, re-run |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.25% | 0.26% | 0.07% | 145% | 158% |
| Class TGM | 0.53% | 0.45% | 0.45% | 144% | 138% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC, re-run |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.18% | 0.14% | 0.25% | 141% | 190% |
| Class TGM | 0.31% | 0.34% | 0.23% | 141% | 151% |

Table 2. The results of test using VTM5.0 as anchor under low QP conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC, re-run |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.29% | 0.21% | 0.26% | 101% | 117% |
| Class TGM | 0.92% | 0.77% | 0.76% | 99% | 110% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC, re-run |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.75% | 0.22% | 0.15% | 101% | 122% |
| Class TGM | -2.67% | 0.54% | 0.58% | 103% | 126% |

# References

[1] W. Zhu, L. Zhang, J. Xu, K. Zhang, and H. Liu, “CE7-related: Reduction for context coded bins in transform skip mode,” Document of Joint Video Experts Team, JVET-O0575.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team, JVET-N1027.

[4] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).