| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0800-v2 |
| --- | --- |

| Title: | AHG12: A summary of HLS proposals on coded picture regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of high-level syntax proposals submitted to this JVET meeting related to coded picture regions, including sub-pictures, slices, tiles, bricks, and wavefront parallel processing (WPP). A list of design questions is provided for each categories of proposals to help structure the review of the proposals.

In v2 of this contribution, some updates are made to the document categoriztion and the summary.

# Sequence-level independent regions (10)

## List of design questions

### On coding of sequence-level independent regions

- To support coding of sequence-level independent regions based on a normative approach or based on non-normative encoder motion constraints like in MCTS as in HEVC?

  - Based on a normative approach (all the 10 proposals in 6.18.1)

  - Based on encoder motion constraints (none)

- If to support coding of sequence-level independent regions based on a normative approach,

  - to have the normative approach based on treating of rectangular regional boundaries as picture boundaries in decoding motion compensation (O0138, O0141, O0394)

    - Should treating of rectangular regional boundaries as picture boundaries based on sub-pictures or slices?

      - Based on slices (O0138, O0394)

      - Based on sub-pictures (O0141, O0394)

  - to have the normative approach based on other approaches (none)

- To code sequence-level independent regions of one picture in one layer or in multiple layers?

  - One layer, all sub-pictures of one picture are in one layer (O0138, O0141, O0176, O0182, O0183, O0394, O0493, O0555, O0700)

  - Multiple layers, each sub-picture region is in its own layer (O0334)

### On signalling of sequence-level independent regions

- Where to signal location, ID, and size of sequence-level independent region?

  - In the SPS (O0141, O0555)

  - In the PPS (O0176)

  - In the VPS (O0182, O0334)

  - In a new decoding control NAL unit (O0394)

  - In a new SEI message (O0700)

- If to specify conformance for sequence-level independent regions, where to signal the conformance region (e.g., output sub-picture sets)?

  - In the SPS (O0141, O0555)

  - In the VPS (O0183)

- For conformance for sequence-level independent regions, to explicitly signal the PTL information or to indicate that the level for any sub-picture or sub-picture set can be derived based on scaling of the picture size relative to the sub-picture or sub-picture set sample size, respectively, when not possible, signal PTL in an SEI message?

  - To explicitly signal (O0141)

  - To indicate that the level can be derived, and if not, signal in an SEI message (O0555)

- To signal a sub-picture ID in the slice header?

  - Yes (O0141, O0555)

### Other aspects covered in these contributions

- Signal maximum decoded picture width and height in DPS and decoded picture width and height in PPS, and remove pic_width_in_luma_samples and pic_height_in_luma_samples from SPS? (O0182)

- Update Sub-bitstream extraction process to support sub-picture extraction and reposition across multiple layers (O0183)?

- Provide the possibility to specify partitioning to tiles, bricks, and rectangular slices in SPS, and when the partitioning is specified in SPS, it is not present in the PPS? (O0394)

- Add a new SEI message for nesting of SEI messages for output sub-picture sets? (O0493)

- Use a grid to indicate the sub-picture size and position? (O0555)

## List of contributions

JVET-O0138 AHG12: Treating region boundaries as picture boundaries [Hendry, S. Hong, J. Chen, Y.-K. Wang (Futurewei)]

JVET-O0141 AHG12: Sub-picture based motion-constrained independent regions [Y.-K. Wang, Hendry, J. Chen (Futurewei), R. Skupin, Y. Sanchez, K. Suehring (HHI)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposal 6 in the contribution belongs to this agenda item.

JVET-O0182 AHG12: On picture and sub-picture signaling [Y. He, A. Hamza (InterDigital)]

JVET-O0183 AHG12: On layer-based sub-picture extraction and reposition [Y. He, A. Hamza (InterDigital)]

JVET-O0334 AHG8/AHG12: On sub-picture partitioning support with layers [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-O0394 AHG12: On independently coded picture regions [M. M. Hannuksela (Nokia)]

JVET-O0493 AHG12: On independent regions and output sub-picture sets [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-O0555 AHG12: Sub-pictures and sub-picture sets with level derivation [J. Boyce, L. Xu (Intel)]

JVET-O0700 AHG12: On sub-picture info SEI message [Y. He, A. Hamza (InterDigital)] [late]

# Mixed NAL unit types within a picture (2)

## List of design questions

- To support mixed NAL unit types within a picture or not?

  - Yes (O0140, O0498)

- If yes to 1),

  - Based on a PPS flag that specifies whether all VCL NAL units of a picture have the same NAL unit type (O0140)? Or

  - Base on allowing slices within a picture to have different sets of reference pictures drived from the RPLs and changing the reference marking process such that marking does not happen with the first VCL NAL unit when slices within a picture do have different sets of reference pictures derived from the RPLs (O0498)

- If yes to 1), add the following constraint?

  - For any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type (O0140)

## List of contributions

JVET-O0140 AHG12: On mixed NAL unit types within a picture [Y.-K. Wang, Hendry (Futurewei)]

JVET-O0498 AHG17: On marking process and unequal NALU types per AU [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

# Slice, tile, and brick signalling in PPS and slice header (16)

## List of design questions

### On signalling of rectangular slices in the PPS

- Replace the current signalling with the following: Directly signal the bottom-right brick indices except the last one, which is inferred, and derive all the top-left brick indices? (O0143)

The current signalling is as follows: signal the top-left brick indices except the first one, which is inferred, and deltas for the bottom-right brick indices, and derive the bottom-right brick indices.

- If no to 1),

  - Improve the current singalling by inferring the bottom-right brick index delta of the last slice? (O0156, O0225, O0338, O0403)

  - Replace top_left_brick_idx[ i ] with top_left_brick_idx_minus1[ i ], still u(v) coded? (O0172)

  - Replace top_left_brick_idx[ i ] with top_left_brick_delta[ i ] that is the difference between top_left_brick_idx[ i ] and top_left_brick_idx[ i − 1 ], still u(v) coded with the length Ceil( Log2( NumTilesInPic − TopLeftBrickIdx[ i − 1 ] ) )? (O0172)

  - Apply both b and c above? (O0172)

  - Change the semantics of bottom_right_brick_idx_delta so that it is expressed in units of bricks for the first tile, then in units of tiles? (O0225)

  - Improve the current singalling by inferring the left-top brick index of the last slice? (O0338)

- Add "&&  NumBricksInPic > 2" to the syntax conditions for rect_slice_flag? (O0453)

- Add "&&  NumBricksInPic > 2" to the syntax conditions for num_slices_in_pic_minus1? (O0453)

- Add "&&  num_slices_in_pic_minus1 > 0" to the loop for the syntax elements of rectangular slice addresses? (O0453)

### On signalling of partitioning of a picture into tiles and bricks in the PPS

- Replace syntax element num_brick_rows_minus1 with num_brick_rows_minus2? (O0173, O0176, O0338)

  - If no, change num_brick_rows_minus1[ i ] to num_brick_rows_minus3[ i ]? (O0176)

- Add a loop of brick_column_split_present_flag[ i ] for specifying that at least one tile in any i-th tile column is split into two or more bricks? (O0173)

- Add conditional signalling of the loop of brick_column_split_present_flag[ i ] added by 2)? (O0173)

- Do the following? (O0176)

Move the signalling location of single_brick_per_slice_flag and rect_slice_flag before signalling of brick splitting related syntax elements and to conditionally signal the brick related syntax elements based on rect_slice_flag. This ensures that the information about brick splitting related syntax elements is not signalled for raster scan slices.

Or alternatively, a bitstream conformance constraint is proposed which requires brick related syntax elements to not be signalled for raster scan slices.

- Add single_brick_in_pic_flag to enable one tile with multiple bricks in a picture? (O0338)

- Make the following changes? (O0452)

Change the value range of tile_rows_height_minus1 as follows:

If single_tile_in_pic_flag is equal to 0 and tile_cols_width_minus1 is equal to PicWidthInCtbsY – 1, the value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 2, inclusive. Otherwise, the value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_height_minus1 is inferred to be equal to PicHeightInCtbsY – 1.

And make similar change to the value range of num_tile_rows_minus1 as above.

- Do the follow? (O0452)

Don't signal brick_split_flag[ i ] and brick info for the i-th tile when RowHeight[ i ] is equal to 1, but infer them; when brick_split_flag[ i ] is equal to 1 and RowHeight[ i ] is not greater than 2, don't signal uniform_brick_spacing_flag[ i ] and further brick info, but infer them; and update the value range of num_brick_rows_minus1[ i ] (when present) accordingly.

- Skipping tile row related syntax and directly indicating bricks per tile column; infer that a chain of brick boundaries crossing the entire picture forms a tile boundary (aspect 1 of O0390). See also Section 5.

- A single signalling mode for tile-and-brick partitioning instead of separate uniform and explicit spacing modes (aspect 2 of O0390)

### On signalling of slice IDs in the PPS

- Replace signalled_slice_id_length_minus1 with an offset (i.e., something like signalled_slice_id_length_minus_offset), or alternatively just change the value range of signalled_slice_id_length_minus1? (O0176)

- Change the list of u(v)-coded slice_id[ i ], except slice_id[ 0 ], to be delta and ue(v)-coded? (O0199)

- Move the signalling of slice IDs inside the syntax condition for single_tile_in_pic_flag? (O0269)

- Based on 3), signal the slice ID and the slice location only when single_brick_per_slice_flag is equal to 0, and remove 'i > 0' in the syntax condition for slice position signalling to allow signalling of the first rectangular slice that may start at any places in the picture? (O0269)

- Based on 4), signal the brick splitting, the slice ID and the slice location only when single_brick_per_slice_flag is equal to 0? (O0269)

- Relax the constraint "The slices of a picture shall be in increasing order of their slice_address values" to "If signalled_slice_id_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values"? (O0338)

- Add "&& num_slices_in_pic_minus1 > 0" to the syntax condition for signalled_slice_id_flag? (O0454)

### Other aspects

- Relax the constraint "The slices of a picture shall be in increasing order of their slice_address values" to "If signalled_slice_id_flag is equal to 1, the slices of a picture shall be in increasing order of their slice_address values"? (O0176)

- Do the following? (O0180)

Change the inference "When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0." to "When single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0."

- To change the slice order constraint to be "The slices of a picture shall be in increasing order of brick IDs." (O0199)

- Signal sh_slice_id instead of slice_address when the rect_slice_flag is equal to 1? (O0338)

  - Option 2: Remove the constraint "The slices of a picture shall be in increasing order of their slice_address values."?

  - Option 3: Add the following constraint: "The slices of a picture shall be in increasing order of their slice_id values."?

- Add "&&  NumBricksInPic > 2" to the syntax conditions for num_bricks_in_slice_minus1 in the slice header? (O0453)

- Change "| |  NumBricksInPic > 1" to "&&  num_slices_in_pic_minus1 > 0" in the syntax condition for slice_address? (O0454)

- Add a derivation process change to update the value of NumBricksInPic for an extracted sub-bitstream? (O0454)

- Add a new PPS syntax element max_slice_id_diff? (O0454)

- Add first_slice_in_pic_flag at the beginning of the slice header, unconditioned? (O0610)

## List of contributions

JVET-O0143 AHG12: Signalling of addresses for rectangular slices [R. Sjöberg, M. Damghanian, M. Pettersson, J. Ström (Ericsson), Hendry, Y.-K. Wang (Futurewei)]

JVET-O0156 AHG12: On brick signaling [W. Lim, G. Bang, H. Y. Kim (ETRI)]

JVET-O0172 On rectangular slice address signaling [B.-K. Lee (Xris)]

JVET-O0173 On brick information signaling [B.-K. Lee (Xris)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposals 1 to 4 in the contribution belongs to this agenda item.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 3 in the contribution belongs to this agenda item.

JVET-O0199 AHG12: On rectangular slices [M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0225 AHG17: On signalling of bottom right brick for rectangular slices [V. Drugeon, T. Nishi (Panasonic)]

JVET-O0269 AHG17: Suggested syntax improvement on PPS [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0338 AHG12: On signaling for slice, tile and brick [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0390 AHG17: On tile and brick signalling [M. M. Hannuksela (Nokia)]

JVET-O0403 AHG17: On brick index signalling [P. Bordes, M. Kerdranvat (InterDigital)]

JVET-O0452 AHG12: On tile and brick partition [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0453 AHG12: On rectangular slice signaling [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0454 AHG12: On sub-bitstream extraction [Y.-J. Chang, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0610 AHG17: Association of VCL NAL units to coded pictures [B. Heng, W. Wan (Broadcom)]

# Signalling of brick/WPP entries (4)

## Summary

These contributions can basically be discussed one by one, although one item in O0145 and one item in O0215 are for the same issue.

## List of contributions

JVET-O0144 AHG12: On end_of_subset_one_bit, end_of_brick_one_bit, and byte alignment in the slice data syntax [Hendry, Y.-K. Wang (Futurewei)]

JVET-O0145 AHG12: Miscellaneous AHG12 topics [Y.-K. Wang, Hendry (Futurewei)]

JVET-O0215 AHG12: Clean up on wavefront parallel processing signaling [T. Ikai, E. Sasaki, T. Chujoh (Sharp)]

JVET-O0306 AHG17: Entry point offsets without start code emulation [K. Abe, T. Toma, V. Drugeon (Panasonic)]

# New tile/brick/WPP partitioning schemes (5)

## Summary and design questions

The contributions O0198, aspect 1 of O0390, and O0511 are overlapping, thus should be discussed and decided together, probably based on the following design questions:

- Add a scheme to enable portioning of a tile into a column of bricks? (O0198)

- If no to 1), change the tile column and row partitioning plus brick row partitioning to tile column partitioning plus brick row partitioning (i.e., remove tile row partitioning)? (aspect 1 of O0390, O0511)

- If yes to 2), which design to choose, aspect 1 of O0390 or O0511?

The other two contributions can be discussed one by one.

## List of contributions

JVET-O0198 AHG12: On brick partitioning [M. Coban, A. Ramasubramonian, Y.-J. Chan, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0358 AHG12: On uniform tile spacing [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

JVET-O0390 AHG17: On tile and brick signalling [M. M. Hannuksela (Nokia)]

JVET-O0511 AHG12: On tiles and bricks partitioning [N. Ouedraogo, E. Nassor (Canon)]

JVET-O0462 AHG12: Separated Luma and Chroma wavefront for dual tree [T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

# Loop filtering across boundaries of independently coded regions (4)

## Summary and design questions

For this category of contributions, I think the HLS group should probably just discuss and decide whether turning of in-loop filtering across slice boundaries is needed in case a sub-picture design allowing multiple slices within a sub-picture is adopted, while the rest on actual filtering operations should left for the appropriate low-level group to discuss and decide.

- Is turning of in-loop filtering across slice boundaries needed in case a sub-picture design allowing multiple slices within a sub-picture is adopted?

## List of contributions

JVET-O0142 AHG12: On turning off ALF filtering at brick and slice boundaries [Y.-K. Wang, J. Chen, Hendry (Futurewei)]

Also included in the agenda item 6.5.

JVET-O0494 AHG12: On filtering of independently coded region [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Also included in the agenda item 6.5.

JVET-O0654 AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Also included in the agenda item 6.5.

JVET-O0662 Non-CE5: Modified ALF filtering for Slice, Brick and Virtual boundaries [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

Also included in the agenda item 6.5.