| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-OO0804-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0190 (Non-CE4: LIC model parameters using forward mapped luma samples in LMCS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0190 proposed by Ittiam. JVET-O0190 proposed modifications in local illumination compensation (LIC) model derivation process retains the gains of LIC whilst replacing the inverse mapping (invLUT) of neighbour reconstruction samples with forward mapping (fwdLUT) of reference inter prediction samples. JVET-O0190 provides one result that combines their proposed method with CE4-3.1a. However, only partial results are verified in this document, and the the BD-rate results match exactly with those in JVET-O0190.

# Verification

Ittiam provided the source code of their proposed method described in JVET-O0190 [2]. One verification test was carried out.

The test is conducted under the common test conditions (CTC) defined in [3] using the VTM5.0 software [4]. Table 1 shows the results of using VTM5.0 as the anchor and enabling the proposed method. The BD-rates from our experimental results match exactly those in JVET-O0190. Our runtime comparison may be not accurate.

Table 1. The result of JVET-O0190 using VTM5.0 as anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.43% | -0.45% | 131% | 101% |
| Class A2 | -0.39% | -0.31% | -0.43% | 115% | 102% |
| Class B | -0.47% | -0.23% | -0.36% | 119% | 100% |
| Class C | -0.37% | -0.47% | -0.62% | 118% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.35% | -0.46% | 120% | 99% |
| Class D | -0.13% | -0.02% | -0.04% | 112% | 98% |
| Class F | -0.67% | -0.39% | -0.47% | 112% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.42% | -0.64% | -0.47% | 128% | 104% |
| Class E | 0.00% | 0.03% | -0.40% | 104% | 104% |
| Overall |  |  |  |  |  |
| Class D | -0.17% | -0.17% | -0.04% | 117% | 100% |
| Class F | -1.75% | -1.58% | -1.62% | 119% | 99% |

# References

[1] V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, H. Chen, X. Ma, and H. Yang, “CE4-3.1a and CE4-3.1b: Unidirectional local illumination compensation with affine prediction,” Document of Joint Video Experts Team, JVET-O0066.

[2] J. Shingala, and A. Natesan, “Non-CE4: LIC model parameters using forward mapped luma samples in LMCS,” Document of Joint Video Experts Team, JVET-O0190.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[4] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).