| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0840-v1 |
| --- | --- |

| Title: | Cross-chreck of JVET-O0327 (CE8-related: Default filling method for IBC Mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Chen | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of default filling method of IBC mode proposed in JVET-O0327. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

JVET-O0327 proposes a default filling method for IBC mode in order to replace invalid zero block vectors. Two methods are provided in JVET-O0327. In the first method, if the merge list is still not complete, the default block vector of (-2*cbWidth, 0) is used to fill the merge list, where, cbWidth is the coding block width in luma samples, and block vector in integer pixel accuracy. In the second method, The default block vectors are added in the order of: (-2*cbWidth, 0), (0, -cbHeight), (-4*cbWidth, 0), (0, -2*cbHeight), (-8*cbWidth, 0), (0, -4*cbHeight), where cbWidth and cbHeight are the coding block width and height in luma samples, respectively.

Two tests are conducted in this contribution. It is reportedly shown the coding results are identical with those reported in JVET-O0327.

Experimental results

Experiments were performed based on VTM 4.0 software under JVET common test condition.

1）Method 1

|  | AI Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | -0.01% | 149% | 102% |
| Class A2 | -0.78% | -1.05% | -0.98% | 152% | 102% |
| Class B | -0.78% | -0.71% | -0.83% | 162% | 100% |
| Class C | -0.29% | -0.28% | -0.25% | 160% | 106% |
| Class E | -1.30% | -1.18% | -1.35% | 171% | 99% |
| Overall | -0.63% | -0.63% | -0.67% | 159% | 102% |
| Class D | -0.52% | -0.49% | -0.57% | 151% | 102% |
| Class F | -0.04% | 0.03% | -0.02% | 106% | 101% |
| Class SCC 1080p | -0.05% | -0.03% | -0.03% | 100% | 100% |

|  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.05% | 0.11% | 111% | 95% |
| Class A2 | -0.06% | -0.13% | -0.01% | 110% | 94% |
| Class B | -0.15% | -0.25% | -0.66% | 112% | 94% |
| Class C | 0.10% | -0.35% | -0.11% | 122% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.21% | -0.23% | 114% | 95% |
| Class D | 0.12% | -0.36% | 0.06% | 121% | 104% |
| Class F | -0.09% | -0.17% | 0.11% | 105% | 91% |
| Class SCC 1080p | -0.06% | -0.03% | -0.08% | 105% | 106% |

|  | LD Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.31% | 0.18% | 132% | 96% |
| Class C | 0.14% | 0.29% | 0.06% | 142% | 100% |
| Class E | 0.14% | 0.35% | 0.11% | 128% | 100% |
| Overall | 0.08% | 0.31% | 0.12% | 134% | 98% |
| Class D | 0.07% | -0.27% | 0.76% | 140% | 101% |
| Class F | -0.17% | -0.14% | -0.56% | 112% | 99% |
| Class SCC 1080p | -0.11% | -0.17% | -0.12% | 107% | 104% |

2) Method 2:

|  | AI Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.06% | -0.04% | 142% | 101% |
| Class A2 | -0.80% | -1.01% | -0.99% | 146% | 104% |
| Class B | -0.80% | -0.68% | -0.86% | 158% | 100% |
| Class C | -0.31% | -0.27% | -0.24% | 156% | 103% |
| Class E | -1.30% | -1.27% | -1.28% | 164% | 100% |
| Overall | -0.65% | -0.62% | -0.68% | 154% | 101% |
| Class D | -0.58% | -0.49% | -0.64% | 148% | 96% |
| Class F | -0.07% | 0.05% | -0.03% | 104% | 101% |
| Class SCC 1080p | -0.09% | -0.05% | -0.05% | 101% | 102% |

|  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.17% | 0.07% | 107% | 90% |
| Class A2 | -0.08% | -0.21% | -0.12% | 108% | 101% |
| Class B | -0.16% | -0.30% | -0.71% | 109% | 93% |
| Class C | 0.10% | -0.21% | -0.24% | 117% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.23% | -0.31% | 111% | 95% |
| Class D | 0.00% | -0.29% | -0.04% | 117% | 105% |
| Class F | -0.13% | -0.16% | -0.13% | 102% | 92% |
| Class SCC 1080p | -0.15% | -0.21% | -0.27% | 104% | 105% |

|  | LD Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.02% | -0.21% | 124% | 96% |
| Class C | 0.08% | 0.16% | -0.12% | 134% | 99% |
| Class E | -0.01% | 0.38% | 0.28% | 122% | 100% |
| Overall | 0.02% | 0.16% | -0.06% | 127% | 98% |
| Class D | 0.12% | -0.25% | -0.16% | 134% | 100% |
| Class F | -0.22% | -0.40% | -0.53% | 105% | 104% |
| Class SCC 1080p | -0.17% | -0.30% | -0.27% | 105% | 101% |