| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0848 |
| --- | --- |

| Title: | Non CE3: Modification of MIP interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bing-Jung Fuh, Chang-Hao Yau1, Ching-Chieh Lin2, Chi-Chang Kuo and Chun-Lung Lin3 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Tel: Email: | +886-3-5914490 changhao.yau@itri.com1 JackLin@itri.com2 chunlung@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This contribution proposed the modified interpolation of MIP. The 1D linear interpolation is replaced by a 2D linear interpolation by an interpolation unit to improve the flexibility for the hardware and reduce the interpolation process latency. The results of the proposed method show 0.00%(Y), -0.01%(U) and -0.01%(V) coding gain.

Introduction

The Matrix-based Intra Prediction (MIP) [1] was adopted in the last meeting. There are three stages in the current MIP. The first stage is the downsampling of the reference samples from the boundary to prepare reference samples for the second stage and the third stage. The second stage is the generation of the reduced prediction samples by matrix-vector multiplication. The third stage is the upsampling by using the 1D linear interpolation to generate the final prediction.

For a 64x64 block, the interpolation process can be divided into the first interpolation (vertical) and the second interpolation (horizontal). The first interpolation uses the top boundary averaged samples and the reduced predicted samples as interpolation reference samples. The second interpolation uses the left neighbouring reconstructed samples and the results of the first interpolation as interpolation reference samples. The top boundary averaged samples are the average of the top neighbouring reconstructed samples. The reduced predicted samples are the results of matrix-vector multiplication as shown in Figure 1.

Figure 1: An example of interpolation for the 64x64 block. (a) the first interpolation (b) the second interpolation

In the current design, the second interpolation needs to wait the results of the first interpolation. This contribution proposed to reduce the restrictions on the interpolation order to provide the flexibility for the parallel interpolation processing.

Proposed method

In order to parallelize the interpolation processing, the interpolation reference samples don’t use the results of the first interpolation. For an example of the 64x64 block, which is the worst case for MIP interpolation, the current coding block is divided into multiple 8x8 interpolation units, as shown in Figure 2. When the interpolation unit does not locate at the boundary of the CU, only four reduced prediction samples are used as the interpolation reference samples. For the unification reason, the interpolation unit uses the subsampled left neighbouring reconstructed samples. When the interpolation unit is at the top-left of the CU, the top-left interpolation reference sample is the first sample of the left neighbouring reconstructed samples. Therefore, there is no dependency between interpolation units.

Figure 2: An example of the interpolation units for a 64x64 block.

In Figure 3, the interpolation reference samples A, B, C, and D are the top-left, top-right, bottom-left, and bottom-right samples of the interpolation unit respectively. Considering a W×H block with reduced prediction samples Wred× Hred, the definition of the up-sampling factor is upVer =H/Hred for vertical and upHor =W/Wred for horizontal. The linear interpolated prediction sample is generated by:

P = upHor-dXupVer-dYA+dXupVer-dYB+ upHor-dX*dY*C+dX*dY*D+upHor *upVer2 upHor *upVer

where upVer is the width of the interpolation unit, upHor is the height of the interpolation unit, dX is the horizontal distance from the top-left sample of the interpolation unit to P, dY is the vertical distance from the top-left sample of the interpolation unit to P.

Figure 3: An example of an 8x8 interpolation unit.

Experimental results

The proposed modifications were implemented on VTM5.0. According to the CE3 description in JVET-N1023 [2] and the common test conditions in JVET-N1010 [3], the proposed method was evaluated under intra-only (AI) and random-access (RA) configuration.

Table 1 and table 2 show the experimental results for AI and RA respectively. The AI results show 0.00% (Y), -0.01% (U) and -0.01% (V) gain in terms of average BD-rate with 101% and 100% encoding and decoding time. The RA results show 0.00% (Y), -0.02% (U) and -0.08% (V) gain in terms of average BD-rate with 100% and 100% encoding and decoding time.

Table 1. Results of all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | -0.05% | 101% | 100% |
| Class B | 0.01% | -0.05% | 0.01% | 101% | 100% |
| Class C | 0.01% | -0.03% | 0.03% | 100% | 100% |
| Class E | -0.01% | 0.02% | -0.04% | 101% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class D | 0.01% | 0.17% | 0.02% | 101% | 101% |
| Class F (optional) | 0.02% | -0.07% | 0.02% | 100% | 100% |

Table 2. Results of random access (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.06% | 100% | 100% |
| Class A2 | 0.04% | 0.10% | 0.05% | 100% | 100% |
| Class B | 0.02% | -0.01% | -0.15% | 100% | 100% |
| Class C | -0.05% | -0.06% | -0.18% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.08% | 100% | 100% |
| Class D | 0.02% | -0.23% | 0.03% | 101% | 99% |
| Class F (optional) | 0.00% | -0.06% | 0.12% | 100% | 100% |

Conclusion

In this contribution, the 2D interpolation for MIP is proposed to enhance the flexibility and reduce the latency. The coding unit is divided into several independent interpolation units. The four reference samples are used to generate the final prediction for the interpolation unit. The change in the coding efficiency and the run-time for AI and RA configurations is negligible. It is recommended to further study in a CE.

References

- Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Philipp Helle, Tobias Hinz, Heiko Schwarz, Detlev Marpe, Thomas Wiegand, “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, March 2019.

- Geert Van der Auwera, L. Li, Alexey Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-N1023, March 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).