| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0854 |
| --- | --- |

| Title: | Crosscheck of JVET-O0554 (Removal of infrequently used context models) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-O0554, where 100 least frequently used context models in VVC are proposed to be removed. The removed contexts are replaced by bypass (Method #1) or a proposed generalized bypass (Method #2). The simulation results reportedly match those provided by the proponents.

# Verification

In JVET-O0554 [1], a context model removal method is proposed, in which 100/90/80 context models are proposed to be removed and replaced by bypass or a proposed generalized bypass method. This cross-check report verifys the removal of 100 context models, in which Method #1 is to replace the 100 removed context models with bypass, and Method #2 is to replace the 100 removed context models with the proposed generalized bypass. We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

CTC testing [2] is performed with the provided source code based on VTM-5.0 [3]. The coding results of method #1 and method #2 compared to VTM-5.0 anchor are summarized in Table 1 and Table 2, respectively.

The BD rate differences match with the results from the proposal [1]. The run times are basically in accordance with that from the proposal. Note that timing results are not well comparable due to differences in the simulation environment.

Table 1. Results of Method #1 compared to VTM-5.0 anchor

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.11% | 0.11% | 101% | 101% |
| Class A2 | 0.12% | 0.12% | 0.12% | 101% | 100% |
| Class B | 0.07% | 0.07% | 0.07% | 102% | 101% |
| Class C | 0.08% | 0.08% | 0.08% | 101% | 103% |
| Class E | 0.09% | 0.08% | 0.08% | 101% | 102% |
| Overall | 0.09% | 0.09% | 0.09% | 101% | 101% |
| Class D | 0.10% | 0.09% | 0.09% | 103% | 104% |
| Class F | 0.16% | 0.15% | 0.15% | 101% | 105% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.54% | 0.50% | 0.50% | 102% | 102% |
| Class A2 | 0.44% | 0.40% | 0.40% | 101% | 101% |
| Class B | 0.26% | 0.25% | 0.26% | 101% | 101% |
| Class C | 0.12% | 0.12% | 0.12% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.32% | 0.29% | 0.30% | 101% | 101% |
| Class D | 0.11% | 0.10% | 0.10% | 100% | 100% |
| Class F | 0.28% | 0.28% | 0.27% | 101% | 101% |

Table 2. Results of Method #2 compared to VTM-5.0 anchor

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.07% | 0.07% | 102% | 101% |
| Class A2 | 0.05% | 0.05% | 0.05% | 100% | 100% |
| Class B | 0.03% | 0.03% | 0.03% | 101% | 101% |
| Class C | 0.02% | 0.02% | 0.02% | 100% | 100% |
| Class E | 0.03% | 0.02% | 0.03% | 100% | 101% |
| Overall | 0.04% | 0.04% | 0.04% | 101% | 101% |
| Class D | 0.02% | 0.02% | 0.02% | 102% | 100% |
| Class F | 0.07% | 0.07% | 0.07% | 101% | 101% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.36% | 0.34% | 0.35% | 101% | 101% |
| Class A2 | 0.27% | 0.25% | 0.26% | 101% | 101% |
| Class B | 0.15% | 0.14% | 0.15% | 101% | 101% |
| Class C | 0.05% | 0.05% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.18% | 0.18% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.03% | 100% | 100% |
| Class F | 0.15% | 0.15% | 0.14% | 101% | 102% |

# References

A. Alshin, J. Boyce, Pavel Frolov, Vasily Aristarkhov, “Removal of infrequently used context models” , JVET-O0554, July 2019, Gothenburg, SE.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.