| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0621 |
| --- | --- |

| Title: | Crosscheck of JVET-O0621 (Non-CE3: MIP simplification) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-O0621, where a reduced storage of matrix sets for MIP is proposed. The simulation results reportedly match those provided by the proponents.

# Verification

In the JVET-O0621 proposal [1], the stored matrix table sizes for MIP sizeId0 and sizeId2 sre reduced, where the storage of matrix table for sizeId0 is reduced from 18 to 9, and the storage of matrix table for sizeId2 is reduced from 6 to 3. The other half of table are derived using linear interpolation of the tables of neighboring modes. We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

CTC testing [2] is performed with the provided source code based on VTM-5.0 [3]. The coding results of the proposed method compared to VTM-5.0 anchor are summarized in Table 1.

The BD rate differences match with the results from the proposal [1]. The run times are basically in accordance with that from the proposal. Note that timing results are not well comparable due to differences in the simulation environment.

Table 1. Results of the proposed method compared to VTM-5.0 anchor

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | -0.01% | 101% | 101% |
| Class A2 | 0.03% | 0.08% | 0.00% | 101% | 101% |
| Class B | 0.04% | -0.03% | 0.10% | 101% | 102% |
| Class C | 0.04% | 0.05% | 0.11% | 100% | 102% |
| Class E | 0.02% | -0.03% | -0.19% | 101% | 100% |
| Overall | 0.04% | 0.02% | 0.02% | 101% | 101% |
| Class D | 0.07% | 0.31% | 0.07% | 102% | 104% |
| Class F | 0.02% | 0.11% | 0.14% | 101% | 103% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.30% | 0.08% | 101% | 101% |
| Class A2 | 0.03% | -0.04% | -0.01% | 102% | 102% |
| Class B | 0.04% | 0.01% | -0.27% | 101% | 101% |
| Class C | 0.01% | -0.06% | -0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.08% | -0.08% | 101% | 101% |
| Class D | 0.06% | -0.23% | 0.10% | 101% | 101% |
| Class F | 0.04% | 0.02% | 0.12% | 101% | 102% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.23% | -0.28% | 101% | 103% |
| Class C | 0.06% | 0.12% | -0.04% | 101% | 107% |
| Class E | 0.16% | -0.34% | 0.18% | 101% | 103% |
| Overall | 0.07% | 0.05% | -0.09% | 101% | 104% |
| Class D | 0.02% | 0.11% | 0.59% | 101% | 102% |
| Class F | 0.03% | 0.72% | -0.49% | 103% | 103% |

# References

Y. Yasugi, T. Ikai, “Non-CE3: MIP simplification” , JVET-O0621, July 2019, Gothenburg, SE.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.