| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0861 |
| --- | --- |

| Title: | Crosscheck of JVET-O0304 (Non-CE9: Simplification of BDOF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck of a Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross-check of JVET-O0304. The results provided by the proponents were verified, and the implementation in the reference software was inspected as well.

# Introduction

JVET-O0304 proposes simplification to BDOF by replacing multiplication with sign based multiplication during flow vector estimation. It also proposes to modify the current single right-shift using the most significant bit position of the denominator based division for computing vx and vy with a method consisting of MSB bit position extraction and look up (CCLM like division). JVET-O0304 without LUT based division was also simulated. Below tables summarize results for JVET-O0304 and JVET-O0304 without LUT.

# Simulation Results

The crosscheck results of JVET-O0304 under common test conditions are summarized below.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.01% | 99% | 100% |
| Class A2 | 0.03% | -0.04% | -0.05% | 98% | 100% |
| Class B | 0.02% | 0.00% | -0.03% | 98% | 100% |
| Class C | 0.01% | -0.02% | -0.11% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.05% | 98% | 100% |
| Class D | 0.09% | 0.06% | 0.03% | 98% | 99% |
| Class F | 0.00% | -0.02% | -0.03% | 97% | 100% |

Table 1: Results under CTC – JVET-O0304.

Simulation results without LUT table for proposed method is shown below

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.14% | 0.02% | 98% | 100% |
| Class A2 | 0.05% | 0.02% | 0.02% | 98% | 99% |
| Class B | 0.05% | 0.09% | 0.01% | 98% | 99% |
| Class C | 0.02% | 0.05% | -0.01% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.02% | 0.01% | 98% | 99% |
| Class D | 0.06% | -0.02% | 0.09% | 98% | 98% |
| Class F | 0.01% | 0.01% | 0.02% | 97% | 100% |

Table 2: Results under CTC JVET-O0304 without LUT.

# Conclusion

It is confirmed that the cross-check results are identical with the proponent’s results. It is important to note that second item of LUT-based division can be applied independently on VTM5.0 BDOF also.

# Reference

- Yusuke Kato, Kiyofumi Abe, Tadamasa Toma, “JVET-O0304 Non-CE9: Simplification of BDOF”, 15th meeting, Gothenburg, SE, July 2019.