| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0882-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0168 (Non-CE3: MIP low resolution clipping) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0168 proposed by Qualcomm. JVET-O0168 proposes to reduce the number of required clipping operations for a Matrix-based Intra Prediction (MIP) coded block. The proposed method proposes to do the sample value clipping on MIP reduced prediction prior to the upsampling.

The proposed method in JVET-O0168 is same as the method proposed in JVET-O0160 by Ericsson.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The simulation results in JVET-O0168 on top of VTM5.0 matched exactly the simulation results provided in JVET-O0170.

The verification simulation was run for AI and RA configuration on top of 8-bit MIP implementation which was proposed in JVET-O0084 by HHI. The verification simulation results matched the results in JVET-O0168.

# Verification

Qualcomm had provided the results of their proposed methods in JVET-O0168[1]. The anchor software was provided by HHI in JVET-O0084[2]. The code was updated according to the description in JVET-O0160[3] and JVET-O0168.

The simulation was conducted under the common test conditions (CTC). The anchor software was 8-bit MIP implementation in JVET-O0084. The verification simulation results matched the results in JVET-O0168.

Simulation results of on top of 8-bit MIP implementation compared to JVET-O0084:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,07% | 0,07% | 100% | 100% |
| Class A2 | 0,00% | -0,01% | 0,00% | 100% | 99% |
| Class B | 0,00% | 0,00% | -0,02% | 100% | 100% |
| Class C | 0,01% | 0,02% | -0,05% | 100% | 98% |
| Class E | 0,00% | -0,02% | 0,00% | 99% | 99% |
| Overall | 0,00% | 0,01% | 0,00% | 100% | 99% |
| Class D | 0,00% | 0,03% | -0,02% | 100% | 99% |
| Class F | 0,01% | 0,02% | 0,01% | 99% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,17% | -0,09% | 101% | 101% |
| Class A2 | 0,04% | -0,09% | 0,00% | 100% | 101% |
| Class B | 0,01% | -0,02% | -0,07% | 100% | 101% |
| Class C | -0,01% | -0,07% | -0,04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | -0,01% | -0,05% | 100% | 100% |
| Class D | -0,01% | 0,01% | 0,02% | 100% | 100% |
| Class F | 0,01% | 0,17% | 0,10% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | 0,19% | 0,15% | 101% | 102% |
| Class C | -0,01% | -0,03% | -0,19% | 101% | 101% |
| Class E | 0,02% | 0,18% | -0,27% | 100% | 102% |
| Overall | 0,00% | 0,12% | -0,07% | 101% | 102% |
| Class D | 0,07% | 0,15% | 0,58% | 100% | 100% |
| Class F | 0,05% | -0,08% | 0,27% | 99% | 97% |

# References

[1]. JVET-O0168: Non-CE3: MIP low resolution clipping

[2]. JVET-O0084: 8-bit implementation and simplification of MIP

[3]. JVET-O0160: Non-CE3: Sample value clipping on MIP reduced prediction

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).