| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0884-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0213 (Non-CE6: Block size restriction of LFNST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0213 proposed by LG Electronics Inc. JVET-O0213 proposed two restrictions on block size of Low Frequency Non-Separable Transform (LFNST). The first proposed change was to restrict the maximum size that LFNST is allowed to be 64 × 64. The second proposed change was to signal the LFNST index in TU-level which guarantees that LFNST index derivation must be performed within maximum size TU.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for AI and RA configuration for change 1 and change 2 respectively. The verification simulation results matched the results in JVET-O0213.

# Verification

LG Electronics Inc had provided the source code of their proposed methods in JVET-O0213[1]. The update of the code had been reviewed and confirmed consistent with the proposal description.

The simulation was conducted under the common test conditions (CTC). The anchor software was VTM5.0. Since the current CTC configuration does not have a LFNST block with a block size larger than 64 × 64, the proposed changes do not impact the coding performance compared to VTM5.0.

Simulation results of proposed change 1 (maximum allowed LFNST size 64 × 64):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 102% |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 102% |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 100% |
| Class C | 0,00% | 0,00% | 0,00% | 105% | 104% |
| Class E | 0,00% | 0,00% | 0,00% | 97% | 103% |
| Overall | 0,00% | 0,00% | 0,00% | 101% | 102% |
| Class D | 0,00% | 0,00% | 0,00% | 97% | 100% |
| Class F | 0,00% | 0,00% | 0,00% | 102% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 101% | 97% |
| Class A2 | 0,00% | 0,00% | 0,00% | 101% | 98% |
| Class B | 0,00% | 0,00% | 0,00% | 101% | 99% |
| Class C | 0,00% | 0,00% | 0,00% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,00% | 0,00% | 101% | 99% |
| Class D | 0,00% | 0,00% | 0,00% | 101% | 101% |
| Class F | 0,00% | 0,00% | 0,00% | 107% | 98% |

Simulation results of proposed change 2 (signal LFNST index in TU-level):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 101% | 101% |
| Class A2 | 0,00% | 0,00% | 0,00% | 99% | 99% |
| Class B | 0,00% | 0,00% | 0,00% | 97% | 97% |
| Class C | 0,00% | 0,00% | 0,00% | 105% | 103% |
| Class E | 0,00% | 0,00% | 0,00% | 97% | 99% |
| Overall | 0,00% | 0,00% | 0,00% | 100% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 99% | 101% |
| Class F | 0,00% | 0,00% | 0,00% | 99% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 99% | 98% |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 100% |
| Class B | 0,00% | 0,00% | 0,00% | 99% | 95% |
| Class C | 0,00% | 0,00% | 0,00% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,00% | 0,00% | 99% | 96% |
| Class D | 0,00% | 0,00% | 0,00% | 99% | 96% |
| Class F | 0,00% | 0,00% | 0,00% | 104% | 90% |

# References

[1]. JVET-O0213: Non-CE6: Block size restriction of LFNST

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).