| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0894 |
| --- | --- |

| Title: | Crosscheck of JVET-O0499 (Parameter alignment for pipelined ALF processing) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-O0499. It is confirmed that the cross-checked results are identical with the proponent’s results except the simulation time is unreliable.

# Introduction

Brief technical description of JVET-O0499.

In the last meeting in Geneva, the concept of ALF virtual boundaries was adopted. It introduces a sample dependency boundary 4 sample lines above the bottom of the CTU, so that the first N-4 sample lines can be processed, while the filtering of the last four lines is delayed until the next CTU line is processed (as the content might be changed during the application of deblocking filter in the following CTU line). This concept encourages the implementation of ALF working between the virtual boundaries. With the current implementation, this introduces a parameter dependency, which requires an additional parameter line buffer. In the current implementation, processing of an ALF block requires application of two separate parameter sets (from previous and current line). It is proposed to move the scope of ALF per-CTU parameters from between CTU boundaries to between ALF virtual boundaries.

# Simulation results

The simulation is conducted using VTM-5.0 with the test condition described in JVET-N1010.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 142% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 137% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 131% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 122% |
| Class E | 0.00% | -0.02% | 0.01% | 100% | 139% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 133% |
| Class D | 0.00% | 0.01% | 0.01% | 100% | 120% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 130% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | -0.01% | 100% | 146% |
| Class A2 | 0.00% | -0.01% | 0.03% | 100% | 143% |
| Class B | -0.01% | 0.01% | -0.04% | 101% | 138% |
| Class C | -0.02% | -0.03% | -0.06% | 100% | 134% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.03% | 100% | 139% |
| Class D | 0.00% | -0.04% | 0.08% | 99% | 131% |
| Class F | 0.02% | 0.04% | 0.03% | 100% | 127% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.27% | 0.05% | 99% | 136% |
| Class C | 0.05% | 0.14% | -0.22% | 101% | 137% |
| Class E | 0.06% | 0.19% | -0.47% | 100% | 130% |
| Overall | 0.03% | 0.21% | -0.17% | 100% | 135% |
| Class D | -0.03% | 0.24% | 0.61% | 100% | 136% |
| Class F | 0.00% | -0.01% | -0.08% | 100% | 134% |

# Conclusion

It is confirmed that the cross-checked results are identical with the proponent’s results except the running time is not reliable in the cross-checking.