| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0898 |
| --- | --- |

| Title: | Extra results to JVET-N605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC” |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rémi Jullian, Ya Chen, Franck Galpin, Edouard François, Michel Kerdranvat InterDigital R&D France 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 33 95 first-name.last-name@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents extra results complementing JVET-N0605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC” by using the latest versions of the reference codecs, on full length sequences (in JVET-N0605 only one second was encoded), with closer test conditions between AV1 and the others.

The experiments were conducted according to the common test condition (CTC) of JVET [2] in Random access 10 bits but also in a variant with only one intra picture at the first frame of the sequence. Specific investigations were carried out to ensure comparable configuration between AV1 and the others.

The performance according the average BD-rate PSNR variations of VTM-5.0 (VVC following March 2019 JVET meeting), AV1, and ETM 2.01 (EVC after March 2019 MPEG meeting) versus HM-16.18 are respectively -31.4%, -20.1%, and -17.4% with one intra only, and -31.7%, -9.2%, and -16.7% with one second intra period . Reported figures show that the AV1 software is optimized for the streaming use case (-20.1% versus HEVC), with a two-pass encoding process, while VTM5.0 and ETM2.0.1 use one single pass. For use case closer to broadcast using regular random-access points, AV1 performance is much lower (-9.2% versus HEVC). Performance difference between VTM5.0 and EVC2.0.1 is around 14-15% in both scenarios.

# Introduction

This contribution presents extra results complementing JVET-N0605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC”. The testing conditions between AV1 and the other codecs have been better aligned. Two test conditions are used: random access as described in JVET CTC [2], and a variant with only one intra picture at the first frame of the sequence. After a description of the test conditions and AV1 software configuration, the relative performance (BD-rate, encoder and decoder run times variations) between these codecs are provided in the two test conditions.

# Test conditions

The test conditions are based on the JVET CTC [2] with two settings:

- Setting1: random access with one second intra period,

- Setting2: one intra picture only at the first frame of the sequence.

The reason of using Setting2 is that significantly better AV1 performance are observed with this setting. Forcing the GOP structure is not well handled by the AV1 encoder. On the other hand, VTM and ETM encoders can be easily configured to support Setting2.

The following software versions were used:

- HM-16.18,

- VTM-5.0,

- ETM2.01 (EVC) from May 14th, 2019,

- AV1 commit cb43f76 from May 19th, 2019.

In JVET-N605, HM, VTM and ETM were configured in Setting1, while AV1 was configured in Setting2.

For a fairer comparison, in this contribution, two sets of results are provided for the three codecs, using Setting1, and using Setting2. Setting1 gives more advantage to VTM and EVC as AV1, Setting2 on the contrary favors more AV1 which is optimized for this setting.

# AV1 configuration

## AV1 two passes encoding

AOM recommends running two passes of encoding to reach the best performance because the first pass computes important statistics on the full sequence to guide the second pass. Another reason could be that the one pass configuration is not maintained because not corresponding to AOM use case. A test was made on the Class D sequences; the gain is 20.60% with two passes but only 7.04% with one pass. One should notice that GOP structure when using 1 pass encoding is very different from the hierarchical GOP structure of the VTM. The encoder/decoder running time versus HM-16.18 for two-passes is 739% / 209%, and for one pass 676% / 193%. Two passes clearly provide a look ahead on the encoding parameters the other coding solutions have not. Very likely, this look ahead is not providing a noticeable advantage to one-pass only because the video clips are short (10 seconds) and stable in their content. So, two pass encoding is used.

## AV1 Quantization scheme

Further investigations on configuration parameters were conducted to ensure that the bits allocation was like the VTM, HM, ETM encoding software. Indeed, the quantization function is not the same in AV1 as in VTM or HM. Quantizer values 32, 40, 48, 56 were chosen and quantizer values in VTM, HM and ETM were fixed to match each bit rate.

“End-usage” defining the quantization control is set to “q”: constant quality, which achieves constant quality level without regards to bitrate. Then “cq-level” allows to fix a base quantizer on the full clip in order to compute the BD-rate curves.

The meaning of “group of frames” in AV1 can be compared to “group of pictures” in MPEG codecs. The “group of frames” was fixed to 16 like in JVET CTC. The verification was done by plotting the curve of quantization parameters per picture. The quantization parameters offset per type of picture is used in a hierarchical GOF of 16 like in CTC configuration files of VTM or HM encoders.

Figure 1: AV1 curve of quantization parameter per picture – 1 Intra picture only

Figure 2: AV1 curve of quantization parameter per picture – 1sec Intra period

## AV1 command line

The difference between the two test conditions are on parameters kf-min-dist and kf-dist-max which are  the minimum and maximum distance between key frames. For “one intra only” condition the values are 0 and 1000 (>sequence length). For the “one second intra period” condition both values are equal to the number of frames per second.

| AV1 configuration |  |  |
| --- | --- | --- |
| Command line (example): ./aomenc seq.yuv --limit=total_frame_nb --cq-level=qp -o bitstream.webm --fpf=stats.fpf --width=width --height=height --fps=frame_rate --input-bit-depth=input_bit_depth --cpu-used=2 --kf-min-dist=0 --kf-max-dist=1000 --auto-alt-ref=1 --lag-in-frames=25 --min-gf-interval=16 --max-gf-interval=16 --threads=1 -v --i420 --skip=0 --codec=av1 --passes=2 --end-usage=q --minsection-pct=0 --maxsection-pct=2000 --static-thresh=0 --drop-frame=0 --tune=psnr --q-hist=0 --rate-hist=0 --psnr --bit-depth=10 --arnr-maxframes=7 --arnr-strength=3 |  |  |
| Passes | 2 | 1st pass used to build statistics, 2nd to encode |
| kf-min-dist kf-max-dist | Frame_freq (or 0) Frame_freq (or 1000) | 1 second (or 0) 1 second (or > sequence length) |
| auto-alt-ref arnr-maxframes arnr-strength | 1 7 3 | Alt-ref type of picture parameters |
| min-gf-interval max-gf-interval | 16 16 | Group of frames (similar to GOP) |
| End-usage | q | - q: constant quality, achieves constant quality level without regard to bitrate - cq: constrained quality, achieves constant quality level below a specified upper bitrate - vbr: variable citrate - cbr: constant bitrate |
| cq-level | 32, 40, 48, 56 | Constant Quality level (QP value) |
| cpu-used | 2 | Encoder speed up: 0, 1, 2 cpu-used=0 is the slowest one with the best quality |
| minsection-pct maxsection-pct | 0 2000 | Variation of bit rate in percentage in case of rate control |

Table 1: AV1 configuration

# Results

## BD-rate

In the following tables the results are provided using the following objective metrics: the Bjøntegaard PSNR Delta-Rate [9].

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE). 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results.

In order to have a better overview of the performances a combined PSNR is used in this study:

- PSNRYUV = (6xPSNRY + PSNRU + PSNRV) / 8.

It allows to mitigate the encoding strategy regarding the bit budget balance between luminance and chrominance.

The tables show the performance according to the PSNR BD-rate metrics in percentage with regards to a reference that can be any of the codecs. A negative percentage means a gain while a positive percentage means a loss regarding the reference.

Test results for the setting with one intra picture only on the first frame of the sequence are shown in Table 2, Table 3 and Table 4.

| BD-Rate PSNR YUV | Reference (1 intra only) UHD-HD |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1 intra only) UHD-HD |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 53.4% | 25.2% | 26.4% |
|  | VTM-5.0 | -34.6% |  | -15.1% | -18.1% |
|  | AV1 | -19.9% | 18.2% |  | -0.3% |
|  | ETM-2.01 | -20.7% | 22.2% | 0.9% |  |

Table 2: Results on UHD and HD sequences (Classes A1, A2 and B) for 1 intra only

| BD-Rate PSNR YUV | Reference (1 intra only) WVGA-WQVGA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1 intra only) WVGA-WQVGA |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 37.6% | 27.0% | 14.8% |
|  | VTM-5.0 | -27.1% |  | -6.6% | -16.5% |
|  | AV1 | -20.3% | 7.9% |  | -9.0% |
|  | ETM-2.01 | -12.9% | 20.2% | 11.7% |  |

Table 3: Results on WVGA and WQVGA formats (Classes C and D) for 1 intra only

| BD-Rate PSNR YUV | Reference (1 intra only) All |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1 intra only) All |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 46.7% | 25.9% | 21.5% |
|  | VTM-5.0 | -31.4% |  | -11.5% | -17.4% |
|  | AV1 | -20.1% | 13.8% |  | -4.0% |
|  | ETM-2.01 | -17.4% | 21.3% | 5.5% |  |

Table 4: Overall results (all sequences) for 1 intra only

Test results for the setting with second intra period are presented in Table 5, Table 6 and Table 7.

| BD-Rate PSNR YUV | Reference (1s intra period) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1s intra period) |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 56.2% | 12.3% | 26.2% |
|  | VTM-5.0 | -35.7% |  | -25.5% | -19.3% |
|  | AV1(2) | -10.7% | 34.8% |  | 10.7% |
|  | ETM-2.01 | -20.5% | 24.0% | -9.1% |  |

Table 5: Results on UHD and HD sequences (Classes A1, A2 and B) for 1s intra period

| BD-Rate PSNR YUV | Reference (1s intra period) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1s intra period) |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 36.0% | 8.1% | 13.1% |
|  | VTM-5.0 | -26.3% |  | -19.4% | -16.8% |
|  | AV1(2) | -7.0% | 24.4% |  | 4.7% |
|  | ETM-2.01 | -11.6% | 20.6% | -3.9% |  |

Table 6: Results on WVGA and WQVGA formats (Classes C and D) for 1s intra period

| BD-Rate PSNR YUV | Reference (1s intra period) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test (1s intra period) |  | HM | VTM-5.0 | AV1 | ETM-2.01 |
|  | HM |  | 47.7% | 10.5% | 20.7% |
|  | VTM-5.0 | -31.7% |  | -23.0% | -18.2% |
|  | AV1(2) | -9.2% | 30.4% |  | 8.2% |
|  | ETM-2.01 | -16.7% | 22.6% | -6.9% |  |

Table 7: Overall results (all sequences) for 1s intra period

## Run time

All simulations are run in single thread on the same platform in order to compare run times at encoder and decoder. Table 8 and Table 9 report the run time factor for the encoding and the decoding versus the HM-16.18 for UHD and HD sequences (Classes A1, A2, and B). The run time provides an estimate of the complexity. “Nx” means N times the HM-16.18 run time. Run times for the two test conditions are very close.

| Run time Vs HM-16.18 (1 intra only) | VTM-5.0 | AV1 | ETM-2.01 |
| --- | --- | --- | --- |
| Encoding | 12.17x | 4.97x | 4.76x |
| Decoding | 1.62x | 2.69x | 1.61x |

Table 8: Run times versus HM-16.18 for 1 intra only

| Run time Vs HM-16.18 (1s intra period) | VTM-5.0 | AV1 | ETM-2.01 |
| --- | --- | --- | --- |
| Encoding | 12.74x | 4.93x | 4.74x |
| Decoding | 1.60x | 2.57x | 1.56x |

Table 9: Run times versus HM-16.18 for 1s intra period

The platform characteristics are:

| CPU type | Intel(R) Xeon(R) Gold 6142 |
| --- | --- |
| Hyper threading | Off |
| Turbo mode | On |
| Compiler | gcc 6.3.0 |
| OS | CentOS7 |

Table 10: Platform characteristics

## Summary

The figures below summarize the performances of each video coding solutions with regards to HM.

Figure 3: BD-Rate versus encoding time over HM 16.18 for 1 intra only

Figure 4: BD-Rate versus decoding time over HM 16.18 for 1 intra only

Figure 5: BD-Rate versus encoding time over HM 16.18 for 1s intra period

Figure 6: BD-Rate versus decoding time over HM 16.18 for 1s intra period

# Conclusion

This comparative study on video coding solutions already specified HEVC (IEC/ISO, ITU-T), and AV1 (AOM), or still developing VVC (IEC/ISO, ITU-T), and EVC (IEC/ISO) has been conducted with as close as possible testing conditions. Indeed, AV1 software is designed as a product capable of encoding full movies with configuration to fix the ratio between encoding time and quality, the rate control, the number of passes etc. On the contrary the other codecs are test models which are designed to develop algorithms in a collaborative environment without all these features. A high attention was put to bring all the codecs in common testing conditions. The main difference is the two-passes encoding for AV1, while only one-pass is used for the other codecs because the one-pass seems not maintained in AV1 software. However, on short length clips with stable content the effect of the first pass is assumed to be negligible.

The performances according to the average BD-rate PSNR metrics of VTM-5.0 (VVC following Geneva meeting March 2019), AV1, and ETM 2.01 (EVC after Geneva meeting) versus HM-16.18 are respectively -31.4%, -20.1%, and -17.4% gains with one intra only, and -31.7%, -9.2%, and -16.7% gains with one second intra period. The compression gain of the publicly available AV1 software has been optimized for the streaming use case (-20.1% versus HEVC) with two-passes and without broadcast-like random-access. In this use case, AV1 is a serious competitor to VVC and EVC.

# References

- H.265 (04/2015), Coding of moving video: High Efficiency Video Coding, ITU-T

- JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

- JVET-O1002, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

- ISO/IEC JTC1/SC29/WG11 N18448, “Working Draft 2 of Essential Video Coding”, May 17th, 2019, Geneva

- ISO/IEC JTC1/SC29/WG11 MPEG2018/N18449, “Test Model of Essential Video Coding (ETM 1.0)”, May 17th, 2019, Geneva

- AV1 specification: https://aomediacodec.github.io/av1-spec/

- AV1 software commit cb43f76 from May 19th, 2019

- Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6 VCEG 13th meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33 (available at http://wftp3.itu.int/av-arch/video-site/0104_Aus/).

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6 VCEG 35th meeting, Berlin, Germany, 16–18 July, 2008, Doc. VCEG-AI11 (available at http://wftp3.itu.int/av-arch/video-site/0807_Ber/).

- ISO/IEC JTC1/SC29/WG11 MPEG2019/ m46554, “Tool comparison between VVC (VTM3.0) and NVC”, Jan. 2019, Marrakech

- “Analysis of Emerging Video Codecs: Coding Tools, Compression Efficiency and Complexity”, MediaKind (Ericsson), SMPTE 2018 annual conference