| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0914 |
| --- | --- |

| Title: | CE8-related: Improved color generation method for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao-Hsiung Hung, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-6581293 chaohsiu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes an improved color generation method based on CE test 8-2.1 [1] on palette mode tested on top of VTM-5.0. The results show that:

For 4:2:0 test sequences,

- the proposed method provides 4.41%, 2.12%, and 0.71% BD-rate gains for luma in TGM sequences, with 102%, 104% and 107 % in encoding time, and 102%, 102% and 104% in decoding time under AI, RA, and LD configurations, respectively.

- the proposed method provides 0.74%, 0.75%, and x.xx % BD-rate gains for luma in class F sequences, with 108%, 117% and xxx % in encoding time, and 107%, 108% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

For 4:4:4 test sequences,

- when DualITree turned off, the proposed method achieves 16.34%, 10.76%, and 6.34% BD-rate gain in TGM 1080p sequences, and 5.79%, 4.21%, and x.xx % BD-rate gains overall for luma, with 103%, 103% and xxx % in encoding time, and 99%, 99% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

- when DualITree turned on, the proposed method achieves 7.17%, 6.12%, and 5.31% BD-rate gain in TGM 1080p sequences, and 2.55%, 2.31%, and x.xx % BD-rate gains overall for luma with 111%, 104% and xxx % in encoding time, and 104%, 100% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

We also proposed a adaptive selection algorithm. If the check condition is satisfied, we will use the proposed generation method. Otherwise we use the original color generation method in CE test 8-2.1. The results with the adaptive selection algorithm show that:

For 4:2:0 test sequences,

- the proposed method provides 4.50%, 2.14%, and 0.57% BD-rate gains for luma in TGM sequences, with 104%, 106% and 107% in encoding time, and 103%, 103% and 103% in decoding time under AI, RA, and LD configurations, respectively.

- the proposed method provides 0.73%, 0.69%, and x.xx % BD-rate gains for luma in class F sequences, with 109%, 115% and xxx % in encoding time, and 108%, 110% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

For 4:4:4 test sequences,

- when DualITree turned off, the proposed method achieves 16.45%, 10.77%, and 6.29% BD-rate gain in TGM 1080p sequences, and 5.80%, 4.22%, and x.xx % BD-rate gains overall for luma, with 103%, 102% and xxx % in encoding time, and 100%, 99% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

- when DualITree turned on, the proposed method achieves 7.14%, 5.93%, and 5.08% BD-rate gain in TGM 1080p sequences, and 2.48%, 2.07%, and x.xx % BD-rate gains overall for luma with 109%, 104% and xxx % in encoding time, and 103%, 100% and xxx % in decoding time under AI, RA, and LD configurations, respectively.

# Introduction

The color generation method in CE 8-2.1 scans the whole CU. For a new scanned pixel, we calculate the difference between the pixel value and each color group mean. We then classified the pixel to the color group with the minimum difference. Then the group mean of that color group will be updated. After finishing classifying all pixels in the CU, each color group mean is a color on the palette table. The new classified pixel will make the group mean drifting. The finial generated palette color may be inaccurate.

The proposed method first constructs the histogram of the whole CU. Then it generates the palette table from the histogram.

# Proposed methods

## Improved color generation method

Figure 1. The histogram of the CU.

The proposed method first scans the whole CU and generates the corresponding histogram, as that shown in Figure 1. It finds the color C1 with the largest pixel number P1 in the histogram. The peaks, including P1, in the range E/2 around C1 in Figure 1 will be removed from the histogram. The average of removed peaks is a palette table color. These processes will continue until the histogram is empty. We set E = Error_Limit / 2 for luma and E = Error_Limit for chroma. Error_Limit depends on the quantization level.

## Adaptive Selection Algorithm

When adding a new pixel to a histogram, scanning the whole histogram to find the same pixel will be necessary. The histogram of natural content sequences may contain many colors. Constructing the histogram for a CU takes much time. The proposed color generation method also unsuitable for natural content sequences. We propose an adaptive selection algorithm as follows. When after scanning A pixels in the CU, the maximum accumulated pixel number of the peak in the temporary histogram is ≤ B. If this condition is satisfied, we stop constructing the histogram and generate palette table colors by original method. We set A = (total pixel number of CU) / 16 and B = 3. The proposed adaptive selection algorithm suspends histogram construction, it also improves the encoding time.

# Simulation results

We test 2 methods here. Method 1 is the proposed color generation method. Method 2 is the proposed color generation method with the adaptive selection algorithm.

## Test sequences 4:2:0

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 1 shows the method 1 results while Table 2 shows method 2 results.

Table 1: Results of method 1 on test sequences 4:2:0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | -0.03% | 116% | 102% |
| Class A2 | 0.05% | 0.06% | 0.04% | 114% | 99% |
| Class B | 0.05% | -0.01% | 0.08% | 111% | 106% |
| Class C | 0.05% | 0.06% | 0.06% | 111% | 107% |
| Class E | 0.06% | 0.01% | -0.05% | 111% | 102% |
| Overall | 0.04% | 0.02% | 0.03% | 112% | 104% |
| Class D | 0.03% | 0.29% | -0.03% | 108% | 110% |
| Class F | -0.74% | -0.50% | -0.41% | 108% | 107% |
| Class TGM | -4.41% | -3.44% | -2.78% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.12% | 103% | 102% |
| Class A2 | 0.07% | -0.08% | 0.08% | 102% | 102% |
| Class B | 0.04% | -0.08% | -0.26% | 102% | 102% |
| Class C | -0.01% | -0.09% | -0.08% | 104% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.09% | -0.07% | 103% | 103% |
| Class D | 0.04% | -0.27% | 0.14% | 103% | 109% |
| Class F | -0.75% | -0.66% | -0.53% | 117% | 108% |
| Class TGM | -2.12% | -1.78% | -1.49% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.03% | 0.30% | 0.04% | 101% | 106% |
| Class E | 0.06% | 0.17% | 0.18% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | 0.40% | 1.51% | 104% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | -0.71% | -0.45% | -0.27% | 107% | 104% |

Table 2: Results of method 2 on test sequences 4:2:0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.00% | 111% | 102% |
| Class A2 | 0.05% | 0.06% | 0.05% | 110% | 104% |
| Class B | 0.05% | 0.01% | 0.06% | 110% | 107% |
| Class C | 0.05% | 0.06% | 0.06% | 110% | 111% |
| Class E | 0.06% | 0.04% | -0.08% | 110% | 105% |
| Overall | 0.04% | 0.03% | 0.02% | 110% | 106% |
| Class D | 0.03% | 0.27% | -0.02% | 109% | 110% |
| Class F | -0.73% | -0.59% | -0.58% | 109% | 108% |
| Class TGM | -4.50% | -3.94% | -3.27% | 104% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.14% | 103% | 102% |
| Class A2 | 0.06% | -0.10% | 0.09% | 103% | 102% |
| Class B | 0.04% | -0.06% | -0.23% | 103% | 103% |
| Class C | 0.00% | -0.01% | -0.11% | 105% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.06% | 104% | 104% |
| Class D | 0.04% | -0.20% | 0.15% | 104% | 109% |
| Class F | -0.69% | -0.62% | -0.56% | 115% | 110% |
| Class TGM | -2.14% | -2.08% | -1.65% | 106% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | 0.23% | -0.12% | 102% | 106% |
| Class E | 0.06% | -0.05% | 0.02% | 101% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | 0.26% | 0.36% | 105% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | -0.57% | -0.46% | -0.21% | 107% | 103% |

## Test sequences 4:4:4, DualITree = 0

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned off for all Classes. The IBC, HashME, and RDPCM are turned on for all classes. Table 3 shows the method 1 results while Table 4 shows method 2 results.

Table 3: Results of method 1 on test sequences 4:4:4, DualITree = 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -16.34% | -22.18% | -21.90% | 99% | 89% |
| TGM 720p | -4.73% | -9.69% | -11.25% | 102% | 99% |
| Animation | 0.03% | -0.13% | -0.12% | 105% | 108% |
| Mixed content | -0.91% | -3.80% | -4.59% | 104% | 104% |
| Camera-Captured content | 0.01% | 0.01% | 0.04% | 110% | 102% |
| Overall | -5.79% | -9.28% | -9.77% | 103% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.76% | -16.95% | -16.50% | 89% | 91% |
| TGM 720p | -4.84% | -8.94% | -11.63% | 104% | 101% |
| Animation | 0.03% | -0.08% | -0.08% | 117% | 104% |
| Mixed content | -0.28% | -2.88% | -3.36% | 111% | 101% |
| Camera-Captured content | -0.02% | 0.24% | 0.18% | 106% | 103% |
| Overall | -4.21% | -7.46% | -8.16% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.34% | -10.89% | -10.53% | 101% | 98% |
| TGM 720p | -1.67% | -5.39% | -6.46% | 104% | 95% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | 0.44% | -2.60% | -3.12% | 110% | 100% |
| Camera-Captured content | -0.03% | 0.03% | -0.11% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4: Results of method 2 on test sequences 4:4:4, DualITree = 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -16.45% | -22.34% | -22.01% | 100% | 91% |
| TGM 720p | -4.67% | -9.75% | -11.35% | 104% | 100% |
| Animation | 0.03% | -0.19% | -0.22% | 105% | 107% |
| Mixed content | -0.86% | -4.19% | -4.77% | 104% | 104% |
| Camera-Captured content | 0.01% | 0.01% | 0.04% | 106% | 104% |
| Overall | -5.80% | -9.42% | -9.87% | 103% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.77% | -17.05% | -16.67% | 91% | 92% |
| TGM 720p | -4.77% | -8.94% | -11.84% | 104% | 100% |
| Animation | 0.03% | -0.21% | -0.11% | 113% | 104% |
| Mixed content | -0.36% | -3.29% | -3.76% | 109% | 101% |
| Camera-Captured content | -0.01% | 0.24% | 0.18% | 104% | 103% |
| Overall | -4.22% | -7.58% | -8.35% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.29% | -11.02% | -10.63% | 102% | 101% |
| TGM 720p | -1.81% | -5.48% | -7.06% | 104% | 97% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | 0.37% | -2.43% | -3.65% | 108% | 101% |
| Camera-Captured content | -0.03% | 0.04% | -0.12% | 101% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test sequences 4:4:4, DualITree = 1

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC, HashME, and RDPCM are turned on for all classes. Table 5 shows the method 1 results while Table 6 shows method 2 results.

Table 5: Results of method 1 on test sequences 4:4:4, DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.17% | -13.14% | -13.16% | 109% | 99% |
| TGM 720p | -2.29% | -6.39% | -8.00% | 108% | 104% |
| Animation | 0.03% | -0.97% | -0.68% | 109% | 111% |
| Mixed content | -0.19% | -5.45% | -5.20% | 109% | 105% |
| Camera-Captured content | 0.06% | 0.03% | 0.00% | 124% | 106% |
| Overall | -2.55% | -6.42% | -6.78% | 111% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.12% | -12.57% | -11.97% | 97% | 97% |
| TGM 720p | -1.99% | -6.37% | -7.48% | 101% | 97% |
| Animation | 0.02% | -0.37% | -0.31% | 119% | 105% |
| Mixed content | -0.79% | -6.24% | -5.53% | 111% | 100% |
| Camera-Captured content | 0.06% | 0.10% | -0.03% | 105% | 105% |
| Overall | -2.31% | -6.33% | -6.34% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.31% | -9.92% | -9.33% | 103% | 102% |
| TGM 720p | -1.03% | -4.87% | -5.57% | 104% | 98% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -0.69% | -6.39% | -5.76% | 111% | 101% |
| Camera-Captured content | 0.03% | -0.04% | -0.04% | 102% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 6: Results of method 2 on test sequences 4:4:4, DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.14% | -13.36% | -13.45% | 110% | 98% |
| TGM 720p | -2.07% | -6.66% | -8.33% | 108% | 103% |
| Animation | 0.05% | -1.04% | -0.73% | 108% | 112% |
| Mixed content | -0.18% | -5.50% | -5.19% | 109% | 104% |
| Camera-Captured content | 0.06% | 0.03% | 0.02% | 110% | 104% |
| Overall | -2.48% | -6.57% | -6.94% | 109% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.93% | -12.78% | -12.16% | 97% | 98% |
| TGM 720p | -1.42% | -6.45% | -7.75% | 101% | 97% |
| Animation | 0.04% | -0.40% | -0.32% | 115% | 105% |
| Mixed content | -0.61% | -6.13% | -5.46% | 110% | 100% |
| Camera-Captured content | 0.05% | 0.08% | -0.06% | 105% | 105% |
| Overall | -2.07% | -6.40% | -6.45% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.08% | -10.00% | -9.55% | 102% | 99% |
| TGM 720p | -1.09% | -4.90% | -6.03% | 104% | 99% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -0.76% | -6.41% | -5.69% | 109% | 99% |
| Camera-Captured content | 0.04% | -0.07% | 0.00% | 101% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

- Y.-H. Chao et al, “CE8-2.1: Palette mode in HEVC”, JVET-O0119, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).