| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0915 |
| --- | --- |

| Title: | Crosscheck of JVET-O0466: Non-CE6 : interaction between MTS and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck report |  |  |
| Author(s) or Contact(s): | Christopher Hollmann Torshamnsgatan 23 16840 Stockholm, Sweden | Tel: Email: | +46107146437 christopher.hollmann@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports a crosscheck of document JVET-O0466. All test points are reported to match the provided ones by the proponent and software and specification match the description provided in the document.

# Proposal description

The document JVET-O0466 proposes to restrict the usage of the Low Frequency Non-Separable Transform (LFNST) based on the used transform combination of the Multiple Transform Selection (MTS) tool. Which combinations of LFNST and MTS are allowed is signaled with an SPS flag. There are two tests:

- Test #1: LFNST index = 0 if MTS index > 0

- Test #2: LFNST index = 0 if MTS index > 2

# Crosscheck results

All test points (bit rate and PSNR) match exactly the values provided by the proponent.

The software matches description in the document.

The changes to the specification are consistent with the proposal described in the document and the modifications with the software.

# Detailed simulation results

## Test #1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.15% | 0.20% | 95% | 100% |
| Class A2 | 0.03% | 0.11% | 0.05% | 95% | 98% |
| Class B | 0.07% | 0.08% | 0.16% | 93% | 100% |
| Class C | 0.09% | -0.03% | 0.10% | 100% | 102% |
| Class E | 0.08% | 0.04% | -0.01% | 91% | 101% |
| Overall | 0.07% | 0.06% | 0.11% | 95% | 100% |
| Class D | 0.08% | 0.20% | -0.10% | 96% | 101% |
| Class F | 0.04% | 0.07% | 0.12% | 97% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.14% | 0.09% | 98% | 97% |
| Class A2 | -0.01% | 0.02% | -0.03% | 98% | 97% |
| Class B | -0.04% | -0.06% | -0.11% | 98% | 102% |
| Class C | -0.04% | -0.11% | 0.01% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.02% | 98% | 100% |
| Class D | 0.07% | -0.04% | 0.38% | 98% | 103% |
| Class F | 0.03% | -0.10% | -0.03% | 103% | 100% |

## Test #2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.01% | 96% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 93% |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 101% |
| Class C | 0.00% | 0.01% | 0.00% | 103% | 103% |
| Class E | 0.00% | -0.01% | 0.00% | 97% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 99% | 99% |
| Class D | 0.00% | -0.01% | -0.01% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.04% | 98% | 97% |
| Class A2 | 0.01% | 0.05% | 0.08% | 100% | 101% |
| Class B | 0.00% | 0.01% | -0.06% | 100% | 99% |
| Class C | -0.01% | 0.02% | 0.03% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.04% | 0.05% | 99% | 95% |
| Class F | 0.01% | 0.01% | 0.01% | 99% | 96% |