| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0919-v2 |
| --- | --- |

| Title: | Non-CE8: Minimum QP for Transform Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yung-Hsuan Chao, Hongtao Wang, Muhammed Coban | Tel: Email: | martak@qti.qualcomm.com yunghsua@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to limit the minimum allowed Quantization Parameter (QP) in transform skip when the nearly lossless condition is reached. Specifically, the baseQP for transform skip is clipped to the value 4 + 6*(internalBitDepth - inputBitDepth). The results of the proposed bugfix are as follows.

Low QPs 2 – 7	(Luma BD-rates for CTC, Class F, TGM)

- Overall: -0.18%(AI) -0.01%(RA) -0.10%(LB)

- Class F: -1.51%(AI) -0.05%(RA) -0.40%(LB)

- Class TGM: -1.60%(AI) -1.72%(RA) 0.09%(LB)

# Proposed method

For the transform skip mode, the base QP for luma is derived as

baseQp = qpy + qpBdOffset;

where qpBdOffset is equal to 6*( internalBitDepth - 8). In proposal [1], a limit of minimum allowed QP is introduced which clips the baseQp to

baseQp = max (qpy, 4) + qpBdOffset;

For test sequences with inputBitDepth equal to 8 (currently tested in class F and TGM), nearly lossless coding of the TS coefficients can be achieved at the proposed clipping value. However, the lossless condition should be different depending on the inputBitDepth of the test sequences. In this document, it is proposed to clip the base QP as follows:

baseQp = qpy + qpBdOffset;

baseQp = max (baseQp, 4 + 6* (internalBitDepth – inputBitDepth))

and signal the value of inputBitDepth in SPS. Alternatively, minimum QP values that can be used to TS could be signalled, whichcan be derived as:

qpy’ = 4 – 6*( inputBitDepth-8)

or the delta QP between the original QP and the minimum allowed QP, which can be derived as:

∆qpy = qpy’ – qpy = 4 – 6*( inputBitDepth-8) - qpy

# Experimental results

The results are reported using the Low QPs test conditions defined in CE7 and CE8 (QP = 2, 7, 12, 17).

Table I: Results for the AI configuration

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class B | -0.13% | 0.05% | 0.06% | 102% | 103% |
| Class C | -0.11% | 0.00% | 0.00% | 101% | 105% |
| Class E | -0.72% | 0.03% | 0.03% | 101% | 104% |
| Overall | -0.18% | 0.02% | 0.02% | 102% | 103% |
| Class D | -0.05% | 0.00% | 0.01% | 102% | 105% |
| Class F | -1.51% | -0.04% | -0.02% | 100% | 105% |
| TGM | -1.60% | -0.30% | -0.30% | 100% | 95% |

Table II: Results for the RA configuration

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 95% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class B | -0.03% | 0.01% | 0.01% | 104% | 106% |
| Class C | -0.02% | 0.01% | 0.01% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.01% | 102% | 102% |
| Class D | -0.02% | 0.00% | 0.00% | 103% | 102% |
| Class F | -0.05% | -0.22% | -0.23% | 104% | 108% |
| TGM | -1.72% | -0.28% | -0.29% | 102% | 105% |

Table III: Results for the LB configuration

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.02% | 0.02% | 104% | 108% |
| Class C | -0.02% | 0.01% | 0.02% | 103% | 106% |
| Class E | -0.32% | 0.20% | 0.30% | 104% | 105% |
| Overall | -0.10% | 0.06% | 0.09% | 104% | 107% |
| Class D | -0.01% | 0.00% | 0.00% | 102% | 106% |
| Class F | -0.40% | 0.01% | 0.04% | 105% | 114% |
| TGM | 0.09% | -0.06% | -0.07% | 104% | 103% |

# Reference

[1] T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Minimum Allowed QP for Transform Skip Mode”, JVET document JVET-O0405, Gothenburg, SE, 3–12 July 2019

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).