| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0924 |
| --- | --- |

| Title: | Crosscheck of JVET-O0223 (Non-CE7: Quantization matrices with single identifier and prediction from larger ones) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Julien Le Tanou 5 rue Louis Jacques Daguerre 35136 St Jacques de la Lande France | Tel: Email: | +33 2 56 85 56 77 julien.letanou@mediakind.com |
| Source: | MediaKind, R&D |  |  |

_____________________________

# Abstract

This input document reports crosscheck results of JVET-O0223 on Quantization matrices with single identifier and prediction from larger ones from Interdigital. JVET-O0223 proposes to modify quantization matrix (QM) signaling and prediction mechanism, initially adopted in VVC WD5 (as per JVET-N0847). Authors in JVET-O0223 provided draft text changes on top of current VVC WD5, as well as text count statistics for highlighting text simplification. Draft text and statistics have been verified and are reported consistent with the proposed changes. In addition, a GNU Octave source code, a test set of popular QMs and a test procedure for running code were provided by Authors to measure the bit cost reduction of the proposed QMs signaling and prediction process in comparison with HEVC and VVC WD5 (as per JVET-N0847) mechanisms. The current crosscheck reports that the GNU Octave source code is only implementing the encoding processes for bit generations and cost measurements. Decoding processes for compliance verification were not provided, nor crosschecked. However, the implemented encoding processes seem consistent with the proposed modifications in JVET-O0223, as well with the related specification texts in HEVC and in VVC WD5 being used for comparison. Following the provided test procedure, code has been successfully run. The bit costs and relative bit savings results are reported to match the ones in JVET-O0223. No implementation or test results on top of VTM-5.0 were provided nor cross-checked for performance and compliance verification purposes.

# Introduction

Quantization matrices (QMs) support was adopted in VVC WD5 [1], based on contribution JVET-N0847 [2]. Custom QMs can be signaled in SPS or PPS, and prediction can be used to reduce bit cost. The QM is identified by two parameters: a transform block size id (sizeId) and a matrix type id (matrixId). QM prediction is restricted to matrices of same block-size identifiers. JVET-N0847, as HEVC, describes a specific derivation process for each block size.

JVET-O0223 [3] proposes to modify QM signaling and prediction process, with the will of simplification, and such any matrix can be predicted from any previously signaled one (i.e. from/to different matrix sizes). It contains changes related to:

- The derivation and usage of matrix index for identifying a QM

  - In VVC WD5, after adoption of JVET-N0847, a QM is identified by two parameters, matrixId and sizeId.

  - In JVET-O0223, the QM is identified by one single parameter matrixId as a compound of the transform size and matrix type identifiers, with the identifiers indexed in decreasing block size order.

- The QM prediction process

  - In VVC WD5 when signaling custom QMs in scaling_list_data syntax, it is possible to predict a QM either from default values, or from previously transmitted one but only if of same sizeId.

  - In JVET-O0223, the QM can be predicted from any previously transmitted one (i.e. of different matrix sizes) using either a copy or decimation process.

- The transmission of all coefficients for size 64

  - In JVET-O0223, to enable prediction of smaller QMs from size 64, all coefficients for size-64 are transmitted in the scaling list syntax, even though the bottom-right quadrant is never used by VVC transformation and quantization process. As a side effect it means the specific syntax and zero-out process for size 64 matrix in VVC WD5 is removed.

- The QM selection and resize process

  - In JVET-O0223, the derivation process for QM first selects the right QM index depending on block parameters, then unifies the processes of decimation for rectangular blocks, repetition for blocks larger than 8x8, and chroma format adaptation into one single process.

# Crosscheck results

## Text complexity comparison with JVET-N0847

Using statistics from Microsoft Word and draft texts from JVET-M1001-v7, JVET-N0847, JVET-N1001-v8 and JVET-O0223, the statistics reported in section 3.1 of JVET-O0223 have been verified.

Consistency of draft text changes with proposal has been verified.

## Coding efficiency

GNU Octave source code, a test set of popular QMs and a test procedure for running code were provided.

The current crosscheck reports that the GNU Octave source code is only implementing the encoding processes for bit generations and cost measurements. Decoding processes for compliance verification were not provided nor crosschecked.

However, the implemented encoding processes seem consistent with the proposed modifications in JVET-O0223, as well with the related specification texts in HEVC and in VVC WD5 (as per JVET-N0847), being used for comparison.

Following the provided test procedure, the code has been successfully run using GNU Octave v5.1.0. The bit costs and relative bit savings results for each method are reported in Table 1.

The numbers are matching the ones reported in JVET-O0223.

It is noted that no implementation or test results on top of VTM-5.0 were provided nor cross-checked for performance and compliance verification purposes.

Table 1

| Test name | HEVC | N0847 | N0847 vs HEVC | Proposed | Proposed vs N0847 | Proposed vs HEVC |
| --- | --- | --- | --- | --- | --- | --- |
| JPEG-like | 3007 | 3729 | +722 (+24%) | 1059 | -2670 (-72%) | -1948 (-65%) |
| MPEG2-like | 1067 | 1333 | +266 (+25%) | 489 | -844 (-63%) | -578 (-54%) |
| h264-like | 950 | 1224 | +274 (+29%) | 480 | -744 (-61%) | -470 (-49%) |
| DTT-like | 2264 | 2940 | +676 (+30%) | 1142 | -1798 (-61%) | -1122 (-50%) |
| HEVC | 1281 | 1554 | +273 (+21%) | 535 | -1019 (-66%) | -746 (-58%) |
| symmetry1 | 2090 | 2658 | +568 (+27%) | 892 | -1766 (-66%) | -1198 (-57%) |
| symmetry2 | 1272 | 1620 | +348 (+27%) | 997 | -623 (-38%) | -275 (-22%) |
| symmetry3 | 2660 | 3220 | +560 (+21%) | 2094 | -1126 (-35%) | -566 (-21%) |
| symmetry4 | 2542 | 3146 | +604 (+24%) | 2064 | -1082 (-34%) | -478 (-19%) |
| symmetry7 | 3366 | 4264 | +898 (+27%) | 2675 | -1589 (-37%) | -691 (-21%) |
| asymmetry1 | 2414 | 3078 | +664 (+28%) | 1004 | -2074 (-67%) | -1410 (-58%) |
| asymmetry2 | 1394 | 1778 | +384 (+28%) | 1057 | -721 (-41%) | -337 (-24%) |
| asymmetry3 | 2962 | 3624 | +662 (+22%) | 2272 | -1352 (-37%) | -690 (-23%) |
| asymmetry4 | 3560 | 4344 | +784 (+22%) | 2828 | -1516 (-35%) | -732 (-21%) |
| asymmetry7 | 3946 | 5406 | +1460 (+37%) | 3268 | -2138 (-40%) | -678 (-17%) |
| Average | 2318 | 2928 | +610 (+26%) | 1524 | -1404 (-48%) | -795 (-34%) |

# Reference

- B.Bross, J.Chen, S.Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, March 2019

- O.Chubach, T.Toma, S.C.Lim, et al., “CE7-related: Support of quantization matrices for VVC”, JVET-N0847, Geneva, CH, March 2019

- P.de Lagrange, F.Leleannec, E.Francois, et al., “Non-CE7: Quantization matrices with single identifier and prediction from larger ones”, JVET-O0223, Gothenburg, SE, July 2019