| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SWE, 03–12 July 2019 | Document: JVET-O0925-r1 |
| --- | --- |

| Title: | Non-CE3: Simplifications of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Philipp Merkle, Philipp Helle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand (HHI) Adarsh K. Ramasubramonian, Thibaud Biatek, Geert Van der Auwera, Luong Pham Van, Marta Karczewicz (Qualcomm) Jangwon Choi, Jin Heo, Jaehyun Lim, Mehdi Salehifar, Seung-Hwan Kim (LGE) Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki (Sony) Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson) | Tel: Email: | +49-30-31002-562 +82-2-6912-6561 +81-50-3750-2744 jonathan.pfaff@hhi.fraunhofer.de aramsub@qti.qualcomm.com jangwon84.choi@lge.com kenji.kondo@sony.com zhi.a.zhang@ericsson.com |
| Source: | Fraunhofer HHI, Qualcomm Technologies Inc., LGE Electronics Inc., Sony Corporations, Ericsson |  |  |

_____________________________

# Abstract

This joint proposal combines several proposals on matrix based intra prediction (MIP) that were submitted to the 15th JVET Meeting in Gothenburg from July 03th to July 12th, 2019. It proposes to adopt technologies that were proposed in the following documents: JVET-O0160, JVET-O0161, JVET-O0168, JVET-O0169, JVET-O0187, JVET-O0396, JVET-O0407, JVET-O0481, JVET-O0485.

The BD-rate performance of the proposed combination compared to VTM-5.0 is as follows:

-0.11% / -0.17% / -0.10% (93% enc. / 99% dec.) for AI configuration and tba for RA configuration.

# Proposed Method

It proposes to adopt the following changes of MIP:

- MIP is implemented in 8 bit, as proposed in JVET-O0481, Method 2.

- The reference sample derivation for MIP is performed exactly as for the conventional intra prediction modes, as proposed in JVET-O0187, Proposal 3.

- For the upsampling step used in the MIP-prediction, original reference samples are used instead of downsampled ones, as proposed in JVET-O0161, Aspect 1.

- The extra handling of negative values is removed from the upsampling, as proposed in JVET-O0160 and in JVET-O0407.

- Clipping is performed before upsampling and not after upsampling, as proposed in JVET-O0160 and in JVET-O0168.

- The mapping tables from MIP modes to conventional intra prediction modes are removed. Instead, MIP modes are always mapped to the planar mode, as proposed in JVET-O0187, Proposal 1, and in JVET-O0396, Method 1a.

- For the coding of the MIP-modes, the MIP-MPMs are no longer used and the mapping from conventional intra prediction modes to MIP modes is removed. Instead, the MIP modes are coded using truncated binary code, as proposed in JVET-O0169.

- MIP is harmonized with unified MPM-coding: In the list of the five non-planar MPMs, the DC mode is replaced with an angular mode if one of the neighboring modes is angular, as proposed in JVET-O0485.

- LFNST is enabled for MIP on large blocks. Here, the LFNST transforms corresponding to the planar mode are used, as proposed in JVET-O0485.

It is asserted that some of these technologies were also proposed in several other documents, but for simplicity, the above list restricts to documents submitted by the proponents.

# Experimental Results

Table 1 and Table 2 below report on the coding performance of the proposed method over the VTM-5.0 anchor.

Table 1. Results of the proposed method over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,10% | -0,16% | -0,07% | 91% | 98% |
| Class A2 | -0,08% | -0,12% | -0,13% | 93% | 98% |
| Class B | -0,11% | -0,15% | -0,12% | 93% | 100% |
| Class C | -0,10% | -0,12% | 0,01% | 93% | 100% |
| Class E | -0,18% | -0,34% | -0,19% | 93% | 100% |
| Overall | -0,11% | -0,17% | -0,10% | 93% | 99% |
| Class D | -0,08% | 0,02% | -0,16% | 93% | 100% |
| Class F | -0,10% | 0,04% | 0,01% | 98% | 104% |

Table 2. Results of the proposed method over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,03% | -0,32% | -0,11% | 96% | 99% |
| Class A2 |  |  |  |  |  |
| Class B | 0,03% | 0,00% | 0,05% | 96% | 100% |
| Class C | -0,01% | -0,18% | 0,00% | 96% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,06% | 0,03% | 0,18% | 97% | 99% |
| Class F | -0,02% | 0,03% | 0,16% | 97% | 98% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).