| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0935-v1 |
| --- | --- |

| Title: | CE7-related: Alternative configuration of CE7-2.2 joint chroma residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Helmrich, Heiko Schwarz, Tung Nguyen, Christian Rudat, Detlev Marpe, Thomas Wiegand, Bappaditya Ray, Geert Van der Auwera, Adarsh K. Ramasubramonian, Muhammed Coban, Marta Karczewicz | Email: | christian.helmrich@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports simulation results for an alternative configuration of the CE test CE7-2.2.  The test CE7-2.2 extends the joint coding mode for chroma residuals; it supports three different joint coding modes (instead of one in VTM-5) for chroma residuals (which are indicated by the chroma cbfs).  The Cb and Cr residuals are derived from a one or two coded transform block.  In the configuration proposed in this contribution, the two additional joint chroma coding modes are only supported in intra slices; in inter slices, only one of the joint coding modes is supported (the VTM-5 mode).

For the SDR common test conditions, the following average BD rates are reported relative to VTM-5.0:

AI:		–0.22% (Y),    –0.42% (Cb),   –0.06% (Cr) 
RA:	–0.15% (Y),    –0.31% (Cb),     0.47% (Cr) 
LB:		  0.03% (Y),      0.00% (Cb),     0.12% (Cr)

For the HDR common test conditions, the following average BD rates (in %) are reported relative to VTM-5.0:

AI:		(DE100) –2.98,  (L100) –0.06,  (wPSNR) –0.04, –3.85, –0.85,  (PSNR) –0.05, –2.81, –0.96
RA:	(DE100) –2.71,  (L100) –0.04,  (wPSNR) –0.04, –3.75,   0.14,  (PSNR) –0.06, –2.66, –0.11

For the low-QP configuration (QP values 2, 7, 12, 17 and 100 coded frames) for SDR, the following BD rates are reported relative to VTM-5.0:

AI:		–0.18% (Y),      0.00% (Cb),   –0.04% (Cr) 
RA:	  x.xx% (Y),      x.xx% (Cb),     x.xx% (Cr) 
LB:		  0.01% (Y),    –0.12% (Cb),   –0.22% (Cr)

# Introduction

The CE test CE7-2.2 [4] proposes an extension of the joint chroma residual coding technique first described in JVET-M0305 [1] by offering a wider range of joint-coding parameters and modes. Specifically, the test CE7-2.2 proposes an extension by allowing three instead of one joint chroma coding mode as in JVET-N0282 configuration 2, signaled using the chroma coded block flag (CBF) syntax elements. These coding modes apply simplified cross-component rotational transforms.

Thus, in contrast to M0305, in which the addition of one single-channel joint chroma residual coding mode (given by Cr = −Cb) was suggested, in the extension, three modes for joint chroma residual coding with different mixing factors are supported. The modes are additionally characterized by a sign (i. e., sign of the joint-decoding weights, CSign in JVET-N0282 [2]) used for deriving the second chroma residual, which is coded in the slice header. The usage (activation) of a joint chroma coding mode is indicated by a TU-level flag tu_joint_cbcr_residual_flag and the selected mode is implicitly indicated by the chroma CBFs. The flag tu_joint_cbcr_residual_flag is present if either or both chroma CBFs for a TU are equal to 1.

The normative reconstruction process of the chroma residuals (resCb and resCr) from the transmitted transform blocks (resJointC1 and resJointC2) is summarized in the following table.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC1[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC1[ x ][ y ] | 2 |
| 1 | 1 | resCb[ x ][ y ] = resJointC1[ x ][ y ] + resJointC2[ x ][ y ] resCr[ x ][ y ] = CSign * ( resJointC1[ x ][ y ] – resJointC2[ x ][ y ] ) | 4 |
| 0 | 1 | if ( joint_cbcr_alt_mode_flag ) { resCb[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC2[ x ][ y ] } else { resCb[ x ][ y ] = resJointC2[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 } | 3 1 |

In the investigated encoder implementation, the joint chroma components are derived as explained in the following. Depending on the mode (listed in the tables above), resJointC1 and/or resJointC2 are derived as follows:

- If mode is equal to 4 (Hadamard transform, two residuals are transmitted), the joint residuals are determined according to

resJointC1[ x ][ y ] = ( resCb[ x ][ y ] + CSign * resCr[ x ][ y ] ) / 2
	resJointC2[ x ][ y ] = ( resCb[ x ][ y ] – CSign * resCr[ x ][ y ] ) / 2.

- Otherwise, if mode is equal to 2 (single residual with reconstruction Cb = C, Cr = CSign * C), the joint residual is determined according to

resJointC1[ x ][ y ] = ( resCb[ x ][ y ] + CSign * resCr[ x ][ y ] ) / 2.

- Otherwise, if mode is equal to 1 (single residual with reconstruction Cb = C, Cr = (CSign * C) / 2), the joint residual is determined according to

resJointC2[ x ][ y ] = ( 4 * resCb[ x ][ y ] + 2 * CSign * resCr[ x ][ y ] ) / 5.

- Otherwise (mode is equal to 3, i. e., single residual, reconstruction Cr = C, Cb = (CSign * C) / 2), the joint residual is determined according to

resJointC2[ x ][ y ] = ( 4 * resCr[ x ][ y ] + 2 * CSign * resCb[ x ][ y ] ) / 5.

More details on how the encoder selects the TU-wise joint mode are given in Sec. 2.4 of JVET-N0282 [2].

# Proposed Modification

In this contribution, we proposed the following modification of CE test CE7-2.2 [4]:  The three joint chroma coding modes described above are only supported in I slices. In P and B slices, only mode 2 (the original VTM-5 mode) is supported.  In P and B slices, the syntax element tu_joint_cbcr_residual_flag is only present if both chroma cbfs are 1.

In the draft text, only the conditions for the presence of the tu_joint_cbcr_residual_flag and the Cr residual date in the transform unit syntax are modified.  For CE7-2.2, the presence is specified by

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

In this contribution, we propose to modify the transform unit syntax according to (changes are highlighted):

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( slice_type = = I && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && ( slice_type = = I ¦ ¦ !tu_joint_cbcr_residual_flag[ x0 ][ y0 ] ) ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

The text changes relative to the latest draft JVET-N1001-v8 are provided in the accompanying Word documents.

# Experimental Results

The proposed joint chroma coding configurations were tested on top of VTM 5.0 [5] according to the JVET common test conditions for SDR content [6]), the low-QP configuration defined in the CE7 description [3] as well as the common test conditions for HDR/WCG video [7]. Detailed results are provided on the following pages and in the accompanying Excel documents.

## SDR Common Test Conditions

The simulation results for the SDR common test conditions are summarized in Table 1. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

In order to evaluate the improvement of the VTM-5 joint chroma coding due to the suggested modifications, we additionally compared the results of the proposal as well as the results of VTM-5.0 to VTM-5.0 with joint chroma coding disabled. This additional anchor results were obtained by setting the macro JVET_N0054_JOINT_CHROMA in the VTM-5.0 software equal to 0.  The average BD rates of VTM-5.0 and the proposal relative to this additional anchor are summarized in Table 2.

Table 1. Average Bjøntegaard delta-rate results relative to VTM-5.0 for SDR common test conditions.

|  |  | Reference: VTM-5.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Proposed Joint Cb/Cr |  |  |  |  |  | CE7-2.2 Joint Cb/Cr [4] |  |  |  |  |
|  |  | Y | Cb | Cr | EncT | DecT |  | Y | Cb | Cr | EncT | DecT |
| AI | A1 | -0.25% | 0.08% | 0.75% | 102% | 98% |  | -0.25% | 0.08% | 0.75% | 101% | 96% |
|  | A2 | -0.72% | -0.96% | -0.01% | 106% | 99% |  | -0.72% | -0.96% | -0.01% | 104% | 99% |
|  | B | -0.09% | 0.30% | 0.16% | 103% | 99% |  | -0.09% | 0.30% | 0.16% | 103% | 99% |
|  | C | -0.07% | -0.71% | -0.29% | 103% | 101% |  | -0.07% | -0.71% | -0.29% | 101% | 100% |
|  | E | -0.10% | -1.17% | -0.99% | 102% | 102% |  | -0.10% | -1.17% | -0.99% | 102% | 99% |
|  | avg. | -0.22% | -0.42% | -0.06% | 103% | 100% |  | -0.22% | -0.42% | -0.06% | 102% | 99% |
|  | D | -0.07% | -0.74% | -0.20% | 103% | 100% |  | -0.07% | -0.74% | -0.20% | 102% | 98% |
|  | F | -0.17% | -0.64% | -0.95% | 102% | 100% |  | -0.17% | -0.64% | -0.95% | 101% | 98% |
| RA | A1 | -0.26% | -0.20% | 0.77% | 100% | 100% |  | -0.30% | -0.67% | 1.11% | 101% | 103% |
|  | A2 | -0.32% | -0.85% | 0.85% | 100% | 102% |  | -0.38% | -1.77% | 0.51% | 103% | 102% |
|  | B | -0.04% | 0.22% | 0.41% | 100% | 101% |  | -0.02% | 0.29% | 0.57% | 101% | 101% |
|  | C | -0.08% | -0.64% | 0.03% | 101% | 102% |  | -0.03% | -0.72% | 0.12% | 102% | 100% |
|  | avg. | -0.15% | -0.31% | 0.47% | 100% | 101% |  | -0.15% | -0.58% | 0.55% | 102% | 101% |
|  | D | -0.02% | -0.64% | 0.17% | 101% | 100% |  | -0.01% | -0.95% | 0.50% | 101% | 99% |
|  | F | -0.12% | -0.41% | -0.45% | 102% | 98% |  | -0.09% | -0.65% | -0.55% | 101% | 98% |
| LB | B | -0.01% | 0.60% | 0.21% | 101% | 100% |  | 0.03% | 0.92% | 1.93% | 103% | 101% |
|  | C | 0.00% | 0.00% | 0.07% | 100% | 101% |  | 0.07% | 0.21% | 1.26% | 102% | 105% |
|  | E | 0.15% | -0.99% | 0.06% | 101% | 102% |  | 0.25% | -0.17% | 1.52% | 103% | 108% |
|  | avg. | 0.03% | 0.00% | 0.12% | 101% | 101% |  | 0.10% | 0.41% | 1.60% | 103% | 104% |
|  | D | 0.01% | -0.55% | 0.99% | 100% | 101% |  | 0.08% | -0.29% | 3.32% | 101% | 107% |
|  | F | -0.01% | 0.13% | -0.06% | 100% | 100% |  | 0.13% | -0.36% | -0.71% | 102% | 103% |

Table 2. Average Bjøntegaard delta-rate results relative to VTM-5.0 without joint chroma residual coding (for SDR common test conditions).

|  |  | Reference: VTM-5.0 without joint cb/cr residual coding |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Proposed Joint Cb/Cr |  |  |  |  |  | VTM-5.0 Joint Cb/Cr |  |  |  |  |
|  |  | Y | Cb | Cr | EncT | DecT |  | Y | Cb | Cr | EncT | DecT |
| AI | A1 | -0.51% | 0.11% | 1.47% | 105% | 100% |  | -0.26% | -0.03% | 0.72% | 103% | 102% |
|  | A2 | -1.34% | -2.37% | 0.46% | 112% | 101% |  | -0.63% | -1.44% | 0.48% | 106% | 102% |
|  | B | -0.26% | 0.02% | -1.62% | 106% | 99% |  | -0.17% | -0.27% | -1.80% | 102% | 100% |
|  | C | -0.33% | -1.78% | -3.18% | 108% | 101% |  | -0.26% | -1.09% | -2.89% | 105% | 100% |
|  | E | -0.28% | -1.14% | -3.41% | 105% | 100% |  | -0.18% | 0.03% | -2.43% | 103% | 99% |
|  | avg. | -0.50% | -0.95% | -1.40% | 107% | 100% |  | -0.28% | -0.56% | -1.35% | 104% | 100% |
|  | D | -0.31% | -2.07% | -2.82% | 108% | 99% |  | -0.24% | -1.34% | -2.63% | 105% | 99% |
|  | F | -0.46% | -3.01% | -4.68% | 104% | 99% |  | -0.29% | -2.40% | -3.76% | 102% | 99% |
| RA | A1 | -0.64% | -0.46% | 2.19% | 104% | 100% |  | -0.38% | -0.30% | 1.41% | 104% | 100% |
|  | A2 | -0.71% | -3.62% | 1.11% | 104% | 103% |  | -0.39% | -2.81% | 0.26% | 104% | 101% |
|  | B | -0.20% | 0.13% | -0.81% | 103% | 101% |  | -0.17% | -0.10% | -1.25% | 102% | 100% |
|  | C | -0.25% | -0.44% | -2.12% | 103% | 101% |  | -0.17% | 0.19% | -2.15% | 103% | 99% |
|  | avg. | -0.40% | -0.89% | -0.18% | 103% | 101% |  | -0.25% | -0.60% | -0.66% | 103% | 100% |
|  | D | -0.17% | -1.11% | -2.28% | 103% | 100% |  | -0.15% | -0.47% | -2.45% | 103% | 100% |
|  | F | -0.53% | -2.14% | -3.99% | 104% | 99% |  | -0.42% | -1.74% | -3.55% | 102% | 101% |
| LB | B | -0.16% | -3.12% | -4.14% | 103% | 101% |  | -0.14% | -3.70% | -4.35% | 102% | 101% |
|  | C | -0.11% | -3.35% | -4.88% | 103% | 101% |  | -0.11% | -3.34% | -4.93% | 102% | 100% |
|  | E | 0.07% | -3.39% | -5.16% | 100% | 100% |  | -0.08% | -2.40% | -5.22% | 99% | 98% |
|  | avg. | -0.08% | -3.26% | -4.64% | 102% | 101% |  | -0.12% | -3.26% | -4.76% | 101% | 100% |
|  | D | -0.06% | -4.97% | -4.03% | 101% | 100% |  | -0.06% | -4.44% | -4.97% | 101% | 98% |
|  | F | -0.31% | -3.26% | -4.20% | 102% | 102% |  | -0.30% | -3.40% | -4.13% | 102% | 102% |

## HDR Common Test Conditions

The simulation results for the HDR common test conditions are summarized in Table 3. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

Table 3. Average Bjøntegaard delta-rate results relative to VTM-5.0 for HDR common test conditions.

|  |  | All Intra (vs VTM-5.0) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR |  | wPSNR |  |  | PSNR |  |  |  |
|  |  |  | L100 | Y | Cb | Cr | Y | Cb | Cr | EncT | DecT |
| Proposed | H1 | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.04% | -4.26% | -0.33% | 103% | 101% |
|  | H2 |  |  |  |  |  | -0.08% | -0.25% | -2.07% | 102% | 100% |
|  | avg. | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.05% | -2.81% | -0.96% | 103% | 100% |
| CE7-2.2 | H1 | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.04% | -4.26% | -0.33% | 103% | 100% |
|  | H2 |  |  |  |  |  | -0.08% | -0.25% | -2.07% | 102% | 101% |
|  | avg. | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.05% | -2.81% | -0.96% | 103% | 100% |
|  |  | Random Access (vs VTM-5.0) |  |  |  |  |  |  |  |  |  |
|  |  | DE100 | PSNR |  | wPSNR |  |  | PSNR |  |  |  |
|  |  |  | L100 | Y | Cb | Cr | Y | Cb | Cr | EncT | DecT |
| Proposed | H1 | -2.71% | -0.04% | -0.04% | -3.75% | 0.14% | -0.05% | -4.05% | 0.85% | 101% | 102% |
|  | H2 |  |  |  |  |  | -0.07% | -0.22% | -1.79% | 101% | 104% |
|  | avg. | -2.71% | -0.04% | -0.04% | -3.75% | 0.14% | -0.06% | -2.66% | -0.11% | 101% | 103% |
| CE7-2.2 | H1 | -2.84% | -0.03% | -0.03% | -3.85% | -0.24% | -0.04% | -4.16% | 0.55% | 101% | 100% |
|  | H2 |  |  |  |  |  | -0.07% | 0.05% | -2.78% | 101% | 101% |
|  | avg. | -2.84% | -0.03% | -0.03% | -3.85% | -0.24% | -0.05% | -2.63% | -0.66% | 101% | 100% |

## Low QP Results for SDR

The simulation results for the low QP configuration (SDR with QP values 2, 7, 12, 17 and 100 encoded frames) are summarized in Table 4. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

Table 4. Average Bjøntegaard delta-rate results relative to VTM-5.0 for the low-QP configuration (SDR with QP values 2, 7, 12, 17 and 100 coded frames).

|  |  | Reference: VTM-5.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Proposed Joint Cb/Cr |  |  |  |  |  | CE7-2.2 Joint Cb/Cr |  |  |  |  |
|  |  | Y | Cb | Cr | EncT | DecT |  | Y | Cb | Cr | EncT | DecT |
| AI | A1 | -0.01% | -0.09% | 0.03% | 105% | 99% |  | -0.01% | -0.09% | 0.03% | 105% | 99% |
|  | A2 | -0.30% | 0.11% | 0.05% | 110% | 100% |  | -0.30% | 0.11% | 0.05% | 110% | 100% |
|  | B | -0.09% | -0.01% | -0.08% | 109% | 106% |  | -0.09% | -0.01% | -0.08% | 109% | 106% |
|  | C | -0.36% | -0.07% | -0.09% | 105% | 101% |  | -0.36% | -0.07% | -0.09% | 105% | 101% |
|  | E | -0.15% | 0.11% | -0.03% | 109% | 103% |  | -0.15% | 0.11% | -0.03% | 109% | 103% |
|  | avg. | -0.18% | 0.00% | -0.04% | 108% | 102% |  | -0.18% | 0.00% | -0.04% | 108% | 102% |
|  | D | -0.43% | -0.24% | -0.39% | 106% | 97% |  | -0.43% | -0.24% | -0.39% | 106% | 97% |
|  | F | -0.79% | -0.70% | -0.69% | 105% | 103% |  | -0.79% | -0.70% | -0.69% | 105% | 103% |
| RA | A1 |  |  |  |  |  |  | -0.02% | -0.36% | 0.15% | 104% | 98% |
|  | A2 |  |  |  |  |  |  | -0.24% | 0.02% | 0.01% | 108% | 100% |
|  | B | 0.02% | -0.16% | -0.09% | 103% | 102% |  | -0.02% | -0.12% | -0.20% | 107% | 102% |
|  | C | -0.02% | -0.24% | -0.20% | 102% | 102% |  | -0.13% | -0.30% | -0.39% | 106% | 100% |
|  | avg. |  |  |  |  |  |  | -0.09% | -0.19% | -0.14% | 106% | 100% |
|  | D | -0.04% | -0.22% | -0.37% | 102% | 103% |  | -0.17% | -0.37% | -0.61% | 104% | 100% |
|  | F | -0.37% | -0.49% | -0.64% | 102% | 105% |  | -0.56% | -0.74% | -0.98% | 108% | 106% |
| LB | B | 0.01% | -0.08% | -0.07% | 101% | 102% |  | -0.03% | -0.06% | -0.13% | 104% | 101% |
|  | C | 0.01% | -0.19% | -0.25% | 101% | 100% |  | -0.13% | -0.25% | -0.33% | 104% | 100% |
|  | E | 0.00% | -0.08% | -0.44% | 99% | 100% |  | -0.02% | -0.07% | -0.35% | 103% | 99% |
|  | avg. | 0.01% | -0.12% | -0.22% | 101% | 101% |  | -0.06% | -0.13% | -0.25% | 104% | 100% |
|  | D | 0.03% | -0.26% | -0.27% | 101% | 102% |  | -0.12% | -0.36% | -0.31% | 102% | 99% |
|  | F | -0.09% | -0.09% | -0.35% | 101% | 103% |  | -0.43% | -0.45% | -0.59% | 106% | 107% |

# References

- J. Lainema, “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-M0305, Marrakech, MA, Jan. 2019.

- C. Helmrich, C. Rudat, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE7-related: Joint chroma residual coding with multiple modes,” Joint Video Experts Team, doc. JVET-N0282, Geneva, CH, Mar. 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Joint Video Experts Team, doc. JVET-N1027, Geneva, CH, June 2019.

- C. Helmrich, H. Schwarz, T. Nguyen, C. Rudat, D. Marpe, T. Wiegand, B. Ray, G. Van der Auwera, A. K. Ramasubramonian, M. Coban, M. Karczewicz, “CE7: Joint chroma residual coding with multiple modes (tests CE7-2.1, CE7-2.2),” Joint Video Experts Team, doc. JVET-O0105, Gothenburg, SE, July 2019.

- JVET, “VTM version 5.0,” https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-N1010, Geneva, CH, May 2019.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” Joint Video Experts Team, doc. JVET-N1011, Geneva, CH, Apr. 2019.

# Patent Rights Declarations

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).