| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0940 |
| --- | --- |

| Title: | Crosscheck of JVET-O0634: Non-CE9: BDOF simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu | Tel: Email: | 1-858-209-3851 xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Verification

The document reports the results of crosschecking JVET-O0634 where the DMVR is conditionally disabled based on the following conditions.

- Not applied in Nx8 CUs case.

- Not applied in 8x8/4xN CUs case.

3.  Turn BDOF off when DMVR is activated for small blocks (8x16/16x8 CUs).

Test1 is to further check the performance impact on BDOF when Nx8 CUs are removed. Test2 is to check the performance impact on BDOF when 8x8/4xN CUs are removed. Test3 is to check the case that BDOF is off when DMVR is activated for 8x16/16x8 CUs.

The software was provided by the proponent and it is verified that the software implementation is matched with what is described in JVET-O0634. After the cross-checking, all the BD-rates results match exactly with those in JVET-O0634. Due to heterogeneous cores that are used, the encoding and decoding time may not be reliable for the cross-check results.

- Test One

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.01% | 100% | 100% |
| Class A2 | 0.07% | -0.04% | -0.03% | 100% | 100% |
| Class B | 0.06% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.10% | 0.01% | 0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.03% | -0.01% | 100% | 101% |
| Class D | 0.20% | -0.02% | 0.04% | 100% | 103% |
| Class F (optional) | 0.05% | 0.03% | 0.00% | 100% | 102% |

- Test Two

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.05% | 100% | 100% |
| Class A2 | 0.04% | -0.02% | -0.07% | 100% | 100% |
| Class B | 0.03% | 0.00% | 0.02% | 100% | 100% |
| Class C | 0.08% | 0.04% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.04% | 100% | 100% |
| Class D | 0.19% | -0.01% | 0.12% | 100% | 98% |
| Class F (optional) | 0.02% | 0.02% | 0.00% | 100% | 101% |

- Test Three

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | -0.01% | 98% | 98% |
| Class A2 | 0.01% | -0.01% | 0.00% | 107% | 117% |
| Class B | 0.02% | 0.02% | 0.01% | 102% | 105% |
| Class C | 0.02% | 0.01% | 0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.01% | 101% | 104% |
| Class D | 0.09% | 0.07% | 0.10% | 99% | 99% |
| Class F (optional) | 0.02% | 0.04% | 0.02% | 100% | 101% |